3GPP TSG-RAN WG4 Meeting #112-bis	                            R4-2414996

Hefei, China, 14 – 18 October, 2024

Agenda item:	6.21.2.2

Source: 	CMCC

Title: 	Discussion on A-IoT co-existence evaluation

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives, and in last meeting, a WF is approved [2]:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide simulation results based on agreed parameters and discuss the left open issues for co-existence.

- Evaluation cases and parameters

In this contribution, we simulated the following baseline cases.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | note |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | D2R |  |
|  |  |  | b | NR DL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | D2R |  |
|  |  |  | b | NR DL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | c | Reader->NR DL | R2D |  |
|  |  |  | d | NR DL->device | R2D |  |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | g | Reader->NR UL | R2D |  |
|  |  |  | h | NR UL->device | R2D |  |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | D2R |  |
|  |  |  | f | NR UL->reader | D2R |  |
|  |  |  | g | Reader->NR UL | R2D |  |
|  |  |  | h | NR UL->device | R2D |  |

For evaluation parameters, we use the agreed the parameters in WF [1]. For the parameters that have multiple options, we use following values for evaluation.

CW cancellation capability

|  | CW cancellation capability |
| --- | --- |
| D1T1-A2 | 140dB |
| D1T1-B | 150dB |
| D2T2-A2 | 130dB |
| D2T2-B | 140dB |

A-IOT reader and NR BS

|  | In-band |  |
| --- | --- | --- |
|  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | Legacy UE IBE (QPSK) | In-band A-IOT signal interference to NR is not considered when calculating SINR. |
| NR BS | Option 1: 30dBc | In-band A-IOT signal interference to NR is not considered when calculating SINR. |
| A-IOT BS | ACLR of legacy NB -IOT gNB (i.e. ACLR1:40dBc，ACLR2:50dBc) | ACS of legacy gNB |

Tx for device 1 and 2a

- 25dBc with 15KHz bandwidth

Rx for device 1 and 2a

- Assume no frequency selectivity

Active rate of reader for D1T1 and D2T2

- Minimum distance between active readers: 60m

- 2 readers are activated in one drop

Indoor UE percentage for scenario option 1-2

- Option 1: 10%

Issue 4-2-6: Penetration loss for O2I

- Use the equation of 7.4-2 in 38.901

- PLin = 0.5 * d2D-in where d2D-in is the distance to nearest factory/office boundary on the line between Tx and Rx point.

- Set maximum value of d2D-in as 25m

- Evaluation results and conclusion

When NR system is victim (case a,c,e,g), 5% throughput loss is the performance metric as legacy. Based on simulation results, it can be observed that:

- For D1T1 scenario 1-1 and 2-1, with legacy UE only outdoor, there is almost no throughput degradation for NR system for average throughput and cell edge throughput,

- For D1T1 scenario 1-2 and 2-2, with legacy UE indoor percentage 10%, about 10% NR UE throughput degradation for average throughput is observed for case c, about 70% NR UE throughput degradation for cell edge throughput is observed for case c.

- For D2T2 scenario 5-1 and 5-2, about 6% NR BS throughput degradation is observed for average throughput for case g, about 12% NR BS throughput degradation is observed for cell edge throughput for case g.

- No throughput degradation is observed for case a and e, which device is aggressor.

When ambient IOT system is victim (case b,d,f,h), the performance metric is agreed in last meeting as below:

For inter-system interference (between AIOT and NR):

- If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible.

- If SINR degradation is lager than [1]dB, consider following criteria:

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

- Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

For intra-system interference (between AIOT and AIOT):

  - Outage percentage consider SINR level with [10%] BLER

  - FFS on the outage percentage

For inter-system interference, based on evaluation results, it can be observed that:

- For D1T1 scenario 1-1 and 2-1, SINR degradation is less than 1dB, which can be considered that inter-system interference is negligible.

- For D1T1 scenario

For D2R, when reader is victim (case b and f), link level SINR level with 10% BLER is provided as below:

| [0n]Message size | [2a2] chip_rate | Pathloss | TDL-A/D | Device type | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 96-bit | 42kcps (6kbps) | InF-DH NLOS | TDL-A | Device 1 | -2.17 |
| 96-bit | 42kcps (6kbps) | InF-DH NLOS | TDL-A | Device 2a | -4.69 |
| 96-bit | 42kcps (6kbps) | InH-Office LOS | TDL-D | Device 1 | -4.66 |
| 96-bit | 42kcps (6kbps) | InH-Office LOS | TDL-D | Device 2a | -6 |

For R2D, when device is victim (case d and h), link level SINR level with 10% BLER is provided as below:

| [0n]Message size | chip_rate | Raw Data rate | TDL-A/D | Device type | CNR(dB) | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 20-bit | 14 kcps | 7kbps | TDL-A | Device 1 | 16.5 | -1 |
| 20-bit | 14 kcps | 7kbps | TDL-A | Device 2a | 5.4 | -12 |

In the following table, we summarize the observation from co-existence evaluation. The observation highlighted in green, the interference from inter-system (between A-IOT and NR) and intra-system (between A-IOT and A-IOT) is negligible. However, when the victim is A-IOT reader or device, link level simulation results are required to check the SNR for 10% BLER, more discussion is needed to check the value used for companies. Also, the outage percentage of A-IOT system needs further discussion.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Observation |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB Device 1 outage percentage consider SINR level with 10% BLER is 16% Device 2a outage percentage consider SINR level with 10% BLER is 7% |
|  |  |  | c | Reader->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a. |
| 1-2 | D1T1-A2-legacy UE indoor (10% indoor UE) | R2D: DL CW2D and D2R: DL | a | Device->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB Device 1 outage percentage consider SINR level with 10% BLER is 16% Device 2a outage percentage consider SINR level with 10% BLER is 7% |
|  |  |  | c | Reader->NR DL | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a. |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 3% for device 1 and 2% for device 2a |
|  |  |  | c | Reader->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 2-2 | D1T1-B-legacy UE indoor (10% indoor UE) | R2D: DL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is 13dB for cell edge and 4dB for average Outage percentage consider SINR level with 10% BLER is 21% for device 1 and 17% for device 2a |
|  |  |  | c | Reader->NR DL | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 4% for device 1 and 3% for device 2a |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 5-2 | D2T2-A2-legacy UE indoor (10% indoor UE) | R2D: UL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is 20dB for cell edge and 4.8dB for average Outage percentage consider SINR level with 10% BLER is 24% for device 1 and 22% for device 2a |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is 3dB for cell edge and 5dB for average Outage percentage consider SINR level with 10% BLER is 5% for device 1 and 0% for device 2a |

# 3. Conclusions

In this contribution, co-existence evaluation parameters and observations are provided in the following table:

- For the cases highlighted in green, interference is negligible.

- For the cases that victim is A-IOT reader and device, more discussion on the required SNR at 10% BLER and outage percentage is required.

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor -> Victim) | Observation |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB Device 1 outage percentage consider SINR level with 10% BLER is 16% Device 2a outage percentage consider SINR level with 10% BLER is 7% |
|  |  |  | c | Reader->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a. |
| 1-2 | D1T1-A2-legacy UE indoor (10% indoor UE) | R2D: DL CW2D and D2R: DL | a | Device->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | b | NR DL->reader | SINR degradation is less than 1dB Device 1 outage percentage consider SINR level with 10% BLER is 16% Device 2a outage percentage consider SINR level with 10% BLER is 7% |
|  |  |  | c | Reader->NR DL | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a. |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 3% for device 1 and 2% for device 2a |
|  |  |  | c | Reader->NR DL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 2-2 | D1T1-B-legacy UE indoor (10% indoor UE) | R2D: DL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is 13dB for cell edge and 4dB for average Outage percentage consider SINR level with 10% BLER is 21% for device 1 and 17% for device 2a |
|  |  |  | c | Reader->NR DL | about 10% throughput degradation for average throughput is observed, about 70% throughput degradation for cell edge throughput is observed |
|  |  |  | d | NR DL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 4% for device 1 and 3% for device 2a |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB Outage percentage consider SINR level with 10% BLER is 0% for both device 1 and 2a |
| 5-2 | D2T2-A2-legacy UE indoor (10% indoor UE) | R2D: UL CW2D and D2R: UL | e | Device->NR UL | No throughput degradation for NR system for average throughput and cell edge throughput |
|  |  |  | f | NR UL->reader | SINR degradation is 20dB for cell edge and 4.8dB for average Outage percentage consider SINR level with 10% BLER is 24% for device 1 and 22% for device 2a |
|  |  |  | g | Reader->NR UL | about 6% throughput degradation is observed for average throughput, about 12% NR BS throughput degradation is observed for cell edge throughput. |
|  |  |  | h | NR UL->device | SINR degradation is 3dB for cell edge and 5dB for average Outage percentage consider SINR level with 10% BLER is 5% for device 1 and 0% for device 2a |

# 4. Reference

- R4-2414304, WF on Co-existence study for ambient IoT

- R4-2406714, WF on Ambient IoT in NR, CMCC

- R4-2407008, LS on Ambient-IoT evaluation scenarios and assumptions, CMCC

# 4. Annex: link level parameters

Table 3.2.2-1. Link-level Simulation assumptions for R2D

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 900MHz |
| Chip rate | 14 kcps, M=1 |
| Modulation | OOK-4, M = 2 |
| Signal Bandwidth | 1RB (180kHz) |
| Subcarrier spacing | 15kHz |
| Channel model | TDL-A 30ns; TDL-D 30ns |
| Device velocity | 3 km/h |
| Number of RX antennas | 1 |
| Time/frequency synchronization | Ideal |
| RF filter | 3rd @10MHz |
| BB filter | 3rd @60kHz |
| TBS | 24 bits; 40 bits |
| ADC | Ideal quantization, 1.92MHz sampling rate |
| Coding | 1/2 Manchester encoding |
| Receiver mode | Device 1：Edge detector Device 2a：Energy detector |

Table 3.2.2-2. Link-level Simulation assumptions for D2R

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 900MHz |
| Data rate | 6 kbps |
| Chip rate | 6 kbps data rate: BPSK chip rate = 42 kcps |
| Block structure | preamble (131 chip)+ {96b, 24b CRC, 1/3 TBCC, Manchester, BPSK, midamble(5 chip)} + postamble(131 chip) |
| Subcarrier spacing | 15kHz |
| Channel model | TDL-A 30ns; TDL-D 30ns |
| Device velocity | 3 km/h |
| Number of RX antennas | 2 for BS; 1 for intermediate UE |
| Message size | 96 bits; 20 bits; 400bits |
| Sampling frequency | Sampling frequency at receiver side is higher than 32X chip rate, Device 1 SFO: Fe = 10% Device 2a SFO: Fe = 1% |
| Coding | 1/2 Manchester encoding |
| Receiver bandwidth | 6 kbps data rate: DSB 90 kHz for device 1/2a; DSB 180kHz for device 2b |
| Waveform | Unmodulated single tone |
| Modulation | BPSK |
| FEC | 1/3 TBCC |
| D2R receiver | Coherent receiver for device 1/2a, based on preamble, midamble and postamble. |