3GPP TSG-RAN WG4 Meeting #112-bis	                            R4-2414998

Hefei, China, 14 – 18 October, 2024

Agenda item:	6.21.3.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #102 meeting, a new SID[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- Coexistence study of Ambient IoT and NR/LTE.

- RF requirements study for Ambient IoT:

  - Ambient IoT BS transmission and reception

  - Ambient IoT Device, as per the General Scope, transmission and reception

  - Intermediate node (UE), as per the General Scope, transmission and reception

This is the first meeting of RF requirements discussion, in this contribution we list our analysis for A-IoT devices.

- Discussion

Agreement in RAN4#112: - Different RF requirement for Ambient IoT Device 1, Device 2a and Device 2b can be specified Tx (D2R), Agreement in RAN4#112: - Requirements potentially needed: - Transmit OFF power for device 2b - Power control requirement for device 2b - ON/OFF time mask for device 2b - Frequency error for device 2b - In band emissions (IBE) for device 2b - Carrier leakage for device 2b - [SEM] - [ACLR] - [Spurious emissions] - FFS whether requirements are needed or not: - Maximum output power - EVM - Transmit OFF power for device 1/2a - Transmit time mask - Minimum output power - Frequency error for device 2a - In band emissions (IBE) for device 2a - Carrier leakage for device 2a - Occupied bandwidth - Transmit intermodulation - Requirements not needed: - Power control requirement for device 1/2a - ON/OFF time mask for device 1/2a - Frequency error for device 1 - In band emissions (IBE) for device 1 - Carrier leakage for device 1 Rx (R2D), Agreement in RAN4#112: - Requirements potentially needed: - Reference sensitivity - FFS whether requirements are needed or not: - Maximum input power - Receiver intermodulation - Rx spurious emission - ACS - ACSC - In-band blocking - Out-of-band blocking - Requirements not needed: - Spurious response

2.1 Device 1

For device 1, considering its low cost, it’s better to use RFID spec as baseline which only has output power and unwanted emission requirements. Besides, for Rx part, it’s better to add REFSENSE requirements.

Proposal 1: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements. Besides, REFSENSE requirement is also needed.

2.2 Device 2a

Following table lists our initial analysis for Tx requirements of device 2a.

| requirements | Applicable or not |
| --- | --- |
| Transmitter power | All three kinds of device have different output power level. RAN4 further discuss how to define power class based on different device type and architecture. |
| Output power dynamic | Not applicable |
| Power control | Not applicable |
| Transmit signal quality | Frequency error: wait for modulation scheme conclusion EVM: wait for modulation scheme conclusion Carrier leakage: may be needed based on the design of small frequency shift in baseband In-band emission: needed. |
| RF spectrum emissions | Out of band emission: at least for standalone, it’s needed. For in-band and guard-band spectrum deployment mode, FFS how to define such requirements. Spurious emission requirement: needed to meet regulatory requirement. if large frequency shifter is considered for device 2a, RAN4 should further check the spurious emission at specific frequency where harmonic occurs. |

Proposal 2: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of How to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 3: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Following table list our initial analysis for Tx requirements.

| requirements | Applicable or not |
| --- | --- |
| REFSENSE | Necessary and needs to be separately defined for different devices types. |
| Max input level | Necessary and may needs to be separately defined for different devices types. |
| ACS | For standalone, FFS and details based on architecture. |
| blocking | In-band blocking: necessary Out of band blocking: Further discuss the out of band blocking performance based on RF architecture discussion |
| Spurious response | The same analysis as out of band blocking |
| Spurious emission | Spurious emission: to meet regulatory requirement |

Based on above analysis, we have following proposals.

Proposal 4: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 5: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

2.3 Device 2b

For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

Proposal 6: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 3. Conclusions

In this contribution, RF requirements for devices are discussed with following observations and proposals.

Proposal 1: for device 1, taking RFID RF requirements as reference which only define output power and unwanted emission requirements. Besides, REFSENSE requirement is also needed.

Proposal 2: at least for transmit signal quality related requirement, RAN4 should wait for RAN1 conclusion of How to achieve small frequency shift in baseband and/or FDM(A) among devices.

Proposal 3: if RAN4 only simulate in-band spectrum mode, RAN4 further discuss whether/how to define out of band emission requirement.

Proposal 4: REFSENSE and max input level needs to be separately defined for different devices types.

Proposal 5: RAN4 needs to further discuss the out of band blocking performance based on RF architecture discussion.

Proposal 6: For device 2b, it’s suggested to use UE RF framework as baseline and further discuss whether certain relaxation is needed or not.

# 4. Reference

- R4-2414305, WF on RF requirements for A-IoT, Huawei