3GPP TSG-RAN WG4 Meeting #112-bis	                            R4-2415000

Hefei, China, 14 – 18 October, 2024

Agenda item:	6.21.3.4

Source: 	CMCC

Title: 	Discussion on CW cancellation capability

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, some scope clarification for Rel-19 study item on ambient IOT had been discussed. Regarding the RAN4 scope, one conclusion had been reached regarding the CW interference cancellation as follows:

CW interference cancellation at reader RP-241763 asks that RAN4 performs the feasibility study of CW IC and captures the outcome in the TR.

In this contribution, we discuss the CW IC feasibility and provide on our views on how to capture the outcome in the TR.

- Discussion

RAN4 had evaluated self-interference feasibility in study item of SBFD. And the experience of SBFD is that interference cancellation capability is based on companies report and RAN4 categorize the total interference cancellation capability into several aspects and collect all companies’ results. In TR 38.858, the RSIC capability is broken down into four aspects: (1) spatial isolation; (2) frequency isolation; (3) beam nulling/isolation and (4) digital IC

The similar approach can also be used for the feasibility study of CW cancellation capability of ambient IOT reader. For example, CW interference cancellation capability can be broken down into: 1) spatial isolation, 2) RF cancellation, 3) digital cancellation part. The cancellation capability value and the detailed techniques to achieve such value (feasibility) are reported by companies. The detailed TR skeleton can be considered as:

6.7.5 Feasibility study 6.7.5.1 Feasibility study on CW cancellation for D1T1 reader 6.7.5.1.1 Company name … 6.7.5.2 Feasibility study on CW cancellation for D2T2 reader 6.7.5.2.1 Company name …

Proposal 1: The cancellation capability value and the detailed techniques to achieve such value (feasibility) are reported by companies and captured in TR.

Regarding the CW cancellation capability,

- For topology 1, if we assume 10cm isolation distance between CW and receiver Rx, at last 20dB isolation is achievable. For RFIC, simple circulator/coupler could at least achieve 40dBc. For digital IC, high pass filter would help to filter DC CW signal and at least 80dBc is achievable. Then for gNB like reader 1, 140~150dBc self-interference is achievable for inside topology.

- For topology 2, there is no spatial isolation between CW and Rx antenna. But the UE could achieve comparable self-interference cancellation with gNB considering the RFIC and digital IC is not much complex. So it’s assumed 120~130dBc self-interference cancellation is achievable for reader of topology 2.

Proposal 2: For topology 1, 140~150dBc CW cancellation capability can be achieved. For topology 2, 120~130dBc CW cancellation capability can be achieved.

# 3. Conclusions

In this contribution, we discuss the CW IC feasibility and provide on our views on how to capture the outcome in the TR. The proposals are:

Proposal 1: The cancellation capability value and the detailed techniques to achieve such value (feasibility) are reported by companies and captured in TR.

Proposal 2: For topology 1, 140~150dBc CW cancellation capability can be achieved. For topology 2, 120~130dBc CW cancellation capability can be achieved.

# 4. Reference

- RP‑242360, Moderator's summary for offline discussion on Ambient IoT

- R4-2414305, WF on RF requirements for A-IoT, Huawei