3GPP TSG-RAN WG4 Meeting #112bis     	R4-2415003

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.16.2

Source: 	CMCC

Title: 			 (NR_demod_Ph5-Perf) Discussion on interference suppressing performance for 8Rx UE

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some general test scope and test parameters about interference suppressing performance for 8Rx UE, the WF has been approved in [1].

In this contribution, we further discuss the network scenario, test scope and some test parameters.

- Discussion

  - General

PDSCH mapping type

- Candidate options:

  - Option 1: Include both PDSCH type A and type B requirements.

  - Option 2: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only

Only PDSCH TypeA was configured for Rel-17 2/4Rx ICI test case and Rel-18 8Rx case.

If PDSCH Type B is considered, we may need to discuss whether all of PDSCH symbols or part of PDSCH symbols are interfered by interference cell, since Type B usually means more flexible resource allocation, the serving cell and interference cell may have different PDSCH resource allocation.

Further, if we consider all of PDSCH symbols are interfered, we don’t expect much performance difference between mapping type A and mapping type B, since the MMSE-IRC algorithm is transparent to PDSCH mapping type. If we consider part of PDSCH symbols are interfered, it seems hard to find a typical deployment.

From the above, we prefer to keep the same configuration i.e., PDSCH type A as the legacy.

Proposal 1: Consider PDSCH mapping type A only.

  - PDSCH demodulation requirements with MMSE-IRC receiver for inter-cell interference scenario

Interference modelling and power levels

- Candidate options:

  - Option 1: Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4

- Option 1A: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx

- Option 1B: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx

  - Option 2: RAN4 to discuss if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell

In Rel-17 MMSE-IRC receiver for suppressing inter-cell interference research from Demodulation enhancement WI, both Homogeneous and Heterogeneous network deployment was finally specified. For Rel-19 8Rx UE MMSE-IRC receiver, these two interference scenarios should also be introduced, reuse the Rel-17 interference model as a baseline.

As for the MCS of interfering data, we think 16QAM can still be the baseline since the ICI scenario is usually happens at cell edge. Meanwhile, we are also open to investigate higher MCS, for example, 64QAM.

Proposal 2: Introduce both Homogeneous and Heterogeneous network deployment, reuse the Rel-17 interference model as a baseline.

Proposal 3: For interfering data MCS, use 16QAM as the baseline. Open to higher MCS, e.g., 64QAM.

Network Type and SCS/CBW

- Candidate options:

  - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

  - Other options are not precluded

We propose to reuse Rel-17 network type and SCS/CBW setup, which are:

FR1 FDD sync network, 15kHz/10MHz

FR1 TDD sync network, 30kHz/40MHz, TDD pattern 7D1S2U(S=6D+4G+4U)

Proposal 4: reuse Rel-17 network type and SCS/CBW setup, which are:

- FR1 FDD sync network, 15kHz/10MHz

- FR1 TDD sync network, 30kHz/40MHz, TDD pattern 7D1S2U(S=6D+4G+4U)

Rank

- Candidate options:

  - Option 1: Consider higher rank for 8Rx

- Option 1A: RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value

- Option 1B: Rank 2 and Rank 4 as the starting point

- Option 1C: Rank 2 and Rank 4 and Rank 8

- Option 1D: Rank 4

- Option 1E: Consider 2 layers, 4 layers and 6 layers

  - Option 2: Same as Rel-17 2/4Rx, i.e., Rank 1

In UE 8Rx performance requirement, Rank 2/4/8 was specified corresponding to 2/4/8Tx respectively. However, when we considering interference scenario, that UE locates on cell edge with low SINR, it is hard to reach 8 layer transmission. Hence, we propose to consider Rank 2 and Rank 4 as the starting point.

Proposal 5: For serving cell, consider Rank 2 and Rank 4 transmission as the starting point.

Propagation condition

- Candidate options:

  - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

  - Option 2: Consider TDLA30-10 as the starting point

In UE 8Rx performance requirement, TDLC300-100 is used for rank 2 transmission and TDLA30-10 is used for rank 4 transmission.

However, for TDLC300ns delay spread model, the maximum excess tap delay will reach 2595ns, the minimum coherence bandwidth is about 0.385MHz, which is smaller than 2PRB and larger than 1PRB in 30kHz scenario. For now, we are not sure whether 1PRB granularity could acquire relatively accurate interference-plus-noise covariance estimation result for 8Rx. Therefore, we propose to use TDLA30-10 as the baseline, and FFS whether interference suppressing gain can be reached in TDLC300-100 by simulation.

Proposal 6: For serving cell propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

MCS

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., MCS 13

  - Option 2: MCS 16

  - Option 3: MCS 19 for rank 2, MCS 17 for rank 4 and 8

  - Option 4: Define different parameters for FWA, Vehicular and Industrial deployments respectively

For serving cell MCS, we prefer to reuse the same configuration from 8Rx WI, which is MCS 19 (Table 1) for rank 2, MCS 17 (Table 1) for rank 4.

Proposal 7: For serving cell MCS, reuse the same configuration from 8Rx WI, which is MCS 19 (Table 1) for rank 2, MCS 17 (Table 1) for rank 4.

Other parameters

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx

  - Other options are not precluded

For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, we propose to reuse the configuration from 8Rx requirement as the baseline.

| Parameter |  | Value |
| --- | --- | --- |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL | Single-symbol for rank≤4 |
| TRS configuration |  | Symbol#{5, 9} |
| Overhead |  | 0 |
| Codebook for PDCCH for PDSCH tests |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Number of HARQ Processes |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  | Multiplexed |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% 256QAM:3% |
| Test metric |  | SNR@70% max TP |

Proposal 8: For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline.

For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, we propose to reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

| Parameter | Value |
| --- | --- |
| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 in case of 2 interference cells |
| SSB configuration | For homogeneous network, all SSBs (serving cell and interference cells) are in the same time/frequency resources, i.e., first SSB in Slot #0 for all cells. For heterogeneous network, serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources, i.e., first SSB in Slot #0 forserving cell, and second SSB in Slot #0 for interference cell. |
| Transmission rank of interfering PDSCH | random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s) |
| Precoding of interfering PDSCH | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |
| Modulation order of interfering PDSCH | Use 16QAM randomly modulated symbols for initial simulation assumption |
| Timing and frequency offset | The serving cell is 1.5 us for interfering cell 1 and -0.5 us for interfering cell 2 The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 |
| Interference cell modeling | 2Tx Interfering cell has same channel model as for serving cell and it is independently generated for each cell. Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant. |

Proposal 9: For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

  - CQI reporting requirements with MMSE-IRC receiver for inter-cell interference scenario

Test scope

- RAN4 to introduce UE demodulation requirements for both TDD and FDD

- Not to cover PDSCH absolute physical layer throughput requirements for 8Rx

- Candidate options on whether to cover CQI requirements for ICI scenario:

  - Option 1: CQI reporting requirements under ICI should be cover for 8Rx

  - Option 2: No need to cover such requirements

To verify 8Rx UE calculate CSI with MMSE-IRC receiver instead of MMSE-MRC, we support to cover CQI requirements for ICI scenario.

Proposal 10: Cover CQI requirements for ICI scenario.

Propagation condition

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell

  - Option 2: Use AWGN static channel as the baseline, further study whether to involve fading channel

Test metric

- Candidate options:

  - Option 1: Reuse the same test metric for Rel-17 2/4Rx

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

  - Other options are not precluded

Rel-17 2/4Rx used fading channel to verify UE performance of tracking the channel variations and selecting the largest transport format possible with interference. We can follow the same logic, reuse the propagation condition and test metric.

Proposal 11: For propagation condition and test metric, reuse the same configuration for Rel-17 2/4Rx.

Antenna configuration and MIMO correlation for the serving cell

- Candidate options:

  - Option 1: 2T8R ULA Low

  - Option 2: Consider 4T8R

Rank

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., rank 1

  - Option 2: Rank 2

  - Option 3: Rank 4

For antenna configuration, the setup 4T8R from 8Rx CQI reporting requirement can be reused.

For rank, due to inter-cell interference is considered, lower than Rank 4 reporting is also possible under low SINR scenario, we propose no RI restriction configured.

Proposal 12: Consider 4T8R and no RI restriction configured for simulation.

Interference power level

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

  - Other options are not precluded

INR 10.04dB can be reused for initial simulation.

Proposal 13: Reuse INR 10.04dB for initial simulation.

CSI-IM and NZP CSI-RS configurations

- Candidate options:

  - Option 1: Reuse the same configuration for Rel-17 2/4Rx

  - Other options are not precluded

The agreement from R17 2/4Rx inter-cell interference suppressing requirement can be reused for simulation, that:

CSI-IM on serving cell overlapping with PDSCH from interference cell, and

NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

Proposal 14: For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

  - PDSCH demodulation requirements with MMSE-IRC receiver for intra-cell inter-user interference scenario

Interference modelling

- Candidate options on MU-MIMO Beamforming Model:

  - Option 1: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

  - Option 2: Include in a new clause of Annex B.6 of TS 38.101-4

- Candidate options on co-UE number:

  - Option 1: Same as Rel-17 2/4Rx requirements, i.e., 1 target UE + 1 co-UE

  - Option 2: Consider 1 target UE and more than 1 co-UEs

As in WID, the existing interference model for 2/4Rx UE should be used as the baseline, which one target UE with one co-scheduled UE.

Besides, for 8Tx scenario, more than one paired UE may be scheduled in real MU-MIMO network. We also have interest to further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in such scenario.

Proposal 15: Use 1 target UE + 1 interference UE as starting point for initial simulation.

Proposal 16: For 8Tx, further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in more than one paired UE scenario.

As for MU-MIMO Beamforming model, we think the legacy model defined in B.4.2 in TS38.101-4 can be reused, it can be applied for both one co-scheduled UE and multiple co-scheduled UEs case.

Proposal 17: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

Network Type and SCS/CBW

- Candidate options:

  - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD 7D1S2U

  - Other options are not precluded

To minimize test effort, we propose to test the most common configuration, that FDD 15kHz SCS with 10MHz CBW, and TDD 30kHz SCS with 40MHz CBW and 7D1S2U TDD pattern.

Proposal 18: Use FDD 15kHz/10MHz and TDD 30kHz/40MHz 7D1S2U for initial simulation

Propagation condition

- Candidate options:

  - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

  - Option 2: Consider TDLA30-10

TDLC300-100 and TDLA30-10 was usually used for define requirements.

However, for TDLC300ns delay spread model, the maximum excess tap delay will reach 2595ns, the minimum coherence bandwidth is about 0.385MHz, which is smaller than 2PRB and larger than 1PRB in 30kHz scenario. For now, we are not sure whether 1PRB granularity could acquire relatively accurate interference-plus-noise covariance estimation result for 8Rx. Therefore, we propose to use TDLA30-10 as the baseline, and FFS whether interference suppressing gain can be reached in TDLC300-100 by simulation.

Proposal 19: For propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

- Candidate options:

  - Option 1: 2T8R ULA low

  - Option 2: Rank 4+2 with 8T8R

  - Option 3:

- Rank 2+2 for 2T8R

- Rank 4+4 for 4T8R

- Rank 8+8 for 8T8R when 1 paired UE scheduled

  - Option 4:

- Rank 1+1 for 2T8R

- Rank 2+2 for 4T8R

- Rank 2+6 or rank 6+6 for 8T8R

  - Option 5:

- Rank 2+2+2 for 8T8R

- Rank 4+4 for 4T8R

  - Option 6:

- Rank 2+2

- Rank 4+4

We propose to cover 2T8R, 4T8R and 8T8R for initial simulation. Same rank value for both target and paired UE can be considered for simplification, Rank 1+1 for 2T8R, Rank 2+2 for 4T8R, Rank 4+4 for 8T8R when 1 paired UE scheduled.

Proposal 20: Following rank and antenna configuration can be used for initial simulation

- Rank 1+1 for 2T8R

- Rank 2+2 for 4T8R

- Rank 4+4 for 8T8R when 1 paired UE scheduled

MCS

- Candidate options for the target UE:

  - Option 1: Same as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13

  - Option 2:

- MCS 19 Table 1 for Rank 2

- MCS 17 Table 1 for Rank 4

- MCS 17 Table 1 for Rank 8

  - Option 3: Consider MCS 17 and MCS 13

  - Option 4: MCS 19 for rank 2, MCS 13 for rank 4

For target UE, propose to use MCS19 Table 1 for Rank 1, MCS17 Table 1 for Rank 2, MCS13 Table 1 for Rank 4. For paired UE, propose to use random 16QAM signal generation as legacy.

Proposal 21: Following MCS configuration can be used for initial simulation

- For target UE, MCS19 Table 1 for Rank 1, MCS17 Table 1 for Rank 2 and MCS 13 Table 1 for Rank 4

- For paired UE, random 16QAM

Precoding method for the co-scheduled UE

- Candidate options:

  - Option 1: Reuse the same precoding method as MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

  - Option 2: Orthogonal precoding with the target UE, i.e., any column of matrix of any serving UE or co-scheduled UE is orthogonal to any column of matrix of any other UE

Similar as 8Rx requirement, single Panel Type I codebook can be taken as the baseline. For target UE, use random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination. For co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE. The precoding granularity and PRB bundling size can be assumed as 2PRBs.

Proposal 22: For initial simulation, use following configuration as the baseline:

- single Panel Type I codebook

- precoding granularity and PRB bundling size set as 2PRBs

- for target UE, random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination

- for co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE

  - Applicability rule and release independent

Test applicability

- Candidate options:

  - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

  - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

Both Option 1 and Option 2 follows the legacy applicability rule, which ensure UE pass the related test case with minimum test effort. We support both of them.

Proposal 23: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 24: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

Release independent

If no new signalling or UE capability introduced, the requirement should be released independent from Rel-18.

Proposal 25: the requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

- Conclusion

In this contribution, we provides some basic simulation assumptions for UE 8Rx interference suppressing requirement. The proposals are:

Proposal 1: Consider PDSCH mapping type A only.

Proposal 2: Introduce both Homogeneous and Heterogeneous network deployment, reuse the Rel-17 interference model as a baseline.

Proposal 3: For interfering data MCS, use 16QAM as the baseline. Open to higher MCS, e.g., 64QAM.

Proposal 4: reuse Rel-17 network type and SCS/CBW setup, which are:

- FR1 FDD sync network, 15kHz/10MHz

- FR1 TDD sync network, 30kHz/40MHz, TDD pattern 7D1S2U(S=6D+4G+4U)

Proposal 5: For serving cell, consider Rank 2 and Rank 4 transmission as the starting point.

Proposal 6: For serving cell propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Proposal 7: For serving cell MCS, reuse the same configuration from 8Rx WI, which is MCS 19 (Table 1) for rank 2, MCS 17 (Table 1) for rank 4.

Proposal 8: For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline.

Proposal 9: For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline.

Proposal 10: Cover CQI requirements for ICI scenario.

Proposal 11: For propagation condition and test metric, reuse the same configuration for Rel-17 2/4Rx.

Proposal 12: Consider 4T8R and no RI restriction configured for simulation.

Proposal 13: Reuse INR 10.04dB for initial simulation.

Proposal 14: For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are:

- CSI-IM on serving cell overlapping with PDSCH from interference cell

- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

Proposal 15: Use 1 target UE + 1 interference UE as starting point for initial simulation.

Proposal 16: For 8Tx, further study whether interference suppressing gain can be achieved by MMSE-IRC receiver in more than one paired UE scenario.

Proposal 17: Reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4

Proposal 18: Use FDD 15kHz/10MHz and TDD 30kHz/40MHz 7D1S2U for initial simulation

Proposal 19: For propagation condition, consider TDLA30-10 as the starting point, and further decide whether to cover TDLC300-100 by simulating interference suppressing gain.

Proposal 20: Following rank and antenna configuration can be used for initial simulation

- Rank 1+1 for 2T8R

- Rank 2+2 for 4T8R

- Rank 4+4 for 8T8R when 1 paired UE scheduled

Proposal 21: Following MCS configuration can be used for initial simulation

- For target UE, MCS19 Table 1 for Rank 1, MCS17 Table 1 for Rank 2 and MCS 13 Table 1 for Rank 4

- For paired UE, random 16QAM

Proposal 22: For initial simulation, use following configuration as the baseline:

- single Panel Type I codebook

- precoding granularity and PRB bundling size set as 2PRBs

- for target UE, random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination

- for co-scheduled UE, select the precoding matrix to ensure orthogonality with target UE

Proposal 23: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule

Proposal 24: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

Proposal 25: the requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

- Reference

[1] R4-2413570, WF on NR demodulation performance: Phase 5, China Telecom