3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415005

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.1

Source: 	CMCC

Title: 			 (Netw_Energy_NR_enh-Core) Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some general issues and work plans about three objectives in NES enhancement WI. The WF has been approved in [1]. In this paper, we discuss OD-SSB related issues, and identify RAN4 impacts according to the progress of other working groups.

- On-demand SSB(OD-SSB) requirements

Issue 1-1-1: OD-SSB work plan

- Agreement:

  - RAN4 to first focus on the MAC CE based single SCell activation case as starting point.

  - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed

In RAN1#117 and RAN2#127 meeting, following agreements has been achieved:

Agreement from RAN1: For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement from RAN2: RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

Since RAN1 has agreed to support RRC based OD-SSB transmission indication in the case of direct SCell activation, after RAN4 concludes on the MAC CE based single SCell activation case, the OD-SSB based direct SCell activation delay requirement should be specified at least.

Proposal 1:  After RAN4 concludes on the MAC CE based single SCell activation case, at least specify the OD-SSB based direct SCell activation delay requirement.

Issue 1-1-3: Deactivated SCell measurement requirement

- Agreement:

  - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement.

    - Includes both FR1 and FR2-1

- For L3 measurement, RAN1#117 and RAN2#127 meeting achieved following agreements:

Agreement from RAN1: For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement from RAN2: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

OD-SSB based L3 measurement has been supported by RAN1 and RAN2. However, how the UE to perform L3 measurement is still in the discussion now.

In current spec, UE will perform L3 deactivated SCell measurement during deactivation period, the requirement is specified in TS38.133 Clause 9.2.5.1 and 9.2.5.2, which rely on the parameter ‘measCycleSCell’ configured in MeasObject IE by network. After on-demand SSB triggering, if UE still perform measurement sampling refer to the value of ‘measCycleSCell’, which is equal to or larger than the maximum SSB period 160ms, the power will be wasted since network transmit the on-demand SSB densely while UE sample the on-demand SSB sparsely. Therefore, we think the L3 deactivated SCell measurement requirement should be re-designed.

Based on our understanding, the legacy configuration and mapping relationship between measurement objective and measurement report should be reused, when OD-SSB is activated, the OD-SSB based measurement configuration should be configured or activated, UE performs the L3 measurement according to the SMTC or measurement period configuration in MeasObject.

Proposal 2:  For L3 deactivated SCell measurement, after OD-SSB is activated, the OD-SSB based measurement configuration should also be configured or activated,  the specific procedure can be designed by RAN2.

Proposal 3: UE performs the L3 measurement according to the SMTC or measurement period configuration in MeasObject from OD-SSB based measurement configuration.

Transition between always-on SSB and OD-SSB

For Case#2 that Always-on SSB is periodically transmitted on the cell, we think UE can perform the legacy L3 measurement follows ‘measCycleSCell’ before on-demand SSB triggering, and perform new OD-SSB based L3 measurement after OD-SSB triggering.

Proposal 4: For Case#2 that Always-on SSB is periodically transmitted on the cell, UE can perform the legacy L3 measurement follows ‘measCycleSCell’ before on-demand SSB triggering, and perform new OD-SSB based L3 measurement after OD-SSB triggering.

By such UE behavior, there will be a transition period between measurement performed on always-on SSB and measurement performed on OD-SSB.

If always-on SSB and OD-SSB are on the same frequency location with different periodicity, we think the requirements should apply based on the shorter delay before or after the transition when SSB type changes, and the requirement corresponding to the second mode apply after transition.

If always-on SSB and OD-SSB are not on the same frequency location, we think UE shall be allowed to restart the ongoing evaluation using the second SSB type after OD-SSB ON/OFF. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type.

Proposal 5: For the transition period between measurement performed on always-on SSB and measurement performed on OD-SSB:

- If always-on SSB and OD-SSB are on the same frequency location with different periodicity, we think the requirements apply based on the shorter delay before or after the transition when SSB type changes, and the requirement corresponding to the second mode apply after transition.

- If always-on SSB and OD-SSB are not on the same frequency location, we think UE shall be allowed to restart the ongoing evaluation using the second SSB type after OD-SSB ON/OFF. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type.

In addition, since UE will not perform legacy always-on SSB based L3 measurement and OD-SSB based L3 measurement simultaneously, the CSSF assumption and algorithm can be kept as existing way.

Proposal 6: The legacy CSSF can be reused.

Scell activation requirement --- Known and Unknown condition

For legacy SCell activation, the delay requirement was defined in two general cases, which are SCell known case and unknown case. The known case requires UE to sent a valid measurement report during a period before the reception of the SCell activation command. In FR1, the period equal to max(5*measCycleSCell,  5*DRX cycles), which related to the L3 measurement requirement. While as analyzed above, if OD-SSB is activated before SCell activation command, the L3 measurement requirement should be re-designed to achieve network energy saving gain, hence the condition of the known case should be re-designed accordingly.

Proposal 7:  The legacy logic of known condition can be reused, which requires UE to sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

Proposal 8: For the case of OD-SSB is activated before SCell activation command (Scenario#2), RAN4 further update the ‘period’, which should be related with the new L3 deactivated SCell measurement requirement.

Scell activation requirement --- delay requirement

Although the known and unknown condition is unsure now, we think the requirement can still be discussed in these two main cases, known case and unknown case, and the legacy methodology of requirement defining can be reused.

For known case and measurement sampling rate before SCell activation command smaller than or equal to 160ms, fine sync time is needed in SCell activation delay.

For known case and measurement sampling rate before SCell activation command larger than 160ms, AGC and fine sync time is needed in SCell activation delay.

For unknown case, AGC and time sync is needed in SCell activation delay.

For Scenario#2 and Scenario#2A, when network will stop transmit the OD-SSB and how to indicate it to UE is unclear now, thus which RS will be used to perform sync and AGC need to be further studied until more input from RAN1 and RAN2.

Proposal 9: For SCell activation delay requirement, the legacy methodology of defining requirements can be reused, which are:

- For known case and measurement sampling rate before SCell activation command smaller than or equal to 160ms, fine sync time is needed in SCell activation delay.

- For known case and measurement sampling rate before SCell activation command larger than 160ms, AGC and fine sync time is needed in SCell activation delay.

- For unknown case, AGC and time sync is needed in SCell activation delay.

LS on timeline for On-demand SSB operation on SCell

In last meeting, RAN1 achieved following agreement and sent the LS to RAN4 for asking if the working assumption of T_min=m+3Nslotsubframe,μ+1 can be confirmed.

Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min= m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and is as defined in current specification. - RAN4 to confirm that T_min can be equal to - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

After UE receiving OD-SSB indication by MAC-CE in slot n, UE need m slots for feedback HARQ-ACK and 3ms = 3Nslotsubframe,μ for MAC-CE processing. As long as OD-SSB is in the active BWP, no RF re-tuning time is needed. Therefore, we think the working assumption of T_min=m+3Nslotsubframe,μ+1 can be confirmed.

Proposal 10: The working assumption of T_min=m+3Nslotsubframe,μ+1 can be confirmed.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1:  After RAN4 concludes on the MAC CE based single SCell activation case, at least specify the OD-SSB based direct SCell activation delay requirement.

Proposal 2:  For L3 deactivated SCell measurement, after OD-SSB is activated, the OD-SSB based measurement configuration should also be configured or activated,  the specific procedure can be designed by RAN2.

Proposal 3: UE performs the L3 measurement according to the SMTC or measurement period configuration in MeasObject from OD-SSB based measurement configuration.

Proposal 4: For Case#2 that Always-on SSB is periodically transmitted on the cell, UE can perform the legacy L3 measurement follows ‘measCycleSCell’ before on-demand SSB triggering, and perform new OD-SSB based L3 measurement after OD-SSB triggering.

Proposal 5: For the transition period between measurement performed on always-on SSB and measurement performed on OD-SSB:

- If always-on SSB and OD-SSB are on the same frequency location with different periodicity, we think the requirements apply based on the shorter delay before or after the transition when SSB type changes, and the requirement corresponding to the second mode apply after transition.

- If always-on SSB and OD-SSB are not on the same frequency location, we think UE shall be allowed to restart the ongoing evaluation using the second SSB type after OD-SSB ON/OFF. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type.

Proposal 6: The legacy CSSF can be reused.

Proposal 7:  The legacy logic of known condition can be reused, which requires UE to sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

Proposal 8: For the case of OD-SSB is activated before SCell activation command (Scenario#2), RAN4 further update the ‘period’, which should be related with the new L3 deactivated SCell measurement requirement.

Proposal 9: For SCell activation delay requirement, the legacy methodology of defining requirements can be reused, which are:

- For known case and measurement sampling rate before SCell activation command smaller than or equal to 160ms, fine sync time is needed in SCell activation delay.

- For known case and measurement sampling rate before SCell activation command larger than 160ms, AGC and fine sync time is needed in SCell activation delay.

- For unknown case, AGC and time sync is needed in SCell activation delay.

Proposal 10: The working assumption of T_min=m+3Nslotsubframe,μ+1 can be confirmed.

- Reference

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson