3GPP TSG-RAN WG4 Meeting #112bis		R4-2415006

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.2

Source: 	CMCC

Title: 			 (Netw_Energy_NR_enh-Core) Discussion on RRM requirement of signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some general issues and work plans about three objectives in NES enhancement WI.  The WF has been approved in [1]. In this paper, we discuss signal adaptation related issues, and identify RAN4 impacts according to the progress of other working group.

- Adaptation of common signal/channel transmissions

SSB adaptation

For the scenario of SSB adaptation, RAN1 concluded following agreements [3]:

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Currently, RAN1 only agreed to adapt the NCD-SSB in time-domain for Rel-19 NES-capable UE’s SCell. RAN4 can also start the SSB adaptation discussion for SCell NCD-SSB, including related L1 and L3 measurement.

Proposal 1: RAN4 also starts the SSB adaptation discussion for SCell NCD-SSB, including related L1 and L3 measurement.

For the mechanism of SSB adaptation, RAN1 concluded following agreements[4]:

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) is supported, which means there will be SSB periodicity transitions during the measurement.

For the delay requirement during the transition period, we propose to follow the legacy method, that the requirements apply based on the longer delay before or after the transition when SSB periodicity changes, and the requirement corresponding to the second mode apply after transition.

Proposal 2: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: RAN4 also starts the SSB adaptation discussion for SCell NCD-SSB, including related L1 and L3 measurement.

Proposal 2: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied.

- Reference

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson