3GPP TSG-RAN WG4 Meeting # 112bis                                                            R4-2415014

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.10.2.2

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with inter-band CA

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG inter-band CA. This paper will focus on the remaining issues.

# 2. Discussion

## 2.1. n39 UL for CA_n3A-n39A

During last meeting, we have reached the agreement as below.

Agreement: • Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input. - The filter with [X] MHz frequency gap can be considered.

The RF architecture of handheld UE supporting CA_n3A-n39A is shown below. In the current spec, only n39 UL is considered and companies had checked the cross-band isolation from band n3 UL to band n39 DL due to the back effect inside n39. In our opinion, considering the RF architecture difference between handheld UE and CPE, the requirements for legacy UE could not be directly reused for ATG UE, and n39 UL should be considered for ATG CA.

Proposal 1:  Considering the RF architecture difference between handheld UE and CPE, n39 UL should be considered for ATG CA.

Figure 1. RF reference architecture for handheld UE CA_n3-n39

The main problem solved by different RF architectures is the interference from n39 UL to n3 DL, so we reached the agreement that the filter with [X] MHz frequency gap can be considered. Referring to the deployment between these two bands n39 and band n3 in China, there are 5MHz frequency gap. So 5MHz gap as guard band for n39 UL filter could be used.

Proposal 2:  The filter with 5 MHz frequency gap can be used to achieve n39 UL for CA_n3A-n39A ATG.

# 3. Conclusions

In this contribution, ATG UE with inter band CA requirements are discussed with the following proposals:

Proposal 1:  Considering the RF architecture difference between handheld UE and CPE, n39 UL should be considered for ATG CA.

Proposal 2:  The filter with 5 MHz frequency gap can be used to achieve n39 UL for CA_n3A-n39A ATG.

# 4. References

[1] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR