3GPP TSG-RAN WG4 Meeting # 112bis                                                           R4-2415015

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.10.2.1

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with intra-band contiguous CA

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG intra-band CA. This paper will focus on the remaining issues.

# 2. Discussion

ACS requirements for ATG UE without CA had been specified in TS 38.101-1 sub-clause 7.5J. Since the example band combination is CA_n79C, we only consider case that frequency range larger than 3300MHz. The ACS value in clause 7.5A.1 could be reused or specified as the following tables.

Table 1: ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |
| --- | --- | --- |
| Rx Parameter | Units | C |
| ACS | dB | 33.0 |

Table 2: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 31.5 dB |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Test parameters case 2 in clause 7.5A.1 should to be updated.

Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

Proposal 1: The ACS principle and requirements in clause 7.5A.1 for intra-band contiguous CA could be reused except for the test parameters case 2, and case 2 could be defined as Table 3.

# 3. Conclusions

In this contribution, ATG UE with intra-band CA requirements are discussed with the following proposals:

Proposal 1: The ACS principle and requirements in clause 7.5A.1 for intra-band contiguous CA could be reused except for the test parameters case 2, and case 2 could be defined as Table 3.

Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

# 4. References

[1] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR