3GPP TSG-RAN WG4 Meeting # 112bis                                  R4-2415018

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.23.2.1

Source: 	CMCC

Title: 	Discussion on LP-WUS UE system parameters requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#110-bis, the LP-WUS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about system parameters.

- Discussion

2.1 one or multi-Sets of requirements

Last meeting we have reached the agreement below

Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver.

LP-WUS waveform and modulation mode are significantly different from the legacy NR signal, which will affect the final PAPR. The distinct LP-WUS signal implies different categories of receivers. Considering the differences between these two types of receivers, it is challenging to establish a single set of requirements that can satisfy all RF requirements.

According to the current research progress of the RAN1, OOK receivers utilize envelope detection, whereas OFDM receivers employ sequence detection. Moreover, OFDM receivers demonstrate superior performance, which also implies that while ensuring the same coverage distance for the Message 3, OFDM receivers achieve a better Signal-to-Noise Ratio (SNR). In addition, based on our simulation results, the OFDM receiver actually exhibits a better SNR.

Table 1 SNR@1%BLER with 4bit ADC under AWGN

| Modulation | Channel structure | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-1 | 8bit | Envelope | -6.5dB |
|  |  | OFDM | -10.6dB |
|  | 16bit | Envelope | -6.1dB |
|  |  | OFDM | -10.3dB |
| OOK-4 M=2 | 8bit | Envelope | -4.7dB |
|  |  | OFDM | -7.6dB |
|  | 16bit | Envelope | -4.2dB |
|  |  | OFDM | -7.2dB |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -3.7dB |
|  | 16bit | Envelope | -2.2dB |
|  |  | OFDM | -3.3dB |

Proposal 1: SNR should be different for these two types.

### 2.2 Channel raster for LP-WUR

Issue 1-2-1: Channel raster for LP-WUR Agreements: - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS. - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

Both in-band case and standalone case should be considered at this stage. For the standalone case, the center frequency should correspond to the channel raster frequency points of the corresponding working frequency bands. For in-band case, since LP-WUS RBs will be transmitted together with NR RBs, the existing requirements of NR channel raster could be reused for LP-WUS. In summary, channel raster is needed and the existing requirements could be reused.

Proposal 2: Channel raster is needed for LP-WUS and the existing requirements could be reused.

### 2.3 Channel bandwidth and RB numbers

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS Agreements: - Leave RAN1 to make decision. - RAN4 can discuss assumption of RBs under 15kHz SCS for simulation

For RB number for 15KHz SCS depending on RAN1 decision. RAN1 had two options for the number of RBs, 11 RBs and 22 RBs, and these two options should be considered by RAN4.

Proposal 3: Consider 11RBs and 22RBs for LP-WUS signal with 15kHz SCS for simulation.

# 3. Conclusions

Proposal 1: SNR should be different for these two types.

Proposal 2: Channel raster is needed for LP-WUS and the existing requirements could be reused.

Proposal 3: Consider 11RBs and 22RBs for LP-WUS signal with 15kHz SCS for simulation.

# 4. Reference

- R4-2414309, WF on LP-WUS UE RF requirements