3GPP TSG-RAN WG4 Meeting # 112bis                                                            R4-2415020

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.23.2.2

Source: 	CMCC

Title: 	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In this contribution, we focus on discuss of UE LP-WUS RF requirements including REFSENSE, ACS and ASCS.

- Discussion

2.1 REFSENSE requirements

During the last meeting, RAN4 reached the following agreement below.

Issue 2-1-1: Channel model to specify LP-WUS RF requirements Agreement: - Follow typical RF requirements approach, AWGN should be selected. Issue 2-1-2: Target SNR simulation condition Agreement: - Follow similar approach of MR NR, no repetition should be used when simulate target SNR. Issue 2-1-3: Whether RAN4 should conclude a target SNR first Agreement: - Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first. Issue 2-1-4: Target SNR value Agreements: - RAN4 further discuss target SNR value for LP-WUS - FFS single SNR for OOK-based and OFDM-based receiver

SNR value for REFSENS

Considering SNR is relevant to RF and demodulation and RAN1 has two options of OFDM signal and OOK signal, the required SNR for demodulation on OOK and OFDM sequence could be different, so two sets of SNR values could be specified. Based on the simulation results, OFDM receiver perform better than OOK receiver. Since the case OOK-4, M=4 has the worst performance, target SNR should be chosen based on this case.

Table 1 SNR@1%BLER with 4bit ADC under AWGN

| Modulation | Channel structure | Receiver type | SNR@1% BLER |
| --- | --- | --- | --- |
| OOK-1 | 8bit | Envelope | -6.5dB |
|  |  | OFDM | -10.6dB |
|  | 16bit | Envelope | -6.1dB |
|  |  | OFDM | -10.3dB |
| OOK-4 M=2 | 8bit | Envelope | -4.7dB |
|  |  | OFDM | -7.6dB |
|  | 16bit | Envelope | -4.2dB |
|  |  | OFDM | -7.2dB |
| OOK-4 M=4 | 8bit | Envelope | -2.5dB |
|  |  | OFDM | -3.7dB |
|  | 16bit | Envelope | -2.2dB |
|  |  | OFDM | -3.3dB |

Proposal 1: Specify two sets of SNR for LP-WUS based on the case OOK-4 and M=4, such as 2dB for OOK and 3dB for OFDM.

NF Gap between LR and MR

Since RAN4 had an agreement about SNR evaluation:

Agreement: Issue 2-2-2: Baseline architecture for OOK-based LP-WUS Agreement: - use zero-IF receiver as a baseline architecture for envelop based LP_WUR Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS Agreements: - Wait for RAN1’s conclusion on the OFDM-based LP-WUR. Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver. Issue 2-2-9: MDR value for REFSENS test case Agreements: FFS the MDR for conformance testing should be aligned with the value for RF requirement.

Considering the difference between OFDM and OOK signals, two sets of noise figure requirements should be specified. For the OFDM signal, the NF value of the OFDM receiver and legacy UE receiver are similar, so we could assume the legacy UE receiver noise figure value 9dB as the baseline for the OFDM signal.

Proposal 2: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

For the OOK signal receiver, the NF value is much larger than the NF of legacy UE. During the study item phase, we listed some example NF value ranges:

- RF envelope detection: The noise figure is in the range of 12~22 dB.

Consider zero-IF receiver as a baseline architecture for envelop based LP_WUR. We could choose the same NF range for the LP-WUS signal, which is 12-15dB. Since RAN1 chooses 7dB as the NF of MR, and the gap between MR and LR is 2/5/8dB, it is reasonable to choose 12-15dB as the noise figure for the OOK signal.

Proposal 3: Choose 12-15 noise figure as the baseline for the OOK signal.

2.1 ASCS and ACS

When defining ACS and ASCS for LP-WUR, two factors should be considered: the ASCS value on a dB scale and the required guard RB. If we only define the guard RB requirement without specifying an ASCS value, there would be no restriction on the in-band selectivity of LP-WUR. In extreme cases, even LP-WUR with the worst performance might pass the test, as there is only a guard RB limitation. Conversely, if we only define the ASCS value without any guard RB limitation, LP-WUR with the worst performance could also pass the test in extreme cases, as long as the guard RB is large enough.

Since ACS and ASCS cannot be specified directly, we should define the test case for the ASCS requirement in the specification. The test parameters should include the power level, bandwidth, MDR, and other relevant parameters. The MDR will be verified in the test with the guard RBs as defined in the specification.

Proposal 4: For ASCS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

# 3. Conclusions

In this contribution, ASCS, ACS and REFSENSE are discussed with following observations and proposals.

Proposal 1: Specify two sets of SNR for LP-WUS based on the case OOK-4 and M=4, such as 2dB for OOK and 3dB for OFDM.

Proposal 2: Set 9dB as the OFDM noise figure for the OFDM signal as the baseline.

Proposal 3: Choose 12-15 noise figure as the baseline for the OOK signal.

Proposal 4: For ASCS requirement, RAN4 should specify the both guard RB and ASCS/ACS requirements.

# 4. Reference

- R4-2414309, WF on LP-WUS UE RF requirements