3GPP TSG-RAN WG4 Meeting #112bis										R4-2415023

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.20.2.2

Source: 	CMCC

Title: 	Discussion on existing Tx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements.[1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACLR and OBUE

Following capture the agreements from TR 38.858 for ACLR requirements:

For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

And during last meeting, we have reached the following agreement:

Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

For the SBFD network, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. But 0% grid 6 shift is the worst case. So it seems co-location ACLR requirements or equivalent requirements are needed. We list two options:

Option 1: Not define co-location ACLR requirement. Add the note in the spec that the ACLR requirement is not applied to the co-location scenario.

Option 2: Define the co-location ACLR requirement. If co-location ACLR is defined, it is necessary to ensure that the receiver of the BS is not blocked.

Proposal 1: If co-location ACLR is defined, then it is necessary to ensure that the receiver of the BS is not blocked, otherwise the note in the spec that the ACLR requirement is not applied to the co-location scenario is needed.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: If co-location ACLR is defined, then it is necessary to ensure that the receiver of the BS is not blocked, otherwise the note in the spec that the ACLR requirement is not applied to the co-location scenario is needed

# 4. Reference

[1] R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF