# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415025

# Hefei, China, 14th – 18th October, 2024

Agenda item:	6.8.2.1

Source: 	CMCC

Title: 	co-existence study for NTN HPUE

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, a new WID[1] of enhanced requirements and test methodology for NR and IoT NTN is approved with following RAN4 lead RF part objectives:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

After lasting meeting discussion, we have made good progress on simulation parameter discussion. This paper will focus on the deployment and layout issue.

- Discussion

During last meeting, we have reached the following agreement.

Issue 2-3-1 NTN and TN network isolation distance Agreement: - Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex. - Use the isolation distance 1.5km as the starting point - Other values for isolation distance are not precluded

Figure 1 Isolation distance

Following lists the simulation scenarios that are listed to be studied.

Table 1: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

We could focus on Scenario 4 and Scenario 5, where the aggressor is NTN UL and the victim is TN UL/DL. A distance of 1.5 km has been chosen as the isolation distance for both Scenario 4 and 5. In our opinion, since these two scenarios are designed to evaluate the impact on the uplink performance of the TN, ensuring the performance of the TN network is the primary concern we need to address.

Given the actual deployment of the legacy TN network, we cannot guarantee that all NTN HPUE will be used with a 1.5 km isolation from the edge of the TN network, and the RF requirements for NTN HPUE do not provide adequate protection for the TN network's downlink. Furthermore, a geographical separation of 1.5 km would impose a significant limitation on the usage scenarios of NTN HPUE.

Therefore, we believe that scenarios with a 0 km isolation distance should also be considered in the study.

Proposal 1: 0km isolation distance should also be considered for co-existence evaluation..

# 3. Conclusions

Proposal 1: 0km isolation distance should also be considered for co-existence evaluation..

# 4. Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN