# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415029

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.2.2

Source: 	OPPO

Title: 	AIML two-sided model logistical issues

Document for:	Approval

- Discussion

As discussed in the last RAN meeting, logistical issues related to the two-sided AIML model should be addressed in RAN4. In this contribution, we will elaborate on some key issues related to data and model sharing, including storage size, format and other relevant details.

- About Data

Regarding training dataset size (for 32T4R, 13 sub-bands, and 32 bits for each I and Q), 10G per use case for data storage for each company may meet the demand. For example:

- If we upload raw data H, 32 * 4 * 13 * 2 * 32/8 bytes = 13,312 Byte are needed per sample. With 600k samples for training, the training data size would be about 8 GB.

- If we upload eigenvector W, 32 * 13 * 2 * 32/8 bytes = 3,328 Byte are needed per sample. With 600k samples for training, the training data size would be about 2 GB.

In the email discussion after RAN4#112 meeting, following proposals have been considered:

- Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- Total space needed would depend on the number of contributing companies and meetings

- Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies

- Data sharing might not be needed in every meeting

In general, we are OK with the assumptions above. Additionally, we may need to consider and verify whether the training data for different test settings or side conditions can be shared and reused. If not, RAN4 should estimate the number of datasets needed for different test settings or side conditions. Correspondingly, the necessary storage capacity should be increased as well.

Proposal 1: Regarding training dataset size

- Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- Total space needed would depend on the number of contributing companies and meetings

- Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies

  - Data sharing might not be needed in every meeting

  - FFS whether the training data for different test settings or side conditions can be shared and reused

  - FFS the number of datasets needed for different test settings or side conditions

For the test data set, it's typically sufficient for these to constitute approximately 10% of the training data set. Hence, if we're only considering the total data storage space, the space prepared for the training set now is enough to cover the test sets. Additionally, the testing sets and also the validation sets can be derived from the existing training data set.

Proposal 2: The space prepared for the training set now is enough to cover the test and validation sets

Regarding the data format, both .mat (Matlab) and .npy (Numpy) are OK. Considering  .mat (Matlab) may have potentially IP/license issues that need to be further checked. We can use .npy (Numpy) format to do the data transfer as the starting point.

Proposal 3: Use .npy (Numpy) format to do the data transfer as the starting point

- About Model

Regarding model size, for the CNN based CSI model in RAN4 discussion, our simulated results are shown in the table. In general, these results are comparable with other companies' outputs. For transformer based solution, the model size could be extended to about 40MB. Considering more advanced AI/ML solutions may be proposed in the future, reserving 100MB per model for storage could be a preferred size.

During the mail discussion, following proposals have been considered.

- Model size: ~7MB – 100MB

- Total space needed would depend on the number of companies contributing and meetings

- Upper bound should be 600MB per company per year -> ~10GB per year

As highlighted in the data section, for model storage, it's crucial to determine whether a model for different test settings or side conditions could be reused (likely not). RAN4 should consider the number of models needed for different test settings or side conditions. Accordingly, an increase in storage capacity will likely be necessary as well.

Proposal 4: Regarding model size

- Model size: ~7MB – 100MB per model

- Total space needed would depend on the number of companies contributing and meetings

- Upper bound should be 600MB per company per year -> ~10GB per year

  - FFS whether a model for different test settings or side conditions could be reused

  - FFS the number of models needed for different test settings or side conditions

Regarding the model format, TensorFlow and PyTorch are the two most prevalent and widely utilized formats. From our understanding, for RAN4 evaluation, participating companies can provide reference models using TensorFlow or PyTorch. If any company faces difficulties in providing reference models in these two formats, we can further consider what are the challenges they encounter. In addition, if a company needs to use another model format, they can generate it by ONNX or other methods based on their needs.

Proposal 5: Use TensorFlow or PyTorch format to do the model transfer as the starting point

- About submission method and data/model ID

While handling data transfer/storage or model transfer/storage in RAN4, it is likely to involve naming or assigning IDs to datasets or models, such as using Tdoc numbers or other methods as mentioned earlier. We’d like to emphasize that there have been numerous discussions regarding model ID and data ID in both RAN1 and RAN2. We don't want RAN4's discussion on data transfer or model transfer issues to impact those discussions in RAN1 or RAN2. The submission methods and numbering problems(e.g., naming or assigning ID) involved in RAN4's data/model sharing should be decoupled from the data/model IDs discussed in RAN1/RAN2.

Proposal 6: The submission methods and numbering problems(e.g., naming or assigning ID) involved in RAN4's data/model sharing should be decoupled from the data/model IDs discussed in RAN1/RAN2.

- Conclusions

In this contribution, we have discussed some key issues related to data and model sharing, and got following proposals:

Proposal 1: Regarding training dataset size

- Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- Total space needed would depend on the number of contributing companies and meetings

- Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies

  - Data sharing might not be needed in every meeting

  - FFS whether the training data for different test settings or side conditions can be shared and reused

  - FFS the number of datasets needed for different test settings or side conditions

Proposal 2: The space prepared for the training set now is enough to cover the test and validation sets

Proposal 3: Use .npy (Numpy) format to do the data transfer as the starting point

Proposal 4: Regarding model size

- Model size: ~7MB – 100MB per model

- Total space needed would depend on the number of companies contributing and meetings

- Upper bound should be 600MB per company per year -> ~10GB per year

  - FFS whether a model for different test settings or side conditions could be reused

  - FFS the number of models needed for different test settings or side conditions

Proposal 5: Use TensorFlow or PyTorch format to do the model transfer as the starting point

Proposal 6: The submission methods and numbering problems(e.g., naming or assigning ID) involved in RAN4's data/model sharing should be decoupled from the data/model IDs discussed in RAN1/RAN2.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface