# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415030

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.4.1

Source: 	OPPO

Title: 	RRM core requirements for beam management

Document for:	Approval

- Discussion

In 3GPP RAN #102[1], for A/ML based beam management, it is agreed to provide specification support for the core requirements, especially for AI/ML LCM procedures e.g., for performance monitoring. Besides, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models are also agreed in the AI/ML WID, including: specify necessary performance requirements and tests (including metrics), and specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

### 4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. ### 4.2 Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution, we will continue to discuss the testability and interoperability issue on AI/ML based beam management with the focus on KPI and performance requirement, as well as the potential test issues on BM.

- Discussion

In RAN4 R19 tests, for both BM-Case1 and BM-Case2,

- Regarding the input related test, performance requirements on BM model/functionality input (e.g. beam measurement accuracy for set B) should be considered

- Regarding the output related test, three options have been agreed in RAN4 R18, i.e.,

Option 1: RSRP accuracy

Option 2: Beam prediction accuracy

Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among 		top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

For option1, different companies still have different understandings regarding the definition of RSRP accuracy . In RAN4 #110bis meeting, it is agreed to further provide inputs/proposals to refine the definition of RSRP accuracy.

According to R18 TR 38.843, “Average L1-RSRP difference of Top-1 predicted beam” is utilized as the beam prediction accuracy related KPI in RAN1, and defined as “the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam”.

Also in R18 TR 38.843, we could find another related definition that “for AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam”.

In the last meeting, RAN4 agreed “the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam”, and “the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam”.

According to above information, in RAN4 for the definition of RSRP accuracy, we may still have 3 different options for further clarification, i.e.,

|  | The RSRP difference between: |  |  |
| --- | --- | --- | --- |
| Option 1 | the ideal(or measured) L1-RSRP of Top-1 predicted beam | and | the ideal(or measured) L1-RSRP of the Top-1 ideal beam |
| Option 2 | the predicted L1-RSRP of Top-1 predicted beam |  | the ideal(or measured) L1-RSRP of the Top-1 ideal beam |
| Option 3 | the predicted L1-RSRP of Top-1 predicted beam |  | the ideal(or measured) L1-RSRP of the Top-1 predicted beam |

Proposal 1: To refine the definition of RSRP accuracy, following options should be further clarified,

- Option 1: the RSRP difference between the ideal(or measured) L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 2: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 3: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam

For option 3, it's important to note that when the Top-1 predicted beam isn't the ideal beam (e.g., the prediction results in beam A with a very low RSRP, while the ideal beam is actually beam B with a higher RSRP), even if the AI/ML prediction accurately predicts the RSRP for beam A and achieves an outstanding RSRP accuracy KPI, it's difficult to assert that the predicted result (beam A with a very low RSRP) could be effectively utilized.

Observation 1: For option3, when the Top-1 predicted beam isn't the ideal beam, even if the AI/ML prediction accurately predicts the RSRP and achieves an outstanding RSRP accuracy KPI(by the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam), it's difficult to assert that the predicted result could be effectively utilized.

Besides, in RAN1#116 meeting, 4 options regarding the content in the BM report of inference results have been agreed as well, as listed below,

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Based on these agreements, metrics for BM model output tests also depends on the RAN1 discussion, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy may be the only available KPI accordingly (note: also depends on the refined definition of RSRP accuracy)

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs

- Regarding the performance monitoring

- UE can monitor and estimate the performance of AI/ML based BM model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the discussion and conclusions in RAN1 as well.

- Besides, stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

Proposal 2: Regarding the model output tests, the selection of metrics depends on the progress in RAN1,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, beam prediction accuracy may be the only available KPI accordingly (also depends on the refined definition of RSRP accuracy in RAN4),

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, both beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs.

Proposal 3: UE can monitor the performance of AI/ML based CSI model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the progress in RAN1.

Proposal 4: Stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based BM and got following proposals:

Proposal 1: To refine the definition of RSRP accuracy, following options should be further clarified,

- Option 1: the RSRP difference between the ideal(or measured) L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 2: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 ideal beam

- Option 3: the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam

Observation 1: For option3, when the Top-1 predicted beam isn't the ideal beam, even if the AI/ML prediction accurately predicts the RSRP and achieves an outstanding RSRP accuracy KPI(by the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal(or measured) L1-RSRP of the Top-1 predicted beam), it's difficult to assert that the predicted result could be effectively utilized.

Proposal 2: Regarding the model output tests, the selection of metrics depends on the progress in RAN1,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, beam prediction accuracy may be the only available KPI accordingly (also depends on the refined definition of RSRP accuracy in RAN4),

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, both beam prediction accuracy and RSRP accuracy could be utilized as RAN4 test KPIs.

Proposal 3: UE can monitor the performance of AI/ML based CSI model/functionality through RSRP accuracy or Beam prediction accuracy, depends on the progress in RAN1.

Proposal 4: Stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface