# 3GPP TSG-RAN WG4 Meeting #112 bis	                                R4-2415042

# Hefei, China, 14 – 18 October 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on SBFD General aspects

Agenda Item:       6.20.1

Document for:     Discussion

# Introduction

This contribution provides our analysis for SBFD general issues including DL/UL subband configurations, and transmission bandwidth configuration for SBFD.

# Discussion

## DL/UL subband configurations support for SBFD

In previous RAN4 discussion, specifically, in RAN4#112, the following agreements are achieved [1]:

- Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks. - Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility. - BS transmission configuration in BS RF requirements (for information only): Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264.

The subband leakage power is dependent on the number of RBs present in the DL/UL sub bands. Lower sub band sizes will have less adjacent sub band power. As the sub band sizes are not defined, the leakage power and number of guard band RBs cannot be specified. The option-3 (with the highest flexibility) above, in which no restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guard band configurations), is more suitable.

Proposal 1: To consider the option-3 with the highest flexibility i.e., no restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guard band configurations).

## Transmission bandwidth configuration for SBFD

In previous RAN4 discussion, specifically, in RAN4#112, the following agreements are achieved [1]:

- Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed - The existing definition of BS transmission bandwidth configuration (for information only): - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS.

In this contribution, we present our view for minimum frequency allocation for SBFD and its impact on DL throughput. A minimum 20 MHz bandwidth is allocated to the Indian operators in TDD bands. Following is the throughput analysis for 10 MHz and 20 MHz.

Table1: Throughput analysis for 10 MHz and 20 MHz BW

| BW:10MHz FR1 µ:15kHz | BW:20MHz FR1 µ:15kHz |
| --- | --- |
| - If all RBs are allocated for downlink 5G NR, then the throughput is 128.773 Mbps. - On using a scaling factor of 0.5 approximately 26 RB allocated for uplink the throughput will be 64.387 Mbps. - Throughput drop in DL is 50% | - If all RBs are allocated for downlink 5G NR, then the throughput is 262.500 Mbps. - On using a scaling factor of 0.75 approximately 26 RB allocated for uplink the throughput will be 196.875 Mbps. - Throughput drop in DL is 25% |

Observation 1: If the channels are discontinuous, then SBFD in these channels will not be useful; instead, we can use the supplementary uplink channel to increase uplink throughput. As an example, if the operator owns continuous 4x10 MHz channels, then SBFD can be used; otherwise, it will add up complexity without much advantage.

Observation 2: From the frequency allocation to the Indian operator, a minimum 20 MHz BW is allocated in TDD bands.

Observation 3:  If the configured BW is less than 20 MHz then there is a significant drop in the DL throughput as compared to 20 MHz.

Proposal 2: A minimum 20 MHz BW in FR-1 is required for a reasonable SBFD operation; otherwise, the throughput will be reduced significantly.

# Conclusion

In this contribution, we discuss about SBFD general aspects with the following proposals and observations.

Proposal 1: To consider the option-3 with the highest flexibility i.e., no restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guard band configurations).

Observation 1: If the channels are discontinuous, then SBFD in these channels will not be useful; instead, we can use the supplementary uplink channel to increase uplink throughput. As an example, if the operator owns continuous 4x10 MHz channels, then SBFD can be used; otherwise, it will add up complexity without much advantage.

Observation 2: From the frequency allocation to the Indian operator, a minimum 20 MHz BW is allocated in TDD bands.

Observation 3:  If the configured BW is less than 20 MHz then there is a significant drop in the DL throughput as compared to 20 MHz.

Proposal 2: A minimum 20 MHz BW in FR-1 is required for a reasonable SBFD operation; otherwise, the throughput will be reduced significantly.

# Reference

- R4-2413514: Way Forward for [112][307] NR_duplex_evo_General, Samsung.