# 3GPP TSG-RAN WG4 Meeting #112 bis	                                 R4-2415043

# Hefei, China, 14 – 18 October, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on Potentially new requirements for SBFD operation

Agenda Item:       6.20.2.1

Document for:     Discussion

# Introduction

This contribution provides our view on potentially new requirements for SBFD operation. In the RAN4 #112 meeting RAN4 agreed to further study the location of transient period between SBFD and non-SBFD with following agreement [1].

- Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

# Discussion

## Location of transient period between SBFD and non-SBFD

Figure 1: SBFD configuration

Figure 1 shows a SBFD configuration. The possible switching cases may be [DLàSBFD], [SBFDàDL], [NON SBFD UL à SBFD UL] and [SBFD UL à NON SBFD UL].

Observation 1: The transient time should, in general, be the same for both full TDD switching and switching related to SBFD, as the entire TX switching time budget must be the same for both switching the complete UL/DL and the SBFD RBs. In the case of DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used since during this transition the power amplifier (PA) will be always on. Furthermore, since the UE is in half duplex, the transition from non-SBFD DL to SBFD UL will have to consider the PA switching time only. Therefore, no new requirement from the base station [BS] side.

Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD slots.

# Conclusion

In this contribution, we summarised the potentially new requirements for SBFD operation with the following proposals and observations.

Observation 1: The transient time should, in general, be the same for both full TDD switching and switching related to SBFD, as the entire TX switching time budget must be the same for both switching the complete UL/DL and the SBFD RBs. In the case of DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used since during this transition the power amplifier (PA) will be always on. Furthermore, since the UE is in half duplex, the transition from non-SBFD DL to SBFD UL will have to consider the PA switching time only. Therefore, no new requirement from the base station [BS] side.

Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD slots.

# Reference

- R4-2413514: Way Forward for [112][307] NR_duplex_evo_General.