3GPP TSG-RAN4 Meeting #112bis					          	       R4-2415047

Hefei, China, Oct 14th – 18th, 2024

Source: 	Tejas Networks

Title: 	Discussion on BS demodulation performance requirement using MMSE-IRC

Agenda Item:	6.16.3

Document for: 	Discussion

# Introduction

## 4	Objective

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

## 3	Justification

In Rel-19, to verify the UE and BS demodulation performance with advanced receiver and apply demodulation requirements to more scenarios of practical network, the following enhancements are identified:

- BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in neighbouring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system. There is necessity to introduce BS performance requirements for MMSE-IRC advanced receiver under inter-cell interference.

In RAN #103 meeting, the work items were approved for NR demodulation performance enhancements [1]. The justification and objectives related to use of MMSE-IRC for BS performance enhancements in presence of inter-cell interference are described as follows:

Based on WID [1] and WF document [2] of RAN4 #112 meeting, we present our views on simulation parameters, test scenarios, interference modelling requirements and proposals for BS demodulation performance enhancements using MMSE-IRC receiver.

# Discussion

## Signal Model and Receiver

MMSE IRC Receiver Assumption

According to the WF document [2], the estimation of the interference covariance matrix should be performed at the PRB level. However, DMRS can have different pattern and density in NR within PRB. Also, NR supports different SCS. Hence, we propose RAN4 to discuss the granularity of PRB for the interference covariance matrix estimation.

Proposal 1: RAN4 to discuss granularity of PRB for the interference co-variance matrix estimation considering various DMRS pattern and SCS in NR.

## Test scenarios & Interference Modelling

Test Scenario

- Proposals:

  - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104

  - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

According to WF document [2], the following options are proposed for the test scenario:

In TS 36.104, DIP values have been defined for homogenous and heterogenous scenarios of LTE. However, the analysis made by TR 36.884 [3] should not be directly applied to the NR scenario as there exists a huge difference in deployment scenario between NR and LTE, such as cell density (lower ISD), carrier frequency, channel type, delay, doppler, etc.

Proposal 2: We propose to derive new DIP profiles and corresponding methodology for the NR scenarios considering the same TDD pattern across BSs for both HomNet and HetNet scenarios.

According to the WF document [2], support of different TDD formats is proposed. However, different TDD formats across all BSs lead to gNB-gNB co-channel interference scenario, and to handle it, calculation of covariance matrix for gNB-gNB co-channel interference is required along with covariance matrix calculation for inter-cell interference [4].

Observation 1: For the HetNet scenario, if both Macro and Micro BSs utilize different TDD structures, then interference experienced by Micro BS from Macro BS will be high as compared to interference experienced by Macro BS from Micro BS. Hence, separate DIP values must be defined for both Macro and Micro BSs.

Observation 2: For considering different TDD patterns between BSs it is required to study gNB-gNB CLI handling schemes. WID document [4] studies the impact of gNB-gNB CLI handling and corresponding receiver requirements. Since this study is still in progress and the gNB-gNB CLI handling mechanism is yet to be finalized. It is proposed that performance evaluation of HetNet scenarios with different TDD patterns between BSs be de-prioritized.

Proposal 3: We propose de-prioritizing performance evaluation of HetNet scenarios with different TDD patterns between BSs.

Interference Modelling Criteria

- Candidate options on interference modelling criteria

  - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value

  - Option 2: DIP-based modelling

In the WF document [2], following two options were discussed for interference modelling criteria:

As in Rel-17, it is agreed to use INR+SNR based modelling for MMSE-IRC at UE end. Hence, to follow the same convention for BSs, we support option 1 of the WF document [2] to use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Proposal 4: We support option-1 of WF document to use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Interference Profile

- On interference power level:

  - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

  - It is not precluded for companies to bring system level simulation on other profiles.

  - FFS how to account for HPUE in existing LTE profiles.

For interference power analysis, the following options have been considered in the WF document [2]:

According to release 18 of TS 38.101 [5], maximum output power radiated by UE can reach up to 31 dBm, while interference power analysis for LTE has been done with 23 dBm output power UEs. Hence, analysis of the interference profile for NR should include HPUEs (UEs with 31 dBm, 29 dBm, and 26 dBm output power should be included). Also, interference signals from HPUEs experience higher delay spread. Hence, we propose new system-level simulations with the inclusion of HPUEs.

Proposal 5: We propose new system-level simulations with the inclusion of HPUEs.

For practical scenarios, inter-cell interference can have any possible modulation for NR. However, as proposed in the WF document [2], we support option 1, which suggests re-using the same interference modelling as captured in B.6.2 in TS36.104 to transmit a random 16QAM symbol for the neighbour PUSCH.

Proposal 6: We support option 1 of re-using the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH.

Network type

In the WF document [2], it is agreed that performance requirements should be defined only for synchronous deployment scenarios of NR. Typically, base stations are time-synchronized to avoid cross-link interferences. This ensures that the slot and frame structure of the interfering signal align precisely with the desired signal waveform. Hence, we support the use of synchronous scenarios for analysis according to sub-clause 6.2 of Annexure B from TS 36.104

Proposal 7:  We support re-using synchronous interference scenarios from sub-clause B.6.2 of TS 36.104.

Number of Interferers

The LTE study shows that 2Rx antennas at BS cancel single interference successfully. However, for a 2T2R scenario with two interference sources, the gain between MMSE-IRC and MMSE drops. Hence, we agree to use single interference for 2Rx cases and two interference sources for 4Rx and 8Rx cases.

Proposal 8: We agree to use one interference scenario for 2 Rx antenna cases and two interference scenario for 4 Rx and 8 Rx antenna cases.

## Test Parameters and Simulation Assumptions

Test Scope

- Candidate options:

  - Option 1: Cover the following SCS and TDD patterns

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

  - Other options not precluded

As agreed in WID [1], simulation scenarios should assume the FDD and TDD scenarios. In the WF document [2], the following options are considered:

The gain of the MMSE-IRC receiver over the MMSE receiver is independent of bandwidth and FDD/TDD scenario. As FDD bands are smaller than TDD, therefore performance analysis for smaller SCS can be done in FDD mode whereas larger SCS can be chosen for TDD mode analysis. Hence, we propose to use 15KHz SCS for PUSCH performance analysis in FDD mode. While most deployed TDD pattern 7D1S2U can be chosen for TDD mode performance analysis of PUSCH.

Proposal 9: We propose the following test scope for PUSCH performance analysis using MMSE-IRC

- 15 KHz SCS for FDD scenario

- 30 KHz SCS for TDD scenario with 7D1S2U pattern, S=6D:4G:4U

Channel bandwidth

In the WF document [2], the following options are proposed for different waveforms:

- Candidate options for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose

  - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz

  - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz

- Candidate options for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

For MMSE-IRC receivers, performance does not depend upon total bandwidth as covariance matrices are estimated over a granularity of a set of PRBs. Also, based on LTE analysis there is no significant performance change observed due to bandwidth.

Hence, for CP-OFDM, we support option 4 of using extreme bandwidths for each SCS, i.e., 5 MHz and 50 MHz for 15 KHz SCS and use of 10 MHz and 100 MHz for 30 KHz SCS.

Proposal 10: For CP-OFDM, we support option 4 for SCS and BW combination i.e.,

- 5 MHz and 50 MHz bandwidth for 15 KHz

- 10 MHz and 100 MHz for 30 KHz

For DFT-S-OFDM, we support option 1 of using minimum bandwidths for each SCS i.e., 5 MHz for 15 KHz and 10 MHz for 30 KHz.

Proposal 11: For DFT-S-OFDM, we support option 1 for SCS and BW combination i.e.,

- 5 MHz bandwidth for 15 KHz

- 10 MHz bandwidth for 30 KHz

Antenna Configuration

- Candidate options:

  - Option 1: Cover 2/4/8 Rx with 1Tx

  - Option 2: Consider only 4RX and 8RX gNB

  - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx

  - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers

  - Option 5: Prioritize 1Tx

For antenna configuration, the following options are defined in the WF document [2]:

The increase in the number of receiver antennas increases the interference rejection capability of the MMSE-IRC receiver, hence it is required to analyse the gain of MMSE-IRC over verities of receiver antenna possibilities instead of a single value. Therefore, we support the use of 2Rx, 4Rx, and 8Rx antennas for performance analysis. Also, the transmission of >1 stream from the UE side impacts MMSE-IRC performance when compared to a single-stream scenario. However, the number of UEs with 1Tx transmission is larger than 2Tx and 4Tx UEs. Hence, we propose to prioritize 1Tx scenarios for initial analysis, and RAN4 can discuss further the inclusion of 2Tx and 4Tx scenarios for performance analysis.

Proposal 12: We support option 1 to cover 2/4/8 Rx with 1 Tx antenna for initial performance analysis. RAN4 to further discuss the inclusion of 2Tx and 4Tx scenarios.

Propagation Condition

In LTE, EPA and EVA are utilized for HetNet and HomNet scenarios, respectively. In contrast to NR scenario, these channels support relatively low delay doppler. To meet the NR-specific channel condition, TS 38.104 specifies the usage of TDL-A 30 10, TDL-B 100 400, and TDL-C 300 100 channels for NR. As Doppler impacts covariance matrix estimation, the range of Doppler values must be exercised. So, we support the use of TDL-A 30 10, TDL-B 100 400, and TDL-C 300 100 channels for defining performance requirements.

For interference signals, the channel experiences a higher delay spread when compared to the desired signal. Hence, for initial analysis, the delay spread of the interference channel can be assumed to be 1000nSec with the TDL-C channel model for all scenarios.

| Antenna Configuration |  | Modulation Scheme for serving cell UE | Propagation Condition |  |
| --- | --- | --- | --- | --- |
| Serving Cell | Interference Cell |  | Serving Cell | Interference Cell |
| 1x2 | 1x2 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x4 | 1x4 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x8 | 1x8 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |

Proposal 13: We propose following combination of propagation condition for serving and interfering cells.

| Antenna Configuration |  | Modulation Scheme for serving cell UE | Propagation Condition |  |
| --- | --- | --- | --- | --- |
| Serving Cell | Interference Cell |  | Serving Cell | Interference Cell |
| 1x2 | 1x2 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x4 | 1x4 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x8 | 1x8 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |

MCS

- Option 1: Cover QPSK/16QAM/64QAM

  - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024)

  - Option 1B: MCS index 2, 12, 20 in MCS table 1

  - Option 1C: Cover MCS 2, 16, 20

Following were the candidate options for MCS in the WF document [2]:

We support a single MCS for each modulation scheme. Also, due to the presence of HPUEs as interference, it is advisable not to use a higher code rate for cell edge UEs. Hence, we support option 1B to cover modulation schemes and corresponding MCS for desired UE.

| Modulation Scheme | MCS Index |
| --- | --- |
| QPSK | 2 |
| 16 QAM | 12 |
| 64 QAM | 20 |

Proposal 14: We support option 1B to cover modulation schemes and corresponding MCS for desired UE.

| Modulation Scheme | MCS Index |
| --- | --- |
| QPSK | 2 |
| 16 QAM | 12 |
| 64 QAM | 20 |

Waveform

- Candidate options:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

  - Option 2: Cover CP-OFDM only

The candidate options for waveform in the WF document [2] are the following:

In NR, the use of DFT-S-OFDM waveform is a possible scenario. Also, there are ongoing discussion plans to include 1 Tx scenario for UE. Therefore, we support option 1 to define performance requirements for the DFT-s-OFDM and CP-OFDM waveform.

Proposal 15: We support option 1 to define PUSCH performance requirements for the DFT-s-OFDM and CP-OFDM waveform.

DMRS Configuration

As per the WF document [2], an option for the DMRS configuration is specified.

Covariance matrix estimation is crucial in inter-cell interference when the MMSE-IRC receiver is employed. Hence, accurate covariance matrix estimation highly depends upon the number of DMRS sub-carriers per PRB. The density of DMRS sub-carriers per PRB varies based on the DMRS configuration type. Hence, we should enhance the option-1 to include DMRS configuration type-2.

| DM-RS | DM-RS configuration type | 1, 2 |
| --- | --- | --- |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping, and sequence hopping are disabled |

Proposal 16: We propose to include DM-RS configuration type 2 along with other features mentioned in option 1 for performance analysis.

| DM-RS | DM-RS configuration type | 1, 2 |
| --- | --- | --- |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping, and sequence hopping are disabled |

PUSCH Mapping Type

The following were the candidate options for Mapping types in the WF document [2]

- Candidate options:

  - Option 1: Cover both PUSCH mapping Type A and Type B

  - Other options not precluded

:

The gain of the MMSE-IRC receiver also depends upon DMRS positions in a slot. As DMRS initial position and distance between two DMRS symbol varies based on PUSCH mapping type. Hence, we agree with option 1, which defines performance requirements for both PUSCH mapping types.

Proposal 17: We support option 1 to cover PUSCH mapping types A and B both for defining performance requirements.

Test metric

Following were the candidate options for the performance test metric in the WF document [2].

- Candidate options:

  - Option 1: Use 70% of maximum throughput as a metric for NR BS MMSE-IRC demodulation

  - Other options not precluded

We agree to align with TS 38.104 for determining test metric. Hence, we support option 1 to use 70% of maximum throughput as performance metric for NR BS MMSE-IRC calculation.

Proposal 18: We support option 1 to use 70% of maximum throughput as performance metric for NR BS MMSE-IRC calculation.

# Conclusion

In this contribution, we have given our views and proposals on MMSE-IRC performance requirement for BS demodulation.

Proposal 1: RAN4 to discuss granularity of PRB for the interference co-variance matrix estimation considering various DMRS pattern and SCS in NR.

Proposal 2: We propose to derive new DIP profiles and corresponding methodology for the NR scenarios considering the same TDD pattern across BSs for both HomNet and HetNet scenarios.

Proposal 3: We propose de-prioritizing performance evaluation of HetNet scenarios with different TDD patterns between BSs.

Proposal 4: We support option-1 of WF document to use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value.

Proposal 5: We propose new system-level simulations with the inclusion of HPUEs.

Proposal 6: We support option 1 of re-using the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH.

Proposal 7:  We support re-using synchronous interference scenarios from sub-clause B.6.2 of TS 36.104.

Proposal 8: We agree to use one interference scenario for 2 Rx antenna cases and two interference scenario for 4 Rx and 8 Rx antenna cases.

Proposal 9: We propose the following test scope for PUSCH performance analysis using MMSE-IRC

- 15 KHz SCS for FDD scenario

- 30 KHz SCS for TDD scenario with 7D1S2U pattern, S=6D:4G:4U

Proposal 10: For CP-OFDM, we support option 4 for SCS and BW combination i.e.,

- 5 MHz and 50 MHz bandwidth for 15 KHz

- 10 MHz and 100 MHz for 30 KHz

Proposal 11: For DFT-S-OFDM, we support option 1 for SCS and BW combination i.e.,

- 5 MHz bandwidth for 15 KHz

- 10 MHz bandwidth for 30 KHz

Proposal 12: We support option 1 to cover 2/4/8 Rx with 1 Tx antenna for initial performance analysis. RAN4 to further discuss the inclusion of 2Tx and 4Tx scenarios.

Proposal 13: We propose following combination of propagation condition for serving and interfering cells.

| Antenna Configuration |  | Modulation Scheme for serving cell UE | Propagation Condition |  |
| --- | --- | --- | --- | --- |
| Serving Cell | Interference Cell |  | Serving Cell | Interference Cell |
| 1x2 | 1x2 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x4 | 1x4 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |
| 1x8 | 1x8 | MCS 2 | TDL-B 100 400 | TDL-C 1000 100 |
|  |  | MCS 12 | TDL-C 300 100 | TDL-C 1000 100 |
|  |  | MCS 20 | TDL-A 30 10 | TDL-C 1000 100 |

Proposal 14: We support option 1B to cover modulation schemes and corresponding MCS for desired UE.

| Modulation Scheme | MCS Index |
| --- | --- |
| QPSK | 2 |
| 16 QAM | 12 |
| 64 QAM | 20 |

Proposal 15: We support option 1 to define PUSCH performance requirements for the DFT-s-OFDM and CP-OFDM waveform.

Proposal 16: We propose to include DM-RS configuration type 2 along with other features mentioned in option 1 for performance analysis.

| DM-RS | DM-RS configuration type | 1, 2 |
| --- | --- | --- |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID0=0, group hopping, and sequence hopping are disabled |

Proposal 17: We support option 1 to cover PUSCH mapping types A and B both for defining performance requirements.

Proposal 18: We support option 1 to use 70% of maximum throughput as performance metric for NR BS MMSE-IRC calculation.

# Reference

- RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

- R4-2413570, “WF on NR demodulation performance: Phase 5”, China Telcom, RAN#112, Maastricht, Netherlands.

- TR 36.884 “Performance requirements of MMSE-IRC receiver for LTE BS”

- R1-2406716, “Further Considerations and Recommendations for CLI mitigation for Sub-band Full Duplex”, Charter Communications, RAN#118, Maastricht, Netherlands.

- TS 38.101, “User Equipment (UE) radio transmission and reception”, v18.5.0