3GPP TSG-RAN WG4 Meeting #112bis			                                           R4-2415061

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4, R4-2413885]. Therefore, in this discussion, we provided more preliminary analysis on the potential RRM impacts because of the adaptation of common signal/channel transmission in Rel19 NES.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

- Discussion

Below the agreements made in RAN1 on adaptation mechanism of SSB and scenarios was summarized.

R1#116 Agreement Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration R1#116bis Agreement Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized R1#117 Agreement Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

Observation 1: For the SSB adaptation, the more than one SSB burst periodicity will be used. But how to indicate the one NW used for UE measurement are still FFS.

Beyond Rel18, SSBs are always-on transmitted and their periodicity can be configured by NW RRC message. And for the initial access on a cell may assume a default SSB periodicity of 20ms. In order to save NW energy, SSB adaptation periodicity or skipping SSB transmissions on certain occasions can save NW energy.

Figure 1.

Thus, the corresponding requirements impacts in RAN4 RRM are also expected generally. However, from RAN4 RRM perspective, in legacy RRM requirement, both intra and inter-frequency measurements requirement in TS38.133 section 9.2 and 9.3 can be summarized as:

Intra-frequency measurements: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Inter-frequency measurements: Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Observation 2: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message.

Thus UE can follow the indicated SSB adaptation was indicated by NW RRC message or other SIB message as well.

Observation 3: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Thus we can suggest that:
Proposal 1: RAN4 to wait RAN1’s progress on SSB adaptation to define the necessary requirements or update the current spec.

Especially as agreed in the last meeting, RAN1 agreed the adaptation SSB could not used for PCell sync raster. In our view, that is the corresponding SSB based measurement requirements in clause 9 of TS38.133 shall be clarified because they can applicable for both PCell and SCell without any explicit indication (e.g. Table 9.2.5.2-3: for measurement on deactivated SCell).

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Proposal 2: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell.

Additionally, up to now, in our view, one of potential impacts is if the SSB adaption happened within a measurement reporting duration (e.g. 4 measurement samples) as shown in the figure below, usually UE can

- Alt1: continue the measurements with the new SMTC/SSB period

- Alt2: discard the previous measurement samples with the obsolete SMTC period but restart a new measurement reporting procedure.

Figure 2.

Observation 4: the adaptation SSB configuration procedure happened within a successful measurement report period may result in the measurement requirements updates.

In order to simplify RAN4’s work, as the method we used before, we can assume the requirements applicability is no measurement period (SMTC) was not updated within a measurement reporting interval

Therefore, we can propose that:

Proposal 3: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: For the SSB adaptation, the more than one SSB burst periodicity will be used. But how to indicate the one NW used for UE measurement are still FFS.

Observation 2: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message.

Observation 3: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Proposal 1: RAN4 to wait RAN1’s progress on SSB adaptation to define the necessary requirements or update the current spec.

Proposal 2: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell.

Observation 4: The adaptation SSB configuration procedure happened within a successful measurement report period may result in the measurement requirements updates.

Proposal 3: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0

- R4-2413885