3GPP TSG-RAN WG4 Meeting #112bis		                                  R4-2415062

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.15.2.1

Title: 	Discussion on FR2-1 SSB based L3 measurement delay reduction by Rx beam

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NR Radio Resource Management (RRM) Phase 5 was agreed and the objectives are self-contained as below.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

And in the last RAN4 meeting, there were some initial discussion mainly on WI scope [2, R4-2414026]. In this contribution we provide more considerations on the possible enhancement to reduce the SSB based L3 measurement delay.

- Discussion

In the previous RAN4 meeting, the power consumption issue was also raised by many companies.

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping

At same time, another important aspect needed to be clarified is DRX scenario, which is one of most efficient way to reduce UE’s power consumption in current practical deployments. In case of measurement under DRX, the power consumption and the measurement delay itself shall be more carefully traded-off. Thus we suggest to discuss the DRX issue below [2] together.

Issue 1-2-9: FFS: DRX is configured or not [Way forward]: FFS: - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - Option 2 (Ericsson): FBS may cover DRX cases, no need to deprioritize DRX case.

More especially, whether FBS needs to be activated/deactivated can be depended on DRX configuration also. For an example, if DRX cycle is shorter than a specific value (e.g. 128ms) FBS can be activated. Therefore, we suggested that:

Proposal 1: The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration.

In the last meeting, RAN4 agreed that Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF.

Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

Hereby, we provided our salutation on the BSF for scenario 1/2/3/4 firstly.

For the individual part of L3 measurement, beam sweeping factor is assumed to be same. For an example, in current TS38.133 [3] as shown below 8 for FR2-1 power class 1 or 5 in the L3 cell synchronization and measurement.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

However, theoretically in case of UE acquired some rough timing with PSS/SSS synchronization, UE may complete the further measurement or SSB index decoding from the initial beam sweeping status. That is UE can do measurement within the less candidate beams.

Figure 3. Reduced Rx beam sweeping after PSS/SSS detection

Proposal 2: In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection.

- Conclusion

In this contribution, serval issues related toFR2-1 SSB based L3 measurement delay reduction for connected mode in Rel19 are discussed. The observations and proposals can be summarized as:

Proposal 1: The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration.

Proposal 2: In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection.

- References

- RP-240830

- R4-2406392

- TS38.133 v18.5.0

- TS38.331 v17.1.0