3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415073

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for AI mobility

Agenda Item:	6.18.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility.

- Use case - RSRP prediction accuracy requirement - Measurement error impact - Intra-frequency temporal domain prediction - Inter-frequency inter-cell prediction

- Discussion

In RAN#105 meeting, it’s agreed that RAN4 will start to study the impacts on requirements based on RAN2 assumptions[1].

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In the following, we will analyze the possible impacts on requirements based on RAN2 assumptions.

      - 2.1 Use cases

According to the latest TR, RAN2 defines 3 use cases:

  - RRM measurement prediction

  - Measurement event prediction

  - RLF/HO failure prediction

For RRM measurement prediction, it will have impact on both core part and performance part, e.g. measurement period, scheduling restriction, accuracy requirement.

For measurement event prediction, due to limited progress, it’s FFS what’s the impact on RRM.

For RLF/HO failure prediction, HO failure prediction has been periodized. For RLF failure prediction, SINR related prediction will have impact on both core part and performance part, e.g. measurement period, scheduling restriction, accuracy requirement.

According to current progress, RRM measurement prediction has been discussed widely and some agreements has been achieved. Therefore, RAN4 can study the impact of RRM measurement prediction first.

Proposal 1: RRM measurement prediction and RLF failure prediction will have RRM impact. RAN4 to study the impact of RRM measurement prediction first.

      - 2.2 RSRP prediction accuracy requirement

In TR[2], it’s agreed that:

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

Observation 1: In RAN2, RSRP prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell.

In 38.300, the figure for cell-level RSRP and beam level RSRP is defined:

Fig.1 L3 filtered RSRP

- A: measurements (beam specific samples) internal to the physical layer.

- Layer 1 filtering: internal layer 1 filtering of the inputs measured at reference point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) is not constrained by the standard.

- A1: measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

- B: a measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

- C: a measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at reference point B. This measurement is used as input for one or more evaluation of reporting criteria.

From the figure, in RAN2, RSRP prediction accuracy is defined at point C. In legacy RAN4, RSRP measurement accuracy is defined based on physical layer L1 filtered(Point A1) RSRP. Therefore, RAN4 needs to define new RSRP prediction accuracy after L3 filtering at point C.

The performance metric will be:

L3 filtered cell-level RSRP prediction accuracy = predicted L3 filtered cell-level RSRP – ground truth of L3 filtered cell-level RSRP

Proposal 2: RAN4 needs to define L3 filtered cell-level RSRP prediction accuracy requirement.

      - 2.3 Measurement error impact on RSRP prediction accuracy

Currently, in RAN2 simulation assumption, for both training and inference, ideal measured RSRP without error is used as input and label for AI model, similar as AI beam management. While in reality, the input of AI model will be measurement with error. Then there will be mismatch between training and inference. RAN4 may need to evaluate the impact of measurement error on RSRP prediction accuracy.

For AI based beam management, there have been some similar discussions in RAN1 about measurement error impact on accuracy degradation. In RAN1, the measurement error is modeled as a Gaussian distribution for RF and BB, with error ranges of 2dB, 4dB, and 6dB. Generally, prediction accuracy degrades as the measurement error range increases. However, the RSRP prediction degradation is calculated based on different assumptions, leading to a lack of aligned comparisons.

For example:

- Some companies compare prediction degradation based on input data differences while the label is the same, e.g., training with L1-RSRP measurements without error but testing with L1-RSRP measurements with error. Labels for both training and test are ideal L1-RSRP measurement without error.

- Other companies compare prediction degradation based on label differences while the input data are both measurement with error, e.g., L1-RSRP measurements with error is labeled during training while ideal L1-RSRP measurements without error is labeled during testing.

Besides, even if both inputs for training and inference are measurements with error, it’s still FFS they are aligned. For example,

  - Whether the SNR conditions for the two measurement assumptions are the same

  - Whether error model is the same, e.g. error is modeled as Gaussian distribution in training. It’s FFS what error distribution is in actual measurements for testing.

Therefore, if RAN4 would like to consider the degradation due to measurement error, it is better to first align on the basic assumption for training and testing first. Then performance evaluation can be further conducted to analyse the impact of measurement error if needed.

Proposal 3: If RAN4 would like to analyze the RSRP prediction accuracy degradation due to measurement error, suggest RAN4 to align the assumptions of input data and labels for training and test first.

      - 2.4 Intra-frequency temporal domain prediction

RAN2 prioritize 3 scenarios:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Two scenarios are related to intra-frequency intra-cell prediction. One scenario is related to inter-frequency inter-cell prediction. In the next, we will analyse the possible RRM impact on intra-frequency and inter-frequency respectively.

Case A and case B

In TR[2], RAN2 agrees two temporal domain cases, e.g. case A and case B.

Intra-frequency intra-cell temporal domain case A prediction is evaluated for the 2nd study goal by predicting measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell for both FR1 and FR2 scenario without reducing any measurement in observation window. Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub set of measurement instances in temporal domain of the same cell for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies without defining detail TDM pattern. Both case A and case B are applicable for all RRM sub-use cases and focus on at least pure temporal domain.

Furthermore, RAN2 is discussing the definition of case-A and case-B based on observation window and prediction window in draft TR[3] to reflect the benefit of AI, which will have impact on RAN4.

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

For legacy L3 RSRP measurement requirement, measurement will be conducted periodically. RAN4 defined measurement period which is required to satisfy the L1 filtered RSRP accuracy requirement. However, when AI is applied, some measurement instance can be skipped. Take case B for example, the target is to reduce measurement in temporal. Therefore, the measured samples will be reduced.

In prediction window, UE can skip measurement. It’s possible that scheduling restriction can be relaxed.

Observation 2: For Intra-frequency intra-cell temporal domain prediction, RAN2 will define two temporal domain cases in TR, e.g. case A and case B, based on observation window and prediction window.

Observation 3: Observation window and prediction window will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

Observation window and prediction window length

Observation window and prediction length is dependent on different sub-cases.

Currently, RAN2 defined 3 subcases for RSRP prediction.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

- Sub-case 1: RSRP is predicted at L1

For sub-case 1, the prediction happens at L1. In legacy, 5 L1 samples are needed to be measured in one measurement period, which is shown in Fig.2(a). While when AI is applied, for example, only 3 samples are needed to be measured during observation window. Then UE can predict other samples with grey colour, as shown in Fig.2(b). Then measurement period can be reduced. Here observation window is similar as legacy measurement period with reduced samples. Observation window and prediction window length are based on number of L1 samples.

Fig.2(a) measurement period for non-AI L1 filtered RSRP for FR1

Fig.2(a) observation window and prediction window for AI based L1 filtered RSRP

- Sub-case 2: RSRP is predicted at L3

For sub-case 2, filtered L3 RSRP will be predicted by L3 RSRP. Suppose that every 2 L3 filtered RSRP can predict one L3 filtered RSRP. As shown in Fig.2, UE will measure for 10 samples to get 2 L3 results and skip the following L1 5 samples. For this case, the observation window and prediction window length are based on number of L1 measurement period. here, we only give examples for non-sliding case.

Fig.3 observation window and prediction window for AI based L3 filtered RSRP

Sub-case 3: RSRP is predicted at L3

For sub-case 3, RSRP is also predicted at L3. Then observation window and prediction window are similar as sub-case 2.

Observation 4: Observation window and prediction window length are dependent on specific sub-cases.

Proposal 4: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2.

      - 2.5 Inter-frequency inter-cell prediction

In TR[2], RAN2 agrees that:

For both Intra-frequency prediction inter-cell prediction and FR1 to FR1 inter-frequency inter-cell prediction, no measurement is reduced in both temporal and spatial domain for cell to be measured. For FR1 to FR1 inter-frequency inter-cell prediction, focus on the case where cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighboring site. If inter-frequency correlation model is assumed, section 7.6.5 in [4] is taken as baseline for inter-frequency correlation model. FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub-use cases.

For inter-frequency inter-cell RSRP prediction, UE may skip the measurement for another cell on other frequency. Then it’s possible to reduce CSSF.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: RRM measurement prediction and RLF failure prediction will have RRM impact. RAN4 to study the impact of RRM measurement prediction first.

Observation 1: In RAN2, RSRP prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell.

Proposal 2: RAN4 needs to define L3 filtered cell-level RSRP prediction accuracy requirement.

Proposal 3: If RAN4 would like to analyze the RSRP prediction accuracy degradation due to measurement error, suggest RAN4 to align the assumptions of input data and labels for training and test first.

Observation 2: For Intra-frequency intra-cell temporal domain prediction, RAN2 will define two temporal domain cases in TR, e.g. case A and case B, based on observation window and prediction window.

Observation 3: Observation window and prediction window will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

Observation 4: Observation window and prediction window length are dependent on specific sub-cases.

Proposal 4: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Reference

[1] RP-242393 Revised SID: Study on Artificial Intelligence (AI)/ Machine Learning (ML) for mobility in NR, OPPO

[2] R2-2406309, text proposal on TR 38.744 v0.03

[3] R2-2407849, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions.