3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415074

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on RRM impact on measurements related enhancements for LTM

Agenda Item:	6.24.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting - CSI-RS based LTM

- Discussion

2.1 Event triggered L1 measurement reporting

Measurement result for evaluation

Issue 2-2-2: measurement result for event evaluation - Candidate solutions: - Option 1: Use beam level measurement result for event evaluation as baseline and FFS for cell level measurement. (CTC) - Recommended WF - Continue discussion.

The agreement on RAN2 in #127 is as below:

Agreements on L1 MR event evaluation 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering. Agreements on L1 measurement reporting 1. Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2 agrees to use beam level measurement results in LTM event evaluation. Therefore,

Proposal 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Starting point and ending point

For event triggered L1 measurement reporting, similar as L3, the main motivation is to verify that when beam level of cell quality changes, UE can detect the variation timely and report L1-RSRP to NW within a required time based on different event triggering criteria. Then the definition for starting point and ending point for L3 event triggered reporting can be re-used for L1. The total timeline for event triggered L1 reporting is as below:

Proposal 2: For event triggered L1 measurement reporting delay, the definition for starting point and ending point for L3 event triggered reporting can be re-used for L1 as baseline.

L1 reporting delay length

For legacy L3 reporting delay, the maximum delay length is defined as one measurement period for known cell to guarantee that UE can obtain at least one RRM result to compare with criteria and trigger reporting. Besides, no UL resource waiting time and L3 related processing is considered in reporting delay, e.g. including TTT(time to trigger) and L3 filtering.

For L1 reporting delay length, we observe some difference. Rel-19 MIMO session also agree to define L1 event triggered reporting requirement for beam management. Even through there are some differences between beam management and LTM, there are also some similarities. We also raise some following issues discussed here in MIMO session.

Needed measurement time

In order to evaluate whether L1 reporting will be triggered, UE needs to compare L1 results with certain criteria. There are several agreed events in RAN2:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For event LTM3 and LTM5, UE needs to measure for both serving cell and candidate cell to obtain L1 results for comparing. For Event LTM4, UE may need to measure for multiple neighbor cells to find one qualified cell.

For L1 measurement, the RS may be based on SSB or CSI-RS. It’s possible that the periodicity of RS is different from each other and then the measurement period is different. We plot a figure for event LTM3 where the RS periodicity for serving cell and neighbour cell is different. RAN4 needs to discuss how to define measurement time when the measurement periods for different cells are different.

Fig.1 Measurement time for different cells

Observation 1: It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS.

Besides, depending on the progress of other group, it’s FFS whether any other delay is needed for event evaluation, e.g. L1 filtering. RAN4 to wait for more inputs from other WGs for additional measurement delay.

Proposal 3: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period.

Impact of UL resource waiting time

In legacy L3 reporting, UL resource waiting time is not included in total L3 reporting delay length as the main intension is to verify the UE capability to report the variation of channel quality within required time duration. While for UL resource waiting time, the delay is dependent on NW configuration. Besides, since L3 measurement period is long, in most cases, UL resource waiting time will be much shorter than measurement period. UE will not acquire new L3 results before first UL resource is available. There is no impact if UL resource waiting time is not included.

Currently, it’s still under discussing in RAN2 how to configure UL resource. However, L1 measurement is quite faster compared with L3 measurement. Depends on different configuration for L1 measurement period and UL resource availability, it’s possible that L1 measurement period is shorter than UL resource waiting time. Then UE can acquire several new L1-RSRP result before first UL resource is available, as shown in Fig.2.

Fig.2 L1 Measurement period is shorter than UL resource waiting time

There may be some possible issues. For instance, if the UE has obtained several new L1 results after L1 reporting has been triggered,

  - Which L1 result will be included in the L1 report if the event trigger criteria still apply

  - What happens if the new L1 results change and no longer satisfy the event trigger criteria

Therefore, RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting delay. We don’t mean to specify the detail length of UL resource waiting time. But we need to consider the impact of UL waiting time on L1 measurement and L1 reporting.

Proposal 4: RAN4 to discuss whether to consider the impact of UL resource waiting time on L1 reporting requirement.

2.2 CSI-RS based LTM

Known cell

Issue 2-3-9: whether to cover both known and unknown cell for CSI-RS L1 measurement - Candidate solutions: - Option 1: only consider known cell in CSI-RS based L1-RSRP measurement. (Apple, MTK) - Recommended WF - Continue discussion.

Similar as SSB based L1 measurement, for CSI-RS based L1 measurement, L3 measurement will be performed first and only consider known cell case.

Proposal 5: RAN4 only consider known cell for CSI-RS based L1-RSRP measurement.

Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

Observation 2: Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report.

Definition of intra-frequency and inter-frequency

In last meeting, there is some discussion about how to define intra-frequency and inter-frequency measurement for CSI-RS L1-RSRP.

In legacy CSI-RS L3 measurement, the intra-frequency is defined as below:

A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

For definition of intra-frequency, the center frequency of the CSI-RS of neighbor cell will be compared with the center frequency of CSI-RS of the serving cell. However, for legacy CSI-RS measurement of L1-RSRP for serving cell, no center frequency is configured. It’s FFS whether center-frequency of both serving cell and neighbor cell will be configured for Rel-19 LTM. If not, we can define measurement within or without active BWP instead. We suggest to wait for more progress in other WGs.

Observation 3: For legacy CSI-RS measurement of L1-RSRP for serving cell, no center frequency is configured. It’s FFS whether center-frequency of both serving cell and neighbor cell will be configured for Rel-19 LTM.

Proposal 6: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs.

Supported measurement types

Issue 2-3-3: supported measurement types - Candidate solutions: - Option 1: (CATT) - Intra-frequency - Inter-frequency without gap - Inter-frequency with gap - Option 2: (Apple) - Intra-frequency without gap - Inter-frequency with gap - Option 3: deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK) - Option 4: (vivo) - For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases: - CSI-RS based L1 measurement without gap

The main difference about measurement requirement is whether the measurement is performed inside or outside active BWP. For intra-f without gap and inter-f without gap, if both are within active BWP, the requirement is similar. Currently, there is no agreement in RAN1 that whether measurement with gap will be supported for CSI-RS based L1 measurement. RAN4 can first discuss about measurement without gap case for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported.

Proposal 7: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported.

CSI-RS resource configuration

Issue 2-3-6: CSI-RS resource configuration - Candidate solutions: - Option 1: the same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation. (Apple) - Option 1a: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs. (MTK) - Recommended WF - Continue discussion.

The legacy CSI-RS configuration is {D=3, PRB>=48} to guarantee the measurement accuracy. If we consider measurement without gap first, we also suggest to limit the CSI-RS resource configuration for neighbour cell to be within active BWP first.

Proposal 8: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirement for {D=3, PRB≧48} and within active BWP first.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation.

Proposal 2: For event triggered L1 measurement reporting delay, the definition for starting point and ending point for L3 event triggered reporting can be re-used for L1 as baseline.

Observation 1: It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS.

Proposal 3: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period.

Proposal 4: RAN4 to discuss whether to consider the impact of UL resource waiting time on L1 reporting requirement.

Proposal 5: RAN4 only consider known cell for CSI-RS based L1-RSRP measurement.

Observation 2: Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report.

Observation 3: For legacy CSI-RS measurement of L1-RSRP for serving cell, no center frequency is configured. It’s FFS whether center-frequency of both serving cell and neighbor cell will be configured for Rel-19 LTM.

Proposal 6: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs.

Proposal 7: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported.

Proposal 8: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirement for {D=3, PRB≧48} and within active BWP first.

- Reference