3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415076

# Hefei, China, 14th – 18th Oct., 2024

Source: 	Xiaomi

Title: 	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Agenda Item:	6.19.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 MIMO phase 5.

- Aperiodic standalone CJT calibration reporting - Asymmetric DL sTRP/UL mTRP

- Discussion

2.1 Aperiodic standalone CJT calibration reporting

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM core requirements impact. - Option 2: RAN4 to define the measurement delay or measurement behavior for the aperiodic standalone CJT reporting.

Since CJT calibration reporting is aperiodic, there is no RRM core requirement impact.

Proposal 1: For aperiodic CJT calibration reporting, there is no RRM core requirement.

2.2 Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/UL mTRP deployment scenarios, there will be one anchor DL-TRP and multiple UL-only TRPs and they are not co-located. In legacy, each TRP has both downlink and uplink transmission. While for UL-only TRP, there is no downlink. From our understanding, in legacy RAN4 has defined following requirement for uplink transmission for mTRP:

UL TCI state activation, including PL-RS calculation

- UL timing requirement

UL TCI state activation

Source RS in UL TCI state activation

For UL-only TRP, DL-RS is not available. There are only two options for source RS in UL TCI state of UL-only TRP.

- SRS

- DL-RS from the anchor DL-TRP

Since anchor DL-TRP and UL-only TRP are not co-located, the UL beam directions to the two TRPs are different. It’s difficult to apply the DL-RS from anchor DL-TRP as source RS in UL TCI state of UL-only TRP. Therefore, SRS will be configured as source RS for UL-only TRP.

Observation 1: For UL-only TRP, SRS will be configured as source RS in UL TCI state activation.

In legacy RAN4, no requirement is defined when SRS will be configured as source RS for UL TCI state activation. However, for UL-only TRP, RAN4 may need to define requirement for SRS based UL TCI state activation, otherwise, no TX beam can be correctly transmitted to UL-only TRP. While for anchor DL-TRP, DL-RS can still be used as source RS in UL TCI state activation. It’s FFS whether SRS can be configured as source RS for DL-TRP as well.

Proposal 2: For UL-only TRP, RAN4 to define UL TCI state activation requirement based on SRS.

Proposal 3: For anchor DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. RAN4 to discuss whether to also define UL TCI state activation requirement based on SRS for anchor DL-TRP.

Known condition for UL TCI state

For UL TCI state activation requirement, known condition for UL TCI state is defined as pre-requisite. The legacy UL TCI activation procedure is tied to DL beam management in advance, specifically through L1-RSRP measurement and reporting. Different delay requirements are specified for known and unknown TCI states. In the case of a known TCI, the measurement report is based on DL-RS. However, for UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state can’t be re-used.

Observation 2: For UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state activation can’t be re-used.

When SRS is configured as source RS in UL TCI state activation, SRS resource ID will be provided. UE can apply the TX beam associated with the SRS resource ID as the target TX beam. The SRS resource ID needs to be configured for UE to transmit before the UL TCI state activation. In other words, NW needs to configure UE to transmit multiple SRS first and then configure one SRS as source for the following UL TCI state activation. Similar as known TCI state based on DL-RS, a known UL TCI state based on transmission of SRS can also be defined, e.g. the source SRS in UL TCI state activation has been transmitted for UL beam management within [x]ms before UL received UL TCI state activation command.

Proposal 4: RAN4 to discuss how to define known UL TCI state based on SRS.

UL TCI state activation delay for UL-only TRP

In legacy, two types of UL state activation delay requirement are defined:

- MAC CE based UL TCI state activation

- DCI based UL TCI state activation

For DCI based UL TCI state activation, the procedure and delay length are defined in RAN1. Activated TCI state in DCI can include SRS or DL-RS. Therefore, the delay requirement can be re-used for UL-only TRP.

For MAC CE based UL TCI state activation delay, we suggest to only define UL TCI activation for only known case. For known case, which will also include pathloss calculation based on measurement from another TRP and configured or updated pathloss offset associated with this TCI state. The legacy MAC CE based UL TCI state activation delay of known case can be used as baseline.

Proposal 5: Legacy DCI based UL TCI state activation requirement can be re-used for UL-only TRP.

Proposal 6: RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

Two TA

Another crucial aspect involves defining UL timing. In the legacy sDCI scenario, a single timing advance (TA) is typically considered. However, in the context of UL-only TRP and non-co-located DL anchor TRP, ensuring timing offset within CP is challenging. In RAN#105 meeting, the WID is updated and two TA will be considered for RAN1[1].

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

From RAN agreement, two TAs will re-use Rel-18 specification of multi-DCI based multi-TRP. In Rel-18, for each UL TCI state, an associated TAG ID will be provided. Similar, for UL TCI state for asymmetric DL sTRP/UL mTRP in Rel-19, an associated TAG ID will also be configured. Then DL reference timing defined in RAN4 Rel-18 can be re-used for Rel-19 asymmetric DL sTRP/UL mTRP.

Observation 3: For asymmetric DL sTRP/UL mTRP deployment scenarios, two TAs will re-use Rel-18 specification of multi-DCI based multi-TRP.

Proposal 7: If TAG ID is configured for UL TCI state, DL reference timing defined in RAN4 Rel-18 for multi-DCI mTRP can be re-used for Rel-19 asymmetric DL sTRP/UL mTRP.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: For aperiodic CJT calibration reporting, there is no RRM core requirement.

Observation 1: For UL-only TRP, SRS will be configured as source RS in UL TCI state activation.

Proposal 2: For UL-only TRP, RAN4 to define UL TCI state activation requirement based on SRS.

Proposal 3: For anchor DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. RAN4 to discuss whether to also define UL TCI state activation requirement based on SRS for anchor DL-TRP.

Observation 2: For UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state activation can’t be re-used.

Proposal 4: RAN4 to discuss how to define known UL TCI state based on SRS.

Proposal 5: Legacy DCI based UL TCI state activation requirement can be re-used for UL-only TRP.

Proposal 6: RAN4 to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

Observation 3: For asymmetric DL sTRP/UL mTRP deployment scenarios, two TAs will re-use Rel-18 specification of multi-DCI based multi-TRP.

Proposal 7: If TAG ID is configured for UL TCI state, DL reference timing defined in RAN4 Rel-18 for multi-DCI mTRP can be re-used for Rel-19 asymmetric DL sTRP/UL mTRP.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung