3GPP TSG-RAN WG4 Meeting #112bis                                                                                                      R4-2415077

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on BS demodulation performance for MMSE-IRC

Agenda item:	6.16.3

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF [1] on NR demodulation performance: Phase 5 was agreed and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

2.1 Interference modelling

Interference modelling criteria

As per the WF [1], the remaining open issue concerning interference modelling criteria is shown as below:

Interference modelling criteria - Candidate options on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling - Candidate options on DIP definition: - Option 1: Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 - Candidate options on SINR definition: - Option 1: For BS type 1-C and BS type 1-H: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to is defined by its associated DIP value. For BS type 1-O: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a RIB. is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to is defined by its associated DIP value..

For interference modelling criteria, option 1 and option 2 are equivalent. Option 1(INR-based modelling) was used for Rel-17 UE MMSE-IRC requirement, and Option 2(DIP-based modelling) was used for LTE PUSCH MMSE-IRC demodulation requirement, so for NR PUSCH MMSE-IRC demodulation requirement, Option 2(DIP-based modelling) is the most straightforward, if Option 1(INR-based modelling) is used for BS MMSE-IRC demodulation, it can make BS MMSE-IRC demodulation and UE MMSE-IRC demodulation to be consistent, so from the point of view of the consistency of the NR standard, the option 1 is more applicable.

Observation 1: Theoretically INR-based modelling and DIP-based modelling are equivalent.

Proposal 1: Adopt Option 1 for interference modelling criteria as below:

  - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value

For INR-based modelling, the DIP definition is not needed, and SINR definition need to be aligned with definition in TS 38.101-4. If DIP-based modelling is agreed for RAN4, the DIP definition (Option 1) and SINR definition (Option1) are applicable.

Interference profile

As per the WF [1], the remaining open issue concerning interference profile is shown as below:

Interference profile - Candidate options on interference modelling: - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - Other options not precluded - On interference power level: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - FFS how to account for HPUE in existing LTE profiles.

For inter-cell interference signal type, random 16QAM symbol for neighbour PUSCH captured in B.6.2 in TS36.104 are sufficient and can be reused for NR.

Proposal 2: Adopt Option 1 for inter-cell interference signal type as below:

  - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

For LTE, the DIP set1 (-1.11, -10.91) and set2 (-0.43, -13.78) were derived respectively in homogeneous and heterogeneous network scenarios with Methodology 1 for statistical measurements in TR 36.884.  It is noteworthy that in TR 36.884, the derivation of DIP set1 and set2 were based on passive antenna model with ITU UMa and UMi propagation model. Although BS types with active antenna (BS type 1-H, 1-O) are defined for NR, BS with passive antenna (BS type 1-C) is also defined for NR. Moreover, from perspective of interference, the interference from passive antenna is larger than that from active antenna, since the narrow beam generated by active antenna can achieve greater interference suppression. Therefore, it is reasonable that reuse the DIP set1 and set2 for LTE BS MMSE-IRC demodulation for NR BS MMSE-IRC demodulation. So it is sufficient to reuse the LTE interference power level for NR.

Proposal 3: Reuse LTE interference power level DIP set1 and set2 for NR.

Number of interferers

As per the WF [1], the remaining open issue concerning number of interferers is shown as below:

Number of interferers - Candidate options: - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna - Other options not precluded

Option 1 is same as the number of interferers for LTE PUSCH MMSE-IRC demodulation, so option 1 is reasonable for NR PUSCH MMSE-IRC demodulation.

Proposal 4: adopt Option 1 for number of interferers as below:

  - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna

2.2 Test parameters

SCS and TDD patterns

As per the WF [1], the remaining open issue concerning SCS and TDD patterns is shown as below:

Test scope - Candidate options: - Option 1: Cover the following SCS and TDD patterns - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U - Other options not precluded

The SCS and TDD pattern in opton1 are same as that for NR PUSCH normal demodulator, so option 1 is reasonable for PUSCH MMSE-IRC demodulation.

Proposal 5: Adopt Option 1 for SCS and TDD patterns as below:

  - Option 1: Cover the following SCS and TDD patterns

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Channel bandwidth

As per the WF [1], the remaining open issue concerning channel bandwidth is shown as below:

Channel bandwidth - Candidate options for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz - Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

The channel bandwidth for NR PUSCH normal demodulation can be reused for NR PUSCH MMSE-IRC demodulation. So the option1(For 15kHz SCS, cover 5MHz, 10MHz, 20MHz, For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz) is reasonable for CP-OFDM, and option1(For 15kHz SCS, 5MHz, For 30kHz SCS, 10MHz) is reasonable for DFT-s-OFDM.

To avoid work load, the option 3 is best choice.

Proposal 改一下.

Proposal 6: Adopt option 1 for CP-OFDM, and option 1 for DFT-s-OFDM as below:

- For CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

- For DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Antenna configuration

As per the WF [1], the remaining open issue concerning antenna configuration is shown as below:

Antenna configuration - Candidate options: - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4RX and 8RX gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx

Option 1 is same as antenna configuration for LTE PUSCH MMSE-IRC demodulation, so it is reasonable to use option 1 for NR PUSCH MMSE-IRC demodulation requirement.

Proposal 7: Adopt option 1 for antenna configuration as below:

  - Option 1: Cover 2/4/8 Rx with 1Tx

Propagation condition

As per the WF [1], the remaining open issue concerning propagation condition is shown as below:

Propagation condition - Candidate options: - Option 1: - For the serving PUSCH, use TDLA30-10 and TDLC300-100 propagation channel model for HetNet and HomNet respectively - Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000. - Option 2: Select among legacy TDLA30, TDLB100, TDLC300 - Option 2A: - Cover TDLA30-10, TDLB100-400 and TDLC300-100 - Same propagation condition for serving and interference PUSCH - Option 2B: - For the serving PUSCH, use TDLA30-10 for HetNet and TDLB100-100 for HomNet - For the interference PUSCH, use TDLC300-100 - Option 2C: - Cover TDLB100-400, TDLC300-100 and TDLA30-10 low for study purpose, select one for requirement definition - For the interference PUSCH, use TDLC300-100

We can refer to propagation condition for normal PUSCH demodulation, the TDLB100-400 Low, TDLC300-100 Low, TDLA30-10 Low, and TDLA30-10 Low are defined for QPSK, 16QAM, 64QAM and 256QAM respectively. So for QPSK, 16QAM, 64QAM, the TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low are needed. For interference PUSCH, same propagation condition for serving and interference PUSCH in Option 2A and TDLC300-100 in Option 2B and 2C can be acceptable. In LTE, the different propagation condition for serving and interference PUSCH are defined. So we propose,

Proposal 8:  Adopt new option 2D as below:

- Option 2D:

- Cover TDLA30-10, TDLB100-400 and TDLC300-100

- For the interference PUSCH, use TDLC300-100

MCS

As per the WF [1], the remaining open issue concerning MCS is shown as below:

MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20

For option 1A, the modulation and code rate are close to that of MCS 6, 15, 20 for LTE PUSCH MMSE-IRC demodulation, for option 1B and 1C, the MCSs are same with that for NR PUSCH normal demodulation. Option 1B and 1C are same. All options can be acceptable. From NR consistence, Option 1B is best choice,

Proposal 9: adopt Option 1B for MCS as below:

- Option 1B: MCS index 2, 12, 20 in MCS table 1

Waveform

As per the WF [1], the remaining open issue concerning waveform is shown as below:

Waveform - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Since the CP-OFDM and DFT-s-OFDM were defined for NR PUSCH normal demodulation, so it is reasonable to define NR PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM, so we prefer to both CP-OFDM and DFT-s-OFDM waveform.

We only need to consider same waveform type for serving and interference signal, e.g all CP-OFDM, or all DFT-s-OFDM.

Proposal 10:  Adopt same waveform type for serving and interference signal, e.g all CP-OFDM, or all DFT-s-OFDM.

Proposal 11: adopt Option 1 for waveform as below:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

DMRS configuration

As per the WF [1], the remaining open issue concerning DMRS configuration is shown as below:

DMRS configuration - Candidate options: - Option 1: DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); 0. DM-RS sequence generation; NID0=0, group hopping and sequence hopping are disabled. - Other options not precluded

According to sub-clause 6.4.1.1.1 of TS 38.211, the group hopping and sequence hopping are only supported by DFT-s-OFDM, so above option 1 for DMRS configuration is only applicable for DFT-s-OFDM and it is not applicable for CP-OFDM. According to test parameter for CP-OFDM in Table 8.2.1.1-1 and the test parameters for DFT-s-OFDM in Table 8.2.2.1-1, DM-RS configuration for PUSCH normal demodulaton for CP-OFDM and DFT-s-OFDM can be reused for PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM.  It is reasonable to adopt following DMRS configuration for PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM.

| Transform precoding |  | disabled(CP-OFDM) | enabled(DFT-s-OFDM) |
| --- | --- | --- | --- |
| DM-RS | DM-RS configuration type | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB |
|  | DM-RS port(s) | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=0, group hopping and sequence hopping are disabled |

Proposal 12: Adopt following DMRS configuration for PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM.

| Transform precoding |  | disabled(CP-OFDM) | enabled(DFT-s-OFDM) |
| --- | --- | --- | --- |
| DM-RS | DM-RS configuration type | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB |
|  | DM-RS port(s) | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=0, group hopping and sequence hopping are disabled |

PUSCH mapping type

As per the WF [1], the remaining open issue concerning PUSCH mapping type is shown as below:

PUSCH mapping type - Candidate options: - Option 1: Cover both PUSCH mapping Type A and Type B - Other options not precluded

For NR BS normal PUSCH demodulation, both PUSCH mapping Type A and Type B are defined, so it is reasonable to use both PUSCH mapping Type A and Type for PUSCH demodulation for MMSE-IRC.

Proposal 13: Adopt Option 1 for PUSCH mapping type as below:

  - Option 1: Cover both PUSCH mapping Type A and Type B

Test metric

As per the WF [1], the remaining open issue concerning test metric is shown as below:

Test metric - Candidate options: - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation - Other options not precluded

It is sufficient to use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation.

Proposal 14: Adopt Option 1 for test metric as below:

  - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Observation 1: Theoretically INR-based modelling and DIP-based modelling are equivalent.

Proposal 1: Adopt Option 1 for interference modelling criteria as below:

  - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value

Proposal 2: Adopt Option 1 for inter-cell interference signal type as below:

  - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

Proposal 3: Reuse LTE interference power level DIP set1 and set2 for NR.

Proposal 4: adopt Option 1 for number of interferers as below:

  - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna

Proposal 5: Adopt Option 1 for SCS and TDD patterns as below:

  - Option 1: Cover the following SCS and TDD patterns

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Proposal 6: Adopt option 1 for CP-OFDM, and option 1 for DFT-s-OFDM as below:

- For CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

- For DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 7: Adopt option 1 for antenna configuration as below:

  - Option 1: Cover 2/4/8 Rx with 1Tx

Proposal 8:  Adopt new option 2D as below:

- Option 2D:

- Cover TDLA30-10, TDLB100-400 and TDLC300-100

- For the interference PUSCH, use TDLC300-100

Proposal 9: adopt Option 1B for MCS as below:

- Option 1B: MCS index 2, 12, 20 in MCS table 1

Proposal 10:  Adopt same waveform type for serving and interference signal, e.g all CP-OFDM, or all DFT-s-OFDM.

Proposal 11: adopt Option 1 for waveform as below:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

Proposal 12: Adopt following DMRS configuration for PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM.

| Transform precoding |  | disabled(CP-OFDM) | enabled(DFT-s-OFDM) |
| --- | --- | --- | --- |
| DM-RS | DM-RS configuration type | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB |
|  | DM-RS port(s) | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=0, group hopping and sequence hopping are disabled |

Proposal 13: Adopt Option 1 for PUSCH mapping type as below:

  - Option 1: Cover both PUSCH mapping Type A and Type B

Proposal 14: Adopt Option 1 for test metric as below:

  - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation

- References

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telcom, RAN4#112

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements, Moderator (vivo), RAN#103

- R4-2411118, Discussion on BS demodulation performance for MMSE-IRC, CATT, RAN4#112