3GPP TSG-RAN WG4 Meeting #112                                                                                       R4-2415080

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on system parameters for Ku-band for NR NTN

Agenda item:	6.9.3

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF[1] on NTN Ku band system parameter was agreed and remaining issues on Ku-band system parameter in WF require further discussion.

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

Frequency range designation for Ku-band

The Max Doppler shift after pre/post compensation for S-band and Ka-band in Table 6.1.1.1-8 of TR 38.821 is copied as below:

| Max Doppler shift if pre/post compensation mechanism is assumed at satellite payload side | Scenario C2/D2: ● Satellite alt. = 600 km - beam diameter = 50 km (Set 1 - S-band): 1.05 ppm - beam diameter = 20 km (Set 1 - Ka-band): 0.42 ppm - beam diameter = 90 km (Set 2 - S-band): 1.88 ppm - beam diameter = 50 km (Set 2 - Ka-band): 1.05 ppm - beam diameter = 1000 km (max beam foot print size): 15.82 ppm |
| --- | --- |

Referring to above table, the max Doppler shift after pre/post compensation for S-band and Ka-band is 1.88ppm and 1.05ppm, respectively. so max Doppler shift after pre/post compensation for Ku-band can be calculated as (1.88+1.05)/2 =1.465ppm, which is corresponding to 20.51kHz for 14GHz, Therefore, from Doppler shift point of view, a SCS of at least twice 20.51 kHz is required to overcome the Doppler effect. So, 60 kHz and wider SCSs are more applicable for the Ku-band.

Observation 1: The 60kHz SCS and wider is more applicable for Ku-band.

In TS38.108, 60 kHz and120 kHz SCS are specified for FR2-NTN, and 15 kHz, 30 kHz, and 60kHz SCS are specified for FR1-NTN. So for FR2-NTN, 60 kHz and120 kHz SCS are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS is applicable for Ku-band.

Observation 2: For FR2-NTN, 60 kHz and120 kHz SCS are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS is applicable for Ku-band.

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Observation 1: The 60kHz SCS and wider is more applicable for Ku-band.

Observation 2: For FR2-NTN, 60 kHz and120 kHz SCS are applicable for Ku-band, and for FR1-NTN, only 60kHz SCS is applicable for Ku-band.

- References

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104