3GPP TSG-RAN WG4 Meeting #112bis                                                                                                      R4-2415083

Hefei, Anhui, China, 14th –18th October, 2024

Source: 	CATT

Title: 	Discussion on UE spurious emissions for UE in mmWave in NR

Agenda item:	5.21.2

Document for:	Approval

- Introduction

The New WID [1] on mmWave UE spurious emission will start in RAN4#112bis meeting, the impacts of EESS on UE and BS specification need to be discussed.

In this contribution, we provide our views on impacts of EESS protection for n257 and n258 on UE specification.

- Discussion

Network Signaling(s) and associated A-MPR (s)

As per the WID [1], the objectives concerning Network Signaling(s) and associated A-MPR(s) is shown as below:

- For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work

For EESS protection for n257 and n258, the UE maximum output power with additional requirements, UE maximum output power with additional requirements for CA and indication of modified MPR behavior are related to Network Signaling and associated A-MPR, so the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.2.3	  UE maximum output power with additional requirements

- 6.2A.3  UE maximum output power with additional requirements for CA

- H.1	Indication of modified MPR behavior

Observation 1: For Network Signaling(s) and associated A-MPR(s), the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.2.3	  UE maximum output power with additional requirements

- 6.2A.3  UE maximum output power with additional requirements for CA

- H.1	Indication of modified MPR behavior

Additional spurious emission requirement

- As per the WID [1], the objectives concerning additional spurious emission requirement is shown as below:For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258,

For additional spurious emission requirement, the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.5.3.1	Spurious emission band UE co-existence

- 6.5.3.2	Additional spurious emissions

- 6.5A.3.1 Spurious emission band UE co-existence for UL CA

- 6.5A.3.2	 Additional spurious emissions

Observation 2: For additional spurious emission requirement, the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.5.3.1	Spurious emission band UE co-existence

- 6.5.3.2	Additional spurious emissions

- 6.5A.3.1 Spurious emission band UE co-existence for UL CA

- 6.5A.3.2	 Additional spurious emissions

For spurious emission band UE co-existence in sub-clause 6.5.3.1 of TS 38.101-2, the spurious emission in Table 6.5.3.1-1 is copied as below:

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 3 |
| n258 | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n263 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 258 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 259 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

From above table, the spurious emission for n257 for frequency range 23.6-24GHz is +1dBm/200MHz for EESS protection. According to WID[1], to support the commercial deployment, the more stringent UE spurious requirement of −35 dBW/200 MHz or -5dBm/200MHz is needed according the studies on mmWave usage.

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

In order to specify above requirement for UE for EESS protection, it is sufficient to use the Note 4 and Note 5 in the following table to specify 1dBm/200MHz(Note 4) and -5dBm/200MHz(Note 5) for spurious emission for EESS protection.

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1(Note 4) -5(Note 5) | 200 | 3 |
| n258 | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n263 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 258 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 259 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. Note 4: This limit applies to UE brought into use on or before 1 September 2027 except in China and European Union. And this limit applies to UE brought into use on or before 1 January 2024 in European Union. Note 5: This limit applies to UE brought into use after 1 September 2027 except in China and European Union. This limit applies to UE brought into use on in China. And this limit applies to UE brought into use after 1 January 2024 in European Union. |  |  |  |  |  |  |  |

Observation 3: Use the Note 4 and Note 5 in the following table to specify 1dBm/200MHz(Note 4) and -5dBm/200MHz(Note 5) spurious emission for EESS protection.

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1(Note 4) -5(Note 5) | 200 | 3 |
| n258 | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n263 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 258 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 259 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. Note 4: This limit applies to UE brought into use on or before 1 September 2027 except in China and European Union. And this limit applies to UE brought into use on or before 1 January 2024 in European Union. Note 5: This limit applies to UE brought into use after 1 September 2027 except in China and European Union. This limit applies to UE brought into use on in China. And this limit applies to UE brought into use after 1 January 2024 in European Union. |  |  |  |  |  |  |  |

- Conclusion

This contribution provides analysis on impacts of EESS protection for n257 and n258 on UE specification, and the following observations are concluded as follows:

Observation 1: For Network Signaling(s) and associated A-MPR(s), the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.2.3	  UE maximum output power with additional requirements

- 6.2A.3  UE maximum output power with additional requirements for CA

- H.1	Indication of modified MPR behavior

Observation 2: For additional spurious emission requirement, the introduction of additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258 will impact the following parts in TS 38.101-2:

- 6.5.3.1	Spurious emission band UE co-existence

- 6.5.3.2	Additional spurious emissions

- 6.5A.3.1 Spurious emission band UE co-existence for UL CA

- 6.5A.3.2	 Additional spurious emissions

Observation 3: Use the Note 4 and Note 5 in the following table to specify 1dBm/200MHz(Note 4) and -5dBm/200MHz(Note 5) spurious emission for EESS protection.

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1(Note 4) -5(Note 5) | 200 | 3 |
| n258 | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n262 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band n263 | FDL_low | - | FDL_high | TBD | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n263 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 258 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 259 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 262 | FDL_low | - | FDL_high | -5 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. Note 4: This limit applies to UE brought into use on or before 1 September 2027 except in China and European Union. And this limit applies to UE brought into use on or before 1 January 2024 in European Union. Note 5: This limit applies to UE brought into use after 1 September 2027 except in China and European Union. This limit applies to UE brought into use on in China. And this limit applies to UE brought into use after 1 January 2024 in European Union. |  |  |  |  |  |  |  |

- References

- RP-242371, New WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE, RAN#105