3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415088

Hefei, CN, Oct. 14 – Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on SAN RF requirements for the NTN new S-band

Agenda item:	5.17.3

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on NR NTN support new S-band SAN RF requirement was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on SAN RF requirements for introducing the NTN new S-band.

- Discussion

2.1 SAN system parameter

The system parameters for NR NTN support new S-band have been discussed and concluded in the new S-band UE session. Therefore, the system parameters of the SAN should be consistent with the results discussed in the UE part.

For SAN operating band of new NTN S band, it should be aligned with the agreements of UE operating bands from previous meeting, as shown below:

Table 2.1-1: Satellite operating bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

For SAN channel bandwidth of new NTN S band, it should be aligned with the agreements of UE channel bandwidth from previous meeting, as shown below:

Table 2.2-1: SAN channel bandwidths and SCS per operating band in FR1

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

For SAN channel raster of new NTN S band, it should be aligned with the agreements of UE channel raster from previous meeting, as shown below:

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

Table 2.3-2: Applicable NR-ARFCN per operating band for enhanced channel raster

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

For SAN synchronization raster of new NTN S band, it should be aligned with the agreements of UE synchronization raster from previous meeting, as shown below:

Table 2.4-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Therefore, the SAN system parameter could be defined with the tables above, from Table 2.1-1 to Table 2.4-1.

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the SAN system parameters should be defined as Table 2.1-1 to Table 2.4-1.

2.5 SAN RF requirements

Since the SAN RF requirements in TS 38.108 are defined in a band agnostic manner, for all SAN type 1-H and type 1-O requirements should be applicable to new NTN band [n252], which means that there is no SAN RF requirements impacted by introducing new NTN S band.

Proposal 2: For TS 38.108, no RF requirements impact by introducing the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. The existing SAN type 1-H and 1-O RF requirements are applicable to new band.

- Conclusion

This contribution provides our initial views on SAN requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the SAN system parameters should be defined as Table 2.1-1 to Table 2.4-1.

Proposal 2: For TS 38.108, no RF requirements impact by introducing the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. The existing SAN type 1-H and 1-O RF requirements are applicable to new band.

- References

- R4-2414287, WF on Rel-19 FR1 NTN bands, Inmarsat, RAN4 #112