3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415090

Hefei, CN, Oct. 14 –Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on SAN RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	6.8.3.3

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz SAN RF requirement was agreed. However, there are still some remaining issues about the impact for OTA SAN requirements need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on SAN requirements for support spectrum less than 5 MHz.

- Discussion

Table 2.1 provides the Radiated SAN RF Tx and Rx requirements that might be affected by the introduction of 3 MHz channel bandwidth. Meanwhile, Table 2.1 also provides the NR TN BS requirements that are affected by the introduction of 3 MHz channel bandwidth as a reference.

Table 2.1: SAN RF requirements impact by 3 MHz

| NR NTN SAN RF Tx / Rx requirements | NR TN BS spec affected by 3 MHz | Proposal for NTN 3 MHz |
| --- | --- | --- |
| Radiated transmit power | Channel bandwidth agnostic | No specification impact |
| OTA SAN output power | Channel bandwidth agnostic | No specification impact |
| OTA output power dynamics | NR TN BS Type 1-O requirements is specified the same as NR TN BS Type 1-H requirements | No specification impact |
| OTA transmit ON/OFF power | Not applicable for NTN | Not applicable |
| OTA transmitted signal quality | Channel bandwidth agnostic | No specification impact |
| OTA unwanted emissions | NR TN BS Type 1-O requirements is specified the same as NR TN BS Type 1-H requirements | No specification impact |
| OTA transmitter intermodulation | Not applicable for NTN | Not applicable |
| OTA sensitivity | CBW for OSDD is reused from type 1-H BS | No specification impact |
| OTA reference sensitivity level | No 3 MHz CBW for TN | Same approach as introducing 3 MHz CBW for NTN conducted requirements |
| OTA dynamic range | No 3 MHz CBW for TN | Same approach as introducing 3 MHz CBW for NTN conducted requirements |
| OTA In-band selectivity and blocking | No 3 MHz CBW for TN | Same approach as introducing 3 MHz CBW for NTN conducted requirements |
| OTA Out-of-band blocking | EISminSENS related CBW is reused from type 1-H BS | No specification impact |
| OTA receiver spurious emissions | Not applicable for NTN | Not applicable |
| OTA receiver intermodulation | Not applicable for NTN | Not applicable |
| OTA In-channel selectivity | No 3 MHz CBW for TN | Same approach as introducing 3 MHz CBW for NTN conducted requirements |

Therefore, we believe that RAN 4 could use the above table as the starting point for introducing 3 MHz channel bandwidth.

Proposal 1: To use the proposals in Table 2.1 for NTN SAN RF requirements for NR NTN support 3 MHz channel bandwidth.

- Conclusion

This contribution provides our initial views on SAN requirements for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: To use the proposals in Table 2.1 for NTN SAN RF requirements for NR NTN support 3 MHz channel bandwidth.

- References

- R4-2413518, Way Forward for [112][313] NR_IoT_NTN_less_than_5MHz_BSRF, Nokia, RAN4 #112