3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415091

Hefei, CN, Oct. 14 –Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on system parameter for NTN support spectrum less than 5 MHz

Agenda item:	6.8.3.1

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz system parameter was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on system parameter for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on system parameter for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

## Open issues System parameters Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - FFS Issue 1-2-6: Signalling of SS raster - FFS Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN - FFS

According to the description of WID, the introduction of channel bandwidths less than 5 MHz in NTN is due to spectrum limitations/constraints encountered in existing NTN systems. Since the WID only considers FR1-NTN bands, unlike TN networks, the frequency bands are not multiples of 5 MHz. For TN networks, most of the operating bands were defined before the 3 MHz CBW feature was introduced, so the new 3MHz feature will only be defined as optional to ensure legacy UE could work properly. However, for the FR1-NTN UE, the impact of introducing 3MHz CBW is negligible. The mandatory support of the 3MHz CBW feature of NTN UE will maximize the use of the limited NTN spectrum, which is beneficial to the entire NTN system.

Proposal 1: 3MHz channel bandwidth for NR-NTN in the FR1-NTN bands could be mandatory if the introduction of NTN UE supporting 3MHz CBW does not lead to a fragmented market.

For asymmetric channel bandwidth under NR-NTN scenario, the asymmetric bandwidth combination set need to be defined. Typically, the band combination in the asymmetric bandwidth combination set is requested by the operator, and then the specific asymmetric bandwidth combination set will be indicated by the BandCombinationList defined by RAN2. However, the asymmetric bandwidth combination set of 3 MHz channel bandwidth defined in TS 38.306 [2] are shown as below:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

For support3MHz-ChannelBW-Asymmetric-r18 and support3MHz-ChannelBW-Symmetric-r18 capabilities, the asymmetricBandwidthCombinationSet for asymmetric CBW and associated SS/PBCH block for symmetric CBW are both defined in TS 38.101-1 but not TS 38.101-5. Therefore, the descriptions of TS 38.101-5 need to be supplemented.

In addition, neither support3MHz-ChannelBW-Asymmetric-r18 nor support3MHz-ChannelBW-Symmetric-r18 is mandatory. However the introduction of 3 MHz CBW for NR-NTN might be beneficial to the entire NTN system. Therefore, we believed that the existing capabilities of TN network could be reused, but the description related to NTN network could be considered as mandatory.

Proposal 2: The Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 could be reused for NTN system, but the component related to NTN system could be mandatory.

For NTN (e)Redcap UE, support for 3MHz channel bandwidth is not included in the WID, so it is not necessary to discuss the support of 3MHz CBW for the Redcap UE.

Proposal 3: Once the WID has been modified, RAN4 could discuss the support of 3MHz CBW for NTN (e)Redcap UE.

- Conclusion

This contribution provides our initial views on system parameters for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: 3MHz channel bandwidth for NR-NTN in the FR1-NTN bands could be mandatory if the introduction of NTN UE supporting 3MHz CBW does not lead to a fragmented market.

Proposal 2: The Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 could be reused for NTN system, but the component related to NTN system could be mandatory.

Proposal 3: Once the WID has been modified, RAN4 could discuss the support of 3MHz CBW for NTN (e)Redcap UE.

- References

- R4-2413528, WF on [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #112

- TS 38.306, User Equipment (UE) radio access capabilities, V18.2.0, 3GPP Technical Specification