3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415092

Hefei, CN, Oct. 14 –Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	6.8.3.2

Document for:	Approval

- Introduction

In RAN4 #112 meeting, a WF [1] on NR NTN support spectrum less than 5 MHz UE RF requirements was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE RF requirements for NTN support spectrum less than 5 MHz.

- Discussion

The open issues on UE RF requirements for NR NTN support spectrum less than 5 MHz was discussed during the previous meetings, as shown below:

Tx requirements Issue 2-1-1: UE maximum output power - FFS on how to define the additional requirements for transmit power density for 3MHz channel bandwidth in n254 Issue 2-1-2: MPR - FFS Issue 2-1-3: A-MPR - Further check - Neither 3MHz channel bandwidth or A-MPR are needed define in Table 6.2.3.1-1 for N_100 - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1, but no A-MPR need define for below NS values: - NS_01, NS_02N - Need introduce 3MHz channel bandwidth in Table 6.2.3.1-1 and define A-MPR for below NS values: - NS_03N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.2-1/Table 6.2.3.2-1 - NS_04N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.3-1/ Table 6.2.3.3-2 - NS_05N: A-MPR need define for 3MHz channel bandwidth in Table 6.2.3.4-1/ Table 6.2.3.4-2 - NS_24: A-MPR need define for 3MHz channel bandwidth for NTN n256 in a new clause 6.2.3.X in NTN spec 38.101-5. - Companies are encouraged to provide the related A-MPR values Issue 2-1-4: Minimum output power/ Transmit OFF power - Further check - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE - Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz - Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz Issue 2-1-5: Occupied bandwidth - Further check - Addition of 3 MHz into Table 6.5.1-1 in TS 38.101-5 Issue 2-1-6: Spectrum emissions mask - Further check - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE Issue 2-1-7: Additional spectrum emissions mask - FFS Issue 2-1-8: NR ACLR - Further check - Addition of 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5 Issue 2-1-9: General spurious emission - Further check - Whether reuse the TN UE approach for 3 MHz channel bandwidth correspondingly FOOB (6 MHz) Issue 2-1-10: Additional spurious emissions - FFS Issue 2-1-11: Spurious emissions for UE co-existence - FFS Issue 2-1-12: Other Tx requirements Rx requirements Issue 2-2-1: Reference sensitivity - FFS Issue 2-2-2: Maximum input level - FFS Issue 2-2-3: ACS - FFS Issue 2-2-4: In-band blocking - FFS Issue 2-2-5: Out-of-band blocking - Further check - Define out of band blocking requirement similar to TN for 3MHz (REFSENS + 6.0 dB) Issue 2-2-6: Narrow band blocking - Further check - Define narrow band blocking requirement similar to TN for 3MHz (-18dB Pw) Issue 2-2-7: Spurious response - Further check - 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel bandwidth (REFSENS + 6.0 dB) Issue 2-2-8: Intermodulation characteristics - Further check - There is no specification impact. Issue 2-2-9: Spurious emissions - Further check - There is no specification impact.

Table 2.1 provides all UE RF Tx and Rx requirements that might be affected by the introduction of 3 MHz channel bandwidth.

Table 2.1: UE RF requirements impact by 3 MHz

| NR NTN UE RF Tx / Rx requirements | Proposal for NTN 3 MHz |
| --- | --- |
| UE maximum output power | No specification impact |
| UE maximum output power reduction | No specification impact The MPR requirements for NTN UE are refer to TS 38.101-1 |
| UE additional maximum output power reduction | No impact on NS_100 No further relaxation on A-MPR values compared to existing values. |
| Configured transmitted power | No specification impact |
| Output power dynamic | Introduce 3 MHz channel bandwidth No output power dynamic requirements impact, but the measurement bandwidth need to be updated |
| Occupied bandwidth | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Spectrum emissions mask | Introduce 3 MHz channel bandwidth for General spectrum emissions mask Reuse NR TN FR1 UE spectrum emissions limit for 3 MHz CBW |
| Additional spectrum emissions mask | No specification impact |
| Adjacent Channel leakage ratio (ACLR) | Reuse the TN UE approach for 3 MHz channel bandwidth |
| General spurious emissions | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Additional spurious emissions | Introduce 3 MHz channel bandwidth for affected network signalling values |
| Reference sensitivity power level | Introduce 3 MHz channel bandwidth 2.2 dB scaling could be reused from TN approach |
| Maximum input level | Reuse the TN UE requirements for 3 MHz channel bandwidth |
| Adjacent channel selectivity (ACS) | Introduce 3 MHz channel bandwidth Reuse the TN UE requirements for 3 MHz channel bandwidth |
| In-band blocking | Introduce 3 MHz channel bandwidth Reuse the TN UE requirements for 3 MHz channel bandwidth |
| Out-of-band blocking | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Narrow band blocking | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Spurious response | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Wide band intermodulation | No specification impact The IMD requirements for NTN UE are refer to TS 38.101-1 |

Therefore, we believe that RAN 4 could use the above table as the starting point for introducing 3 MHz channel bandwidth.

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- Conclusion

This contribution provides our initial views on UE RF requirements for NTN support spectrum less than 5 MHz. The following proposals are concluded as follows:

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- References

- R4-2413528, WF on [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4 #112