3GPP TSG-RAN WG4 Meeting #112-bis                                                                                     R4-2415095

Hefei, CN, Oct. 14 – Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on NTN SAN RF requirements

Agenda item:	6.26.3

Document for:	Approval

- Introduction

In RAN4 #112 meeting, A WF [1] on NR NTN phase 3 SAN RF requirements was agreed. However, there are still some remaining issues about SAN transient time need to be discussed in the next meeting.

In this contribution, we would like to share our views on RF requirements for NTN SAN.

- Discussion

2.1 Beam switching delay

The agreement on RF requirements enhancements for spatial domain techniques was achieved during the previous meetings, as shown below:

Issue 1-1-4: RF requirements enhancements for spatial domain techniques Agreement: Common current understanding is that (at least for now) there is no RF requirements impact by the beam switching delay of phase array antenna. However, this can be further discussed if other issue is identified, such as different condition for beamforming or SAN beam allocation and configuration.

Referring to the discussion in RAN4 #111 meeting, the discussion focused on where the “delay” comes from. However, for NTN SAN (FR1 and FR2) all requirements are defined on the TAB connector or RIB, as shown below:

Figure 2.1-1: Radiated and conducted reference points for SAN type 1-H

For the requirements defined in TAB connector, it is not able to characterise the antenna related performance. If the phase array antenna is considered to have some delay in beam switching action, the delay comes from the phase array antenna, not the left part of the reference point (RF component). As a result, the delay associated with phased array antennas can only be defined and tested in OTA manner, which causes some additional difficulties for NTN SAN testing.

Observation 1: The delay caused by phase array antenna may need to be tested in OTA manner.

Furthermore, the behaviour of NTN SAN with beam hopping could be considered as an implementation related topic. For SAN that supporting beam hopping feature, the transmission RF chain is usually stay turned on, so the served UE can get higher CBW and power with a focused beam. For phase array antennas, switching the beam usually takes only a few nano-seconds so no additional impact on the RF requirements. As for the delay caused by configuring the beam hopping, this will depend on the implementation such as different algorithms used by the SAN. In extreme cases, the SAN can use a completely pre-defined beam hopping pattern where there is no delay at all.

Therefore, we believed that for NTN SAN, the beam switching delay caused by phase array antenna has no effect on the RF requirements for NTN SAN.

Proposal 1: There is no RF requirements impact by the beam switching delay of phase array antenna.

2.2 Feeder link and gateway for regenerative payload

The agreement on SAN scope, definitions and figures were achieved during the RAN4 #111 meetings, as shown below:

Issue 1-2-5: Alignment of TS 38.101-5 & TS 38.181 with TS 38.108 - Agreement: TS 38.101-5 and TS 38.181 (scope, definitions, SAN figures) to be aligned with modifications related to introduction of regenerative payload in TS 38.108. - NOTE: Some of the modifications proposed in this Rel-19 discussions are actually applicable to Rel-17 specifications (TS 38.108, TS 38.181, TS 38.101-5). Therefore, related CRs were submitted under maintenance Agendas 4.1 and 4.2 in [6-11].

Therefore, the SAN diagram of TS 38.108 was modified after RAN4 #111 meeting. These changes improve the definition of SAN, but introduce new issues. For SAN type 1-H, Figure 2.1-1 in previous discussion shows the connection between satellite payload and satellite gateway. Referring to the definition of feeder link and satellite-gateway in TS 38.108:

feeder link: Wireless link between satellite-gateway and satellite. satellite-gateway: An earth station or gateway is located at the surface of Earth, and providing sufficient RF power and RF sensitivity for accessing to the satellite.

It is notable that the feeder link and satellite-gateway are defined for satellite, which means that they are not affected by the payload type of satellites (transparent or regenerative). However, for Figure 2.1-1, the feeder link connection between satellite-gateway and satellite may require further clarification. The connection between the satellite-gateway and the satellite with a dotted line may bring some ambiguity, and it is difficult to indicate whether the feeder link between the satellite-gateway and the satellite is necessary. In addition, the similar problem also exist for Type 1-O SAN and Type 2-O SAN.

Proposal 2: RAN4 should discuss whether the feeder link and satellite-gateway are mandatory or optional for regenerative payload.

- Conclusion

This contribution provides our initial views on RF requirements for NTN SAN. The following proposals are concluded as follows:

Proposal 1: There is no RF requirements impact by the beam switching delay of phase array antenna.

Proposal 2: RAN4 should discuss whether the feeder link and satellite-gateway are mandatory or optional for regenerative payload.

- References

- R4-2413517, Way Forward for [112][310] NR_NTN_Ph3, Thales, RAN4# 112