3GPP TSG-RAN WG4 Meeting #112-bis                                                                                       R4-2415096

Hefei, CN, Oct. 14 – Oct. 18, 2024

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	6.26.2.1

Document for:	Approval

- Introduction

In RAN4 #112 meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: As baseline - n256: 0.5 dB tightening ## Issue 1-4: Simultaneous operation with GNSS Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g. - Operating bands with UL close to GNSS frequencies (n254, n255) - Emissions on GNSS frequencies - Whether transmission gaps enabling successful GNSS reception are present - Potential impact on field performance - Potential need for specification updates, e.g. frequency error ## Issue 1-5: Specification updates Agreement: - Include information in specification suffix clause on suffix applicability on FR1-NTN, FR2-NTN or both, with RedCap being applicable to FR1-NTN - Use suffix I for RedCap, if accepted by MCC - Add a clause on max output power for RedCap indicating RedCap supports only PC3, similar to TN spec 38.101-1 clause 6.2I.1 ## Issue 2-1: UL capacity enhancements Agreement: - Continue evaluation on potential UE RF specification impact - As starting point, no UE RF impact at least from inter-symbol OCC scheme

According to the current NTN UE spec TS 38.101-5, the maximum output power requirements are shown as below:

### 6.2.1 UE maximum output power The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). Table 6.2.1-1: UE Power Class NR satellite band; Class 3 (dBm); Tolerance (dB). n256; 23; ±2. n255; 23; ±2. n254; 23; ±2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated.

For NTN UE operating at power class 3, the tolerance is ± 2 dB which means that the actual antenna port Tx power can be up to 25 dBm. The tolerance takes into account manufacturing tolerances, temperature and other factors that may have an effect on the transmitter power, meaning that if the power class is relaxed at the design stage (such as 25 dBm), the final transmit power may be much higher than that of PC3(probably 27 dBm).

In fact, the maximum transmission power of PC3 is already close to PC2 (after considering tolerace), which means that further relaxation may cause the NTN (e)RedCap UE to encounter the problems faced by PC2. For instance, additional SAR limit would require the introduction of additional signalling, and higher Tx power may cause some coexistence issues. Therefore, the relaxation of transmit power may require further evaluation. Considering that RAN4 is conducting NTN HPUE related evaluation, it may be possible to leverage the results of the NTN HPUE related evaluation to reduce the work load of RAN4.

Observation 1: The power level for PC3 HD-FDD (e) NTN RedCap could refer to the conclusions of the NTN HPUE evaluation.

Proposal 1: The power level for PC3 could be same for non-RedCap NTN UE and (e)RedCap NTN UE.

According to the current NTN UE spec TS 38.101-5, the frequency error requirements are shown as below:

### 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

NTN RedCap UE has not yet defined the frequency error requirements. However, for a NTN system, the frequency pre-compensation is required for NTN UE completing the demodulation. Therefore, the Frequency error is necessary for NTN RedCap UE. In addition, the current minimum requirement for frequency error are ± 0.1 PPM for NTN UE and TN UE, which is relatively relaxed, so we believe that reuse this requirements may not cause an additional problem.

Proposal 2: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- Conclusion

This contribution provides our initial views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Observation 1: The power level for PC3 HD-FDD (e) NTN RedCap could refer to the conclusions of the NTN HPUE evaluation.

Proposal 1: The power level for PC3 could be same for non-RedCap NTN UE and (e)RedCap NTN UE.

Proposal 2: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- References

- R4-2413527, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 112.