3GPP TSG-RAN WG4 Meeting #112bis	                                                                        R4-2415098

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	CATT

Title: 	Discuss on AIoT co-existence simulation parameters

Agenda item:	6.21.2.2

Document for:	Discussion

- Introduction

This contribution further discusses the remaining issues for AIoT co-existence simulation parameters including NR in-band emission, NR in-band rejection, CW emissions, etc.

- Discussion

  - NR BS in-band emission assumption

For the NR BS in-band emission compared with 1 RB AIoT, there are two options for further discussion,

Option 1: 30dBc

Option 2: 17dBc

We did some simulation to check the NR BS in-band emission. Figure 1 shows spectrum for BB data and BB data + windowing. It can be seen that windowing improves the emission performance for both in-band and out of band.

Figure 1: BS in-band emission

In this simulation, the 17th RB is reserved for AIoT as the simulation scenario in the co-existence simulation. Zooming in the AIoT RB and the adjacent RBs, the spectrum is shown in Figure 2.

Figure 2: Spectrum detail of the blank RB and the adjacent RBs

The emission level on the blank RB is improved a lot, ~ 18 dB more, however due to the transient domain still exists for both sides of the frequencies, the emission level in this RB is still high. In figure 2, it can be seen that the transient domain is about 80 kHz for both sides, ACLR for the windowing signal is ~12 dB which is 1 dB improved for the signal without windowing. Modeling the emission may have two methods, one is to use the model similar to UE IBE. The other is to use ACLR approach. UE IBE similar model is more accurate if the impact on each SCS is evaluated. However, the total noise is estimated for the whole BW in the co-existence simulation, it’s not necessary to model the emission in SCS level.

Observation 1: NR in-band emission has the about 80 kHz transient domain for the blank RB.

Proposal 1: Use ACLR approach to model NR BS in-band emission for co-existence simulation.

Proposal 2: NR BS ACLR for 1 blank RB is assumed as 12 dB when NR BS is transmitted full RB except this blank RB.

  - NR Rx in-band rejection assumption

For NR Rx in-band rejection, the understanding in last RAN4 meeting is aligned that the AIoT signal falls into NR BS/UE in-band directly without any rejection. The issue needs to be check is if there’s any noise brought by the non-orthogonal impact between AIoT signal and NR signal.

2.2.1 AIoT DL impacts NR UE Rx

For the AIoT frame structure and timing design, RAN1 has the following agreement in RAN1#116b meeting,

- For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

The following is the agreements for the R2D waveform in RAN1 #117,

For R2D evaluation purposes, the R2D waveform for DFT-s-OFDM is generated as follows:

- The time domain OOK signal is the M chips of one OFDM symbol.

- A chip is represented (e.g. upsampled) by L samples

  - Companies to report L

- An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal.

  - Companies to report N’, e.g. N’=128 or equal to X

- Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D.

  - Companies report how to map and report X

- An N-points IDFT is performed to obtain the time domain signal.

  - Companies to report N, and how value was selected

- Note: companies report whether/how CP samples are added.

CP

CP

CP

CP

FFT window

For OFDM based waveform, AIoT is designed orthogonal with NR system in RAN1. The possible non-orthogonal may exist when AIoT signal is received by NR BS/UE with different time delay between AIoT signal and NR signal. The most severe scenario is NR signal is transmitted from a BS/UE faraway but AIoT signal is nearby because AIoT cell coverage is very small. Considering 500m NR BS coverage, the transmission time in the air is 1.67 us which is shorter than the CP length of 15 kHz and 30 kHz SCS.

Figure 3: multi-path signals for OFDM symbol

Although CP can mitigate the multipath delay as shown in Figure 3, it should be noted that the multipath signals are from the same signal source, so usually the strongest path in the multipath signals are the nearer than the path with longer delay which usually means lower level at the same time. Then the FFT window for the strongest path can cover all of the signal paths, thanks to CP.

CP

NR UE received AIoT DL signal

NR UE received NR DL signal

1.67 us

CP

CP

CP

CP

CP

AIoT BS Tx timing

NR BS Tx timing

FFT window H

FFT window L

However, when the signals are from different systems, CP may not be able to cover the different paths.

Figure 4: NR UE received nearby AIoT DL signal and NR DL signal

It can be seen from Figure 4 when the time delay is 1.67 us which is half CP for 30 kHz SCS, then the UE Rx FFT window has the possibility to overlap with the next AIoT OFDM symbol.

Observation 2: The transmission time for 500 m NR macro coverage is 1.67 us which is half CP for 30 kHz SCS.

The non-orthogonal impact for NR UE Rx is highly related to the NR UE Rx FFT window position. Taking Figure 4 as an example, if NR UE Rx window is FFT window L, then there’s no non-orthogonal impact to NR UE Rx because FFT window L is in the same AIoT signal. But if the FFT window is moved to the right direction, non-orthogonal impact happens because FFT window includes both parts of the two adjacent AIoT OFDM symbols. The non-orthogonal interference level is related to how many sampling points of the next symbol are included in the FFT operation. Figure 5 shows the scaled SNR compared with AIoT power Vs FFT window.

Figure 5: Scaled SNR Vs points in next AIoT symbol

It can be seen that if more points in next AIoT symbol are included in the NR UE FFT window, the more noise is introduced to NR UE Rx. If FFT window is offset with half CP, scaled SNR is ~ 37dB.

Observation 3: If more points in next AIoT symbol are included in the NR UE FFT window, more noise is introduced to NR UE Rx. If FFT window is offset with half CP, scaled SNR is ~ 37dB.

The SNR is highly related to UE implementation, i.e. Rx FFT window choice, so it’s proposed UE vendors to double check.

Proposal 3: It is proposed that UE vendors to check NR UE Rx FFT window choice implementation to double check AIoT DL signal’s non-orthogonal impact when AIoT reader is deployed nearby but NR BS is very far.

Another critical issue which should be considered is the Rx dynamic range impact. When AIoT BS is near NR UE, the Rx input level for UE can be up to the maximum input level, i.e. -25 dBm if the same MCL is assumed. But NR DL signal can be very low to -100 dBm Rx level. 75 dB dynamic range may be difficult to be supported simultaneously by NR UE Rx. Considering 10 dB PAPR and 75 dB dynamic range, at least 15 bit ADC is needed. It should be confirmed by UE vendor.

Proposal 4: It is proposed that UE vendors check NR UE receives large AIoT DL signal and small NR DL signal simultaneously, i.e. -25 dBm AIoT DL signal and -100 dBm NR DL signal may be received simultaneously by NR UE.

2.2.2 AIoT DL impacts NR BS Rx

When AIoT DL uses UL spectrum, AIoT DL may interference NR UL. For NR UE, there’s TA adjustment to make sure NR UL signal arrives NR BS at the correct time. But for AIoT DL, if AIoT DL symbol boudary is aligned with NR BS, the time delay to NR BS causes symbol boundary misalignment between NR UL signal and AIoT DL signal arrived at NR BS Rx antenna. The maximum misalignment is shown in Figure 6. The minimum time difference is calculated from the minimum distance 100 m between NR BS and AIoT reader.

Figure 6: NR BS received NR UL and AIoT DL signal timing

According to TS 38.104, the EVM window length W is 40% ~ 60% CP length, so when NR BS received NR UL and AIoT DL signal timing difference is smaller than 40% of CP length, there’s no non-orthogonal impact for AIoT DL signal to NR BS Rx.

Observation 4: When NR BS received NR UL and AIoT DL signal timing difference is smaller than 40% of CP length, there’s no non-orthogonal impact for AIoT DL signal to NR BS Rx.

The maximum 1.67 us timing difference is half CP of 30 kHz SCS, but the BS Rx can still choose the proper FFT window to avoid this interference. So we propose,

Proposal 5: The non-orthogonal impact for AIoT DL signal to NR BS Rx is not so critical so it can be ignored in the co-existence simulation.

2.2.3 AIoT UL impacts NR UE Rx

For backscattering based devices, UL signal is CW based M-chip OOK signal. Assuming CW frequency is NR Rx center frequency, the received signal in NR UE is OOK signal without CW wave modulated as shown in Figure 7.

Figure 7: M chip OOK in one OFDM symbol

After FFT, non-orthogonal noise is introduced to NR carriers as shown in Figure 8. We did some other simulations which show that the time delay doesn’t impact the analysis results much.

Figure 7: OOK signal after NR FFT

Observation 5: For backscattering based UL signal non-orthogonal impact to NR UE Rx, 2 chip OOK signal introduces -32 dB interference, 4 chip OOK signal introduces -28 dB interference.

In order to make the model simple, we propose use the same model for M-chip OOK UL.

Proposal 6: Use -30 dB 1 RB scaled emission level to model the backscattering based UL signal non-orthogonal impact to NR UE Rx.

2.2.4 AIoT UL impacts NR BS Rx

For AIoT UL impacts NR BS Rx, the analysis is the similar with NR UE. So they can use the same model.

Proposal 7: Use the same model for the A-IOT UL signal non-orthogonal interference to NR BS and NR UE.

  - CW unwanted emission model

For the carrier wave unwanted emission model for co-existence simulation, it was FFS in last meeting. In our understanding, it’s not necessary to model it in the co-existence simulation. The following figure is copied from TR 38.803.

Figure 8: Measured phase noise and corresponding model for 29.55 GHz (copied from TR 38.803)

It can be seen that phase noise is ~70 dBc at 1kHz offset for 30GHz NR fc. Usually low frequency LO phase noise performance is better than high frequency LO. 70 dBc noise is very low that the impact to the system can be ignored in the co-existence simulation. So we have the following proposal,

Proposal 8: CW unwanted emission is not necessary to be considered and modeled in the co-existence simulation because the phase noise contribution to the total noise is negligible.

- Conclusion

This contribution provides our views on some open issues for AIoT co-existence simulation parameters, we have the following observations and proposals,

For NR BS in-band emission assumption,

Observation 1: NR in-band emission has the about 80 kHz transient domain for the blank RB.

Proposal 1: Use ACLR approach to model NR BS in-band emission for co-existence simulation.

Proposal 2: NR BS ACLR for 1 blank RB is assumed as 12 dB when NR BS is transmitted full RB except this blank RB.

For NR Rx in-band rejection assumption,

Observation 2: The transmission time for 500 m NR macro coverage is 1.67 us which is half CP for 30 kHz SCS.

Observation 3: If more points in next AIoT symbol are included in the NR UE FFT window, more noise is introduced to NR UE Rx. If FFT window is offset with half CP, scaled SNR is ~ 37dB.

Proposal 3: It is proposed that UE vendors to check NR UE Rx FFT window choice implementation to double check AIoT DL signal’s non-orthogonal impact when AIoT reader is deployed nearby but NR BS is very far.

Proposal 4: It is proposed that UE vendors check NR UE receives large AIoT DL signal and small NR DL signal simultaneously, i.e. -25 dBm AIoT DL signal and -100 dBm NR DL signal may be received simultaneously by NR UE.

Observation 4: When NR BS received NR UL and AIoT DL signal timing difference is smaller than 40% of CP length, there’s no non-orthogonal impact for AIoT DL signal to NR BS Rx.

Proposal 5: The non-orthogonal impact for AIoT DL signal to NR BS Rx is not so critical so it can be ignored in the co-existence simulation.

Observation 5: For backscattering based UL signal non-orthogonal impact to NR UE Rx, 2 chip OOK signal introduces -32 dB interference, 4 chip OOK signal introduces -28 dB interference.

Proposal 6: Use -30 dB 1 RB scaled emission level to model the backscattering based UL signal non-orthogonal impact to NR UE Rx.

Proposal 7: Use the same model for the A-IOT UL signal non-orthogonal interference to NR BS and NR UE.

For CW unwanted emission model

Proposal 8: CW unwanted emission is not necessary to be considered and modeled in the co-existence simulation because the phase noise contribution to the total noise is negligible.

- References

[1] R4-2414304, “WF on Co-existence study for ambient IoT”, CMCC, RAN4#112B

[2] TR 38.803