3GPP TSG-RAN WG4 Meeting #112bis	                                                                                       R4-2415100

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	CATT

Title: 	Discussion on SBFD system parameters

Agenda item:	6.20.1

Document for:	Discussion

- Introduction

In the WF[1] agreed in last RAN4 meeting, there’re still many open issues need further discussion. This contribution provides our further analysis and proposals.

- Discussion

  - Specification

In the last meeting’s WF [1], there’re 3 options to handle the specification. After more discussions in the meeting, it seems new specification or embedding the requirements in current spec are not widely accepted, option 2 may be a compromise.

  - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable.

    - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

Considering the requirements are not settled, how to write the spec is not so clear, so option 2a may be better for the recent meetings. When the requirements are fixed, it can be reviewed which way is a clearer.

Observation 1: Option 2a in the WF[1] may be the better choice in recent meetings. How to write the spec can be discussed when most of the requirements are agreed.

  - Band/CBW support for SBFD

For the CBW support for SBFD, there’s some preliminary agreement that CBW > [20] MHz can be supported. The [20] MHz is still in brackets for further discussion. In our understanding, the decision should be based on operators’ request. If there’s no clear request, it’s not preferred to set unnecessary restrictions from the specification perspective.

Observation 2: Unnecessary restrictions on the CBW support for SBFD feature is not preferred from the specification perspective.

For how to implement the band and CBW support, there’re two options, i.e. adding sentence above the table or adding SBFD entry in the table. Both of them work but adding an entry may be more compatible in the future if some new duplex mode is introduced in the later release. So we slightly prefer the SBFD entry approach.

Proposal 1: Use adding SBFD entry to the table to introduce the SBFD bands.

  - Sub-band configuration and guard band size

For the options in the WF [1], we prefer option-2b as the compromise of the option 1 and option 3.

- Option-2b:

  - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

  - DL subband size(s) and guardband size(s) depends on BS implementation

  - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Option 1 brings too much restriction in the specification, if there’re more requests in the future, the standard work burden will be very high. Option 3 is flexible but brings difficulties on how to implement the test in the conformance test specification. So option 2 is a compromise and can meet almost all of the possible operator requests. Compared with option-2a, option 2b has the benefit that the FRCs don’t need to be re-discussed then make the standard discussion relatively easier.

Proposal 2: Option-2b in the WF[1] is agreed to handle the sub-band configuration for SBFD.

For the guard band size, if the above proposal can be approved, it seems the UL GB is fixed, however, in our understanding it’s not the intention of option-2b. How to handle the GB needs more discussion. One of the possible way is to set the restriction on the maximum allowed number of RBs for guard band which is the option 3 in the WF [1].

  - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband:

The restriction for GB may be separated to UL SB GB and DL SB GB. For the DL/UL SB GB, the maximum allowed RBs for GB is to reuse current GB sized if the CBW can be find in the current specification. If the CBW is a new CBW, then 10% CBW can be used as the restriction.

Proposal 3: Option 3 in WF [1] is agreed for the GB size for SBFD.

Proposal 4: The following approach can be considered to handle the maximum GB size for DL/UL GB.

- The maximum allowed number of RBs for GB is separated to DL SB GB and UL SB GB.

- The maximum GB restriction uses the GB size in current specification if it can be found.

- If SB CBW can’t be found in current specification, 10% GB size is used as the maximum GB restriction.

- Conclusion

This contribution provides our further analysis for SBFD general issues, we have the following observation and proposals.

For specification,

Observation 1: Option 2a in the WF[1] may be the better choice in recent meetings. How to write the spec can be discussed when most of the requirements are agreed.

For Band/CBW support for SBFD,

Observation 2: Unnecessary restrictions on the CBW support for SBFD feature is not preferred from the specification perspective.

Proposal 1: Use adding SBFD entry to the table to introduce the SBFD bands.

For sub-band configuration and guard band size,

Proposal 2: Option-2b in the WF[1] is agreed to handle the sub-band configuration for SBFD.

Proposal 3: Option 3 in WF [1] is agreed for the GB size for SBFD.

Proposal 4: The following approach can be considered to handle the maximum GB size for DL/UL GB.

- The maximum allowed number of RBs for GB is separated to DL SB GB and UL SB GB.

- The maximum GB restriction uses the GB size in current specification if it can be found.

- If SB CBW can’t be found in current specification, 10% GB size is used as the maximum GB restriction.

- References

[1] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112

[2] R4-2413407, “Topic summary for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112