3GPP TSG-RAN WG4 Meeting #112bis	                                                           R4-2415103

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on how to handle the RF requirements for 2U/2D CA_n5-n8

Source: 	CATT

Agenda item:	5.3.3

Document for:	Discussion

- Background

2UL CA_n5-n8 was discussed in the WI NR_700800900_combo_en [1] [2], but it was removed due to the limited time in R18. The study conclusion is included in TR 38.872 [3]. R19 CA basket [4] included this band combination again. This contribution summarizes the discussion status in WI NR_700800900_combo_en and proposes how to continue the discussion in R19.

- Discussion

When CA_n5-n8 was discussed in the SI and WI NR_700800900_combo_en, two architectures and 3 requirements options were discussed. The study output including RF architecture and RF requirements evaluation was captured in TR 38.872 for future reference.

The following is the two implementation architectures and the corresponding requirements options.

-------------------------------------------------------

Generally, in this study item, some high level implementations were discussed and shown as below.

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

Option 1: Only support 1UL/2DL CA. Single UL in n5

Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

Note: Potential impacts on RAN2 are observed

2) Dedicated RF filters implementation with partial frequency range

Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

--------------------------------------------------------

Option 1 is already defined in R18 specification. To support 2UL/2DL CA, option 2 and option 3 are on the table. In the discussions in R18, both option 2 and option 3 have supporters. So we propose to define two sets of UE RF requirements, UE can report the capability based on the implementation. For the RF requirements, no new discussion is needed. The MSD requirements for n8UL/n5 DL is converged in R18 WI as captured in TR 38.872,

Table 1: CA_n5-n8 MSD value for 1U/2D n8UL/n5DL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | 12.3 | >ACLR2 |

Based on the output in R18 WI, we propose to continue the discussion as following,

Proposal 1: Continue the discussion on 2UL/2DL CA_n5A-n8A based on the output in R18 WI NR_700800900_combo_en. No new RF evaluations are expected if no critical technical issues are found.

Proposal 2:  Two sets of requirements are defined in the spec, i.e. the full band filter architecture (Non-concurrent n5 DL and n8 UL) and dedicated filter architecture (simultaneous n5 DL and n8 UL).

Proposal 3: LS is sent to RAN2 to request RAN2 to define the UE capability signalling. A draft is provided in the Annex.

- Summary

The study output for 2UL/2DL CA_n5A-n8A in R18 WI NR_700800900_combo_en, we have the following proposals to continue the discussion in R19.

Proposal 1: Continue 2UL/2DL CA_n5A-n8A based on the output in R18 WI NR_700800900_combo_en. No new RF evaluations are expected if no critical technical issues are found.

Proposal 2:  Two sets of requirements are defined in the spec, i.e. the full band filter architecture (Non-concurrent n5 DL and n8 UL) and dedicated filter architecture (simultaneous n5 DL and n8 UL).

Proposal 3: LS is sent to RAN2 to request the UE capability signalling. A draft is provided in the Annex.

Reference

[1] RP-221464, “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, RAN#96

[2] RP-232680, “Revised WID on enhancement for 700/800/900MHz band combinations”, RAN#101

[3] TR 38.872

[4] RP-241674, “New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells”, RAN#104

[5] R4-2306465, “[Draft] LS on non-simultaneous UL and DL from different two bands during UL CA”, RAN4#106b

[6] R2-2306862, “Reply LS on non-simultaneous UL and DL from different two bands during UL CA”

[7] R4-2314656, “Reply LS on non-simultaneous UL and DL from different two bands during UL CA”, RAN4#108

[8] R4-2411077, “Discussion on RF requirements for 2U/2D CA_n5-n8”, RAN4#112

Annex: Draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #112bis	                                                           R4-24xxxxx

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	(Draft) LS on UE capability for 2UL/2DL CA_n5-n8

Response to:

Release:	Rel-19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:                  Huiping Shan

E-mail Address:   shanhuiping@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 has discussed the UE RF requirements for 2U/2D CA_n5-n8. It is agreed that two kinds of UE are defined for support 2U/2D CA_n5-n8,

- UE only supporting non-concurrent n5 DL and n8 UL

- UE supporting simultaneous n5 DL and n8 UL

RAN4 respectively asks RAN2 to define UE capability for 2U/2D CA_n5-n8.

2. Actions:

To RAN WG2:

RAN4 respectively asks RAN2 to define UE capability for 2U/2D CA_n5-n8.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #113	November 18 –November 22, 2024	Orlando, US

TSG RAN WG4 Meeting #114	Feb 17 – Feb 21, 2024	Athens, Greece