3GPP TSG-RAN WG4 Meeting # 112bis                                                        R4-2415106			R4-22xxxxx

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	SBFD in-band blocking and dynamic range simulation results

Source: 	CATT

Agenda item:	6.20.2.3

Document for:	Discussion

- Background

This contribution provides the SBFD in-band blocking and dynamic range simulation results from our company.

- Discussion

  - SBFD simulation scenarios

According to WF[1], simulation scenarios for SBFD in-band blocking include scenario 1, scenario 3, scenario 5, scenario 6, and scenario 9, and the case to be studied is Case 3, i.e. TDD DL to SBFD UL subband, SBFD is {DU} subband configuration.

Table 11.1-1 (from TR 38.858): Adjacent channel co-existence scenarios Scenario; FR; Aggressor; Victim. 1; FR1; Urban Macro; Urban Macro. 2; FR1; Urban Hotspot; Urban Hotspot. 3; FR1; Indoor; Indoor. 4; FR1; Urban Macro; Micro. 5; FR1; Micro; Micro. 6; FR2-1; Urban Macro; Urban Macro. 71; FR2-1; Urban Hotspot; Urban Hotspot. 8; FR2-1; Urban Dense; Urban Dense. 9; FR2-1; Indoor; Indoor. Note 1: This scenario has been down-selected.. Table 11.1-2 (from TR 38.858): Adjacent channel co-existence cases Case; Aggressor; Victim; Slot allocation Aggressor Victim. 1; SBFD; TDD DL. 2; SBFD; TDD UL. 3; TDD DL; SBFD. 4; TDD UL; SBFD. Note: Case 3 and Case 4 are down-selected for Scenario 4..

  - In-band blocking simulation results

2.2.1 Scenario 1: FR1, Urban Macro -> Urban Macro

Figure1: FR1, urban macro -> urban macro, in-band blocking

Table 1: FR1 SBFD urban macro -> urban macro conducted in-band blocking simulation results

|  | 99% CDF(100% grid-shift) | 99% CDF(10% grid-shift) |
| --- | --- | --- |
| Conducted in-band blocking (dBm) | -25.47 | 0.33 |

2.2.2 Scenario 3: FR1, Indoor -> Indoor

Figure2: FR1, indoor -> indoor, in-band blocking

Table 2: FR1 SBFD indoor -> indoor conducted in-band blocking simulation results

|  | 99% CDF(100% grid-shift) |
| --- | --- |
| Conducted in-band blocking (dBm) | -34.42 |

2.2.3 Scenario 5: FR1, Micro -> Micro

Figure3: FR1, micro -> micro, in-band blocking

Table 3: FR1 SBFD micro -> micro conducted in-band blocking simulation results

|  | 99% CDF(100% grid-shift) | 99% CDF(10% grid-shift) |
| --- | --- | --- |
| Conducted in-band blocking (dBm) | -32.19 | -17.24 |

2.2.4 Scenario 6: FR2-1, Urban Macro -> Urban Macro

Figure4: FR2-1, urban macro -> urban macro, IBB over wanted signal

Table 4: FR2-1 SBFD urban macro -> urban macro OTA in-band blocking simulation results

|  | 99% CDF(100% grid-shift) | 99% CDF(10% grid-shift) |
| --- | --- | --- |
| IBB over wanted signal (dB) | 24.7 | 36.6 |

2.2.5 Scenario 9: FR2-1, Indoor -> Indoor

Figure5: FR2-1, indoor -> indoor, IBB over wanted signal

Table 5: FR2-1 SBFD indoor -> indoor OTA in-band blocking simulation results

|  | 99% CDF(100% grid-shift) |
| --- | --- |
| IBB over wanted signal (dB) | 19.9 |

  - Rx dynamic range simulation results

The scenarios for Rx dynamic range simulation are the same as that for SBFD in-band blocking, both uplink signal and gNB-to-gNB CLI signal are considered in the simulations.

2.2.1 Scenario 1: FR1, Urban Macro -> Urban Macro

Figure 6: FR1, urban macro -> urban macro, dynamic range

Table 6: FR1 SBFD urban macro -> urban macro conducted dynamic range simulation results

|  | Acc | 100% grid-shift |  | 10% grid-shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 99% CDF | 100% CDF | 99% CDF | 100% CDF |
| Dynamic range (dBm) | 0dB | -85.4 | -80.0 | -84.1 | -76.9 |
| IoT(dB) |  | 15.5 | 21.0 | 16.8 | 24.0 |
| Dynamic range (dBm) | ±9dB | -85.1 | -78.2 | -83.5 | -77.4 |
| IoT(dB) |  | 15.9 | 22.8 | 17.4 | 23.5 |

2.2.2 Scenario 3: FR1, Indoor -> Indoor

Figure7: FR1, indoor -> indoor, dynamic range

Table 7: FR1 SBFD indoor -> indoor conducted dynamic range simulation results

|  | Acc | 100% grid-shift |  |
| --- | --- | --- | --- |
|  |  | 99% CDF | 100% CDF |
| Dynamic range (dBm) | 0dB | -85.7 | -84.3 |
| IoT(dB) |  | 15.3 | 16.7 |
| Dynamic range (dBm) | ±9dB | -82.8 | -80.0 |
| IoT(dB) |  | 18.2 | 21.0 |

2.2.3 Scenario 5: FR1, Micro -> Micro

Figure 8: FR1, micro -> micro, dynamic range

Table 8: FR1 SBFD micro -> micro conducted dynamic range simulation results

|  | Acc | 100% grid-shift |  | 10% grid-shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 99% CDF | 100% CDF | 99% CDF | 100% CDF |
| Dynamic range (dBm) | 0dB | -91.0 | -85.4 | -88.0 | -82.4 |
| IoT(dB) |  | 10.0 | 15.5 | 13.0 | 18.5 |
| Dynamic range (dBm) | ±9dB | -88.9 | -79.4 | -85.9 | -76.4 |
| IoT(dB) |  | 12.0 | 21.5 | 15.1 | 24.6 |

2.2.4 Scenario 6: FR2-1, Urban Macro -> Urban Macro

Figure 9: FR2-1, urban macro -> urban macro, dynamic range

Table 9: FR1 SBFD urban macro -> urban macro OTA dynamic range simulation results

|  | Acc | 100% grid-shift |  | 10% grid-shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 99% CDF | 99.9% CDF | 99% CDF | 99.9% CDF |
| IoT(dB) | 0dB | 8.9 | 8.9 | 11.5 | 12.2 |
|  | ±9dB | 17.6 | 19.6 | 21.5 | 21.8 |

2.2.5 Scenario 9: FR2-1, Indoor -> Indoor

Figure10: FR2-1, indoor -> indoor, dynamic range

Table 10: FR2-1, SBFD indoor -> indoor OTA dynamic range simulation results

|  | Acc | 100% grid-shift |  |
| --- | --- | --- | --- |
|  |  | 99% CDF | 99.9% CDF |
| IoT(dB) IoT(dB) | 0dB | 0.5 | 1.6 |
|  | ±9dB | 2.6 | 7.6 |

- Conclusion

- Simulation results of in-band blocking are summarized in table 6 for FR1 and table 7 for FR2-1.

Table 11: SBFD conducted in-band blocking simulation results (FR1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | Conducted in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF(100% grid-shift) | 99% CDF(10% grid-shift) |
| 1 | FR1 | Urban Macro -> Urban Macro | -25.47 | 0.33 |
| 3 | FR1 | Indoor -> Indoor | -34.42 | —— |
| 5 | FR1 | Micro -> Micro | -32.19 | -17.24 |

For FR1 urban macro scenario, the in-band blocking signals power levels are much higher than the existing IBB requirement.

For FR1 indoor scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

For FR1 micro scenario, the in-band blocking signals power level of 100% grid shift is close to the existing IBB requirement, but the power level of 10% grid shift is much higher than the existing IBB requirement.

Table 12: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | IBB over wanted signal (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 6 | FR2-1 | Urban Macro -> Urban Macro | 24.7 | 36.6 |
| 9 | FR2-1 | Indoor -> Indoor | 19.9 | —— |

For FR2-1 urban macro scenario, the in-band blocking signals power level is close to the existing IBB requirement, and can reuse the existing requirements.

For FR2-1 indoor scenario, the in-band blocking signals power level is lower than the existing IBB requirement, and can reuse the existing requirements.

- Simulation results of Rx dynamic range for FR1 and FR2-1 are summarized in table 13.

Table 13:  Rx dynamic range simulation results

| Scenario | Frequency | Aggressor -> Victim | Acc | IoT (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100% grid-shift |  | 10% grid-shift |  |
|  |  |  |  | 99% CDF | 100% CDF | 99% CDF | 100% CDF |
| 1 | FR1 | Urban Macro -> Urban Macro | 0dB | 15.5 | 21.0 | 16.8 | 24.0 |
|  |  |  | ±9dB | 15.9 | 22.8 | 17.4 | 23.5 |
| 3 | FR1 | Indoor -> Indoor | 0dB | 15.3 | 16.7 | —— | —— |
|  |  |  | ±9dB | 18.2 | 21.0 | —— | —— |
| 5 | FR1 | Micro -> Micro | 0dB | 10.0 | 15.5 | 13.0 | 18.5 |
|  |  |  | ±9dB | 12.0 | 21.5 | 15.1 | 24.6 |
| 6 | FR2-1 | Urban Macro -> Urban Macro | 0dB | 8.9 | 8.9 | 11.5 | 12.2 |
|  |  |  | ±9dB | 17.6 | 19.6 | 21.5 | 21.8 |
| 9 | FR2-1 | Indoor -> Indoor | 0dB | 0.5 | 1.6 | —— | —— |
|  |  |  | ±9dB | 2.6 | 7.6 | —— | —— |

The simulation results of IoT consider both uplink signal and gNB-to-gNB CLI signal.

When the grid shift is 100% and the CDF is 100%, the IoTs of scenario1, scenario 3 and scenario 5 are close to 20dB, and the IoT of scenario 6 is much higher than existing requirement, IoT of scenario 9 is 7.6dB.

When the grid shift is 10% and the CDF is 100%, IoTs of both FR1 indoor and FR2-1 indoor scenarios can reuse the existing requirements. IoTs of scenario1 and 5 are much higher than 20dB.

Reference

[1] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, HiSilicon , RAN4#112