3GPP TSG-RAN WG4 Meeting # 112bis                                                         R4-2415109

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on potentially new requirements for SBFD operation

Source: 	CATT

Agenda item:	6.20.2.1

Document for:	Discussion

- Background

WF [1] of RAN4#111 concluded that further studies are needed for potentially new RF requirements, in this contribution, we provide our further analysis of them.

- New requirements for SBFD operation

2.1 Transient period

In WF [1], agreements of transmitter transient period are shown following.

- Issue 2-1-3: Location of transient period between SBFD and non-SBFD

- Agreement:

  - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement:

    - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

To minimize the impact on the non-SBFD system, the transition period between SBFD and non-SBFD should be located within the SBFD slot.

Proposal 1: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot.

2.2 In channel new requirements

For the following new RF requirements,

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

Considering the performances of the potential new SBFD in channel requirements are actually included in the Self-interference performance, it may be possible not defining these new requirements.

Proposal 2: The following new requirements are not defined for SBFD BS.

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

- Summary

In this contribution, we have the following proposals.

Proposal 1: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot.

Proposal 2: The following new requirements are not defined for SBFD BS.

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

Reference

[1] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112

[2] TR 38.858