3GPP TSG-RAN WG4 Meeting #112bis	R4-2415120

Hefei, China, Oct 14th – 18th, 2024

Title: 	On radio and antenna parameters for 14.8 - 15.35GHz frequency range

Agenda Item:	6.2.4.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, more discussions have been conducted on the third frequency range of 14.8 – 15.38 GHz regarding the IMT-2030 parameters for compatibility and sharing study, which is captured in the agreed WFs [1][2] for BS and UE respectively. There are still several open parameters for further evaluation and simulation. However, it is important to conclude BS antenna configuration including the antenna array size and sub-array size in this meeting:

Issue 2-1: BS antenna array and sub-array size - Recommended for further discussions - 3072 AEs dual polarization : - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded.

In this contribution, we propose to take 2048 antenna elements for dual polarization based on the simulation results which we have conducted up to now.

# 2 Discussion

The detailed simulation setup and corresponding results are available in our companion paper [3]. However, to determine BS antenna array and sub-array size based on these results, we must take into account the associated costs. To address this, we introduce a new concept: throughput per antenna element, which is calculated by dividing the total throughput by the total number of antenna elements. Although this metric does not represent the total cost directly, it provides an indirect measure of the trade-off between cost and performance.

Observation 1: The throughput per antenna element provides an indirect measurement of the trade-off between cost and performance.

The following figures illustrate the throughput (including DL average throughput, DL edge throughput, UL average throughput, and UL edge throughput) per antenna element for various BS antenna configurations.

For the sake of convenience, we recap the option terms in the following table:

| BS array Option | Array | Sub-array size | Total |
| --- | --- | --- | --- |
| 1 | 16x24 | 4 | 3072 |
| 2-1 | 8x32 | 8 | 4096 |
| 2-2 | 16x32 | 4 | 4096 |
| 3 | 8x32 | 4 | 2048 |
| 4 | 8x16 | 6 | 1536 |

Fig.1, DL average throughput per antenna element

Fig.2, DL edge throughput per antenna element

Fig.3, UL average throughput per antenna element

Fig.4, UL edge throughput per antenna element

From the figures above, Option 3 consistently ranks as either the best or second best in terms of throughput per antenna element (including DL average throughput, DL edge throughput, UL average throughput, and UL edge throughput). While other options may excel in one category, they perform significantly worse in others, making Option 3 the overall superior choice compared to the rest.

Observation 2: Option 3 is the overall superior choice among all the recommended options, consistently ranking either first or second in terms of throughput per antenna element, as demonstrated by our simulation results.

In this case, we propose to take Option 3, i.e., BS antenna array is set as 8x32, and sub-array size as 4 considering the trade-off between cost and performance.

Proposal 1: Set the BS antenna array configuration to 8 x 32 as an optimal trade-off between cost and performance.

Proposal 2: Set the BS sub-array size as 4 as the optimal trade-off between cost and performance.

# 3 Conclusion

In this contribution, we have the following observations and proposals for BS antenna array configuration for 14.8 – 15.35 GHz frequency range:

Observation 1: The throughput per antenna element provides an indirect measurement of the trade-off between cost and performance.

Observation 2: Option 3 is the overall superior choice among all the recommended options, consistently ranking either first or second in terms of throughput per antenna element, as demonstrated by our simulation results.

Proposal 1: Set the BS antenna array configuration to 8 x 32 as an optimal trade-off between cost and performance.

Proposal 2: Set the BS sub-array size as 4 as the optimal trade-off between cost and performance.

# Reference

[1] R4-2414283, “WF on 15GHz BS parameters”, ZTE

[2] R4-2414284, “WF for 15GHz UE parameters”, Qualcomm

[3] R4-241xxxx, “R4-241xxxx On coexistence evaluation results for 14.8 - 15.35GHz frequency range”, CATT