3GPP TSG-RAN WG4 Meeting #112bis	R4-2415123

Hefei, China, Oct 14th – 18th, 2024

Title: 	On sync raster applicability

Agenda Item:	6.4.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, good progress has been made with a WF agreed [1]. While there is a general aspect on sync raster applicability for band n100 as an Scell, the exact specs wording is open for further discussion:

- For band n100 - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - Whether and how to capture the respective notes in the specification can be decided subject to the decisions on UE capability signaling in the WI RF maintenance stage. - The candidate CR text - Option 1: “SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency position defined in Table 5.4.3.1-3.” - Other options not precluded

In this contribution, it seems to us that the agreement and candidate CR text could be misunderstood hence we present our proposals accordingly.

# 2 Discussion

Though the intention is to decide on the CR text after concluding UE capability signaling in WI RF maintenance stage, it is not harmful to discuss the potential texts at this stage since the overall progress seems ahead of the plan and we do not need to just sit there and wait.

The cited Table 5.4.3.1-3 is shown below for the convenience of reader:

And the candidate CR text per agreements seems to state that the only applicable sync raster for Scell with 12 PRB transmission bandwidth configuration within 3MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5MHz channel bandwidth is specified in Table 5.4.3.1-3 for band n100. However, in order to avoid any mis-understanding, we may need to confirm with operators on whether other regular sync raster introduced for 3MHz channel bandwidth as specified in Table 5.4.3.1-2 is also be applicable for band n100.

Observation: The agreement and candidate CR text excludes the applicability of regular sync raster introduced for 3MHz channel bandwidth as specified in Table 5.4.3.1-2.

Proposal 1: In order to avoid misunderstanding, RAN4 to confirm with operators on whether other regular sync raster introduced for 3MHz channel bandwidth as specified in Table 5.4.3.1-2 is also be applicable for band n100.

If operators confirm the other regular sync raster entries as specified in Table 5.4.3.1-2, it implies that the same set of sync raster entries will apply to both the PCell and SCell when using a 3MHz or 5MHz channel bandwidth with a reduced transmission bandwidth configuration, and the candidate CR text could just be simplified as “the same set of sync raster entries applies to both PCell and Scell with a 3MHz or 5MHz channel bandwidth with reduced transmission bandwidth configuration”, or even without any need to clarify this point.

Proposal 2: If operators confirm other regular sync raster entries as specified in Table 5.4.3.1-2, simply state in specs that the same set of sync raster entries applies to both PCell and Scell with a 3MHz or 5MHz channel bandwidth with reduced transmission bandwidth configuration, or even without any need to clarify this point.

# 3 Conclusion

In this contribution, we have the following observation and proposals for sync raster applicability:

Observation: The agreement and candidate CR text excludes the applicability of regular sync raster introduced for 3MHz channel bandwidth as specified in Table 5.4.3.1-2.

Proposal 1: In order to avoid misunderstanding, RAN4 to confirm with operators on whether other regular sync raster introduced for 3MHz channel bandwidth as specified in Table 5.4.3.1-2 is also be applicable for band n100.

Proposal 2: If operators confirm other regular sync raster entries as specified in Table 5.4.3.1-2, simply state in specs that the same set of sync raster entries applies to both PCell and Scell with a 3MHz or 5MHz channel bandwidth with reduced transmission bandwidth configuration, or even without any need to clarify this point.

# Reference

[1] R4-2414451, “WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2”, Intel