3GPP TSG-RAN WG4 Meeting #112bis	R4-2415125

Hefei, China, Oct 14th – 18th, 2024

Title: 	Scope clarification on DL fragmented carriers

Agenda Item:	6.6.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4 has started the discussion on the study of DL fragmented carriers with the following updated objectives [1]:

- Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements for study purpose are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

It appears to us that some clarification may still be required to facilitate the study, and in this contribution we present our proposals on the potential clarification.

# 2 Discussion

Frequency span

For 100MHz frequency span, there were different understandings discussed in RAN4#112:

- Understanding#1: the bandwidth of the whole NR band

- Understanding#2: the minimum span covering all the non-contiguous fragmented DL carriers, i.e., the frequency range between the lower frequency end of the lowest CC and the higher frequency end of the highest component.

Although an agreement reached as shown in the following according to the WF [2], it has not been reflected in the SID.

    - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

Hence, we propose to make the clarification in SID accordingly. One possible way is to move “with frequency span ≤ 100 MHz” after “CA combination” as illustrated below:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band, containing two non-contiguous CCs within a CA combination with frequency span ≤ 100MHz for the inter-operator co-located scenario, considering:…

Proposal 1: RAN4 to revise the objective of the SID as the following to clarify the scope:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band, containing two non-contiguous CCs within a CA combination with frequency span ≤ 100MHz for the inter-operator co-located scenario, considering:…

Performance impact

Operators who intend to enable this feature will ultimately need to accept some level of "cost" in terms of network performance. The potentially impacted UE RF requirements should be derived based on this trade-off. Since the relaxation is applied only to DL reception, it is clear that the impact will be on DL performance between the base station and the UE supporting this feature. A key focus of the study should be to examine the potential relationship between DL performance degradation and the corresponding relaxation of UE RF requirements, as well as to identify acceptable levels of DL performance degradation from the operators' perspective.

Therefore, we propose to elaborate a bit more on the sub-bullet under the main objective bullet:

  - Impacts on DL performance [RAN4] ;

One candidate change could be:

  - Impacts on DL performance versus adjustments to potentially impacted UE RF requirements, and identified acceptable levels of DL performance degradation for this feature from operators’ perspective [RAN4] ;

Proposal 2: RAN4 to clarify the DL performance impacts in the objective of the SID as the following revision:

  - Impacts on DL performance versus adjustments of potentially impacted UE RF requirements, and identified acceptable levels of DL performance degradation for this feature from operators’ perspective [RAN4] ;

# 3 Conclusion

In this contribution, we have the following proposals on the objective and scope clarification on the study of DL fragmented carriers:

Proposal 1: RAN4 to revise the objective of the SID as the following to clarify the scope:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band, containing two non-contiguous CCs within a CA combination with frequency span ≤ 100MHz for the inter-operator co-located scenario, considering:…

Proposal 2: RAN4 to clarify the DL performance impacts in the objective of the SID as the following revision:

Impacts on DL performance versus adjustments to potentially impacted UE RF requirements, and identified acceptable levels of DL performance degradation for this feature from operators’ perspective [RAN4];

# Reference

[1] RP-242404, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek

[2] R4-2414457, “WF on fragmented DL carriers study”, MediaTek