3GPP TSG-RAN WG4 Meeting #112bis	R4-2415126

Hefei, China, Oct 14th – 18th, 2024

Title: 	Discussion on methods for reducing the number of UE Rx chains

Agenda Item:	6.6.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

RAN4#112 has initiated the study for fragmented DL carriers [1] and a WF was approved where the consensus is mostly on some clarification on the scope [2]. Regarding the methods for reducing the number of UE Rx chains, discussions were held however no consensus got reached, hence these issues are open for further discussion.

In this contribution, we will continue the discussion on the open issues and share our perspectives and proposals.

# 2 Discussion

Reference architecture

During the discussion in the previous meeting, an issue was raised that RAN4 needs to investigate only “fully shared” architecture for the two non-contiguous DL carriers in the study item [2]. The key point here is the “interfering carriers” located in the gap of these two non-contiguous CCs. It seems that there is no regulatory guarantee that the interfering carriers will have sub-carrier alignment with the two non-contiguous component carriers, nor is there any assurance of this alignment from the specification.

Observation: There is neither regulation nor specification requirement to ensure the sub-carrier alignment of the interfering carriers located in the gap with the two non-contiguous CC.

If a “fully shared” architecture is used in the UE receive chain, the interfering carriers located in the gap between the two non-contiguous component carriers may degrade orthogonality, resulting in pure interference that could potentially jam the receiver entirely at a certain interference level. As a result, using a “fully shared” architecture in this scenario might offset the advantages of intra-band non-contiguous CA, hence if only the “fully shared” architecture is assumed for the feature of reducing the number of UE Rx chains, the study needs to identify the tolerance threshold from the interfering carriers to enable this feature for DL CA with the two non-contiguous CCs.

Proposal 1: if only the “fully shared” architecture is assumed for the feature of reducing the number of UE Rx chains, RAN4 should identify the tolerance threshold from the interfering carriers to enable this feature for DL CA with the two non-contiguous CCs in the study item.

Power spectral density difference between carriers of co-located adjacent channel operators

There was also discussion on an issue on the PSD difference between the two non-contiguous CCs and the co-location [2]. In the SID, it clearly states that the target scenario is “the inter-operator co-located” [1]:

Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario

And a straightforward understanding on the “inter-operator co-located scenario” is that for the two non-contiguous CCs are from two different operators, and co-sited only without sharing any infrastructure such as RRUs. And if there is a different understanding that the target includes infrastructure sharing, then a revision on the SID is needed to clarify the target scenario.

Proposal 2: RAN4 to follow the current SID and focus on the inter-operator co-located scenario in the study where no infrastructure sharing is assumed, and if infrastructure sharing is intended as well, a revision on the SID is required.

Regarding the PSD difference between the two non-contiguous CCs, it could be received PSD difference from UE perspective, and also transmitted PSD difference from BS perspective. For the former, the current SID clearly indicates that up to 6 dB:

- Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs

And there is also a sub-bullet regarding the PSD difference at transmitter side:

- Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

For the simplicity of the study, RAN4 can start with equal transmitted PSD with each CC transmitting at its maximum output power.

Proposal 3: RAN4 to follow the current SID allowing up to 6 dB received PSD difference at UE and start with equal transmitted PSD with each CC transmitting at its maximum output power.

# 3 Conclusion

In this contribution, we have the following observation and proposals for methods for reducing the number of UE Rx chains:

Observation: There is neither regulation nor specification requirement to ensure the sub-carrier alignment of the interfering carriers located in the gap with the two non-contiguous CC.

Proposal 1: if only the “fully shared” architecture is assumed for the feature of reducing the number of UE Rx chains, RAN4 should identify the tolerance threshold from the interfering carriers to enable this feature for DL CA with the two non-contiguous CCs in the study item.

Proposal 2: RAN4 to follow the current SID and focus on the inter-operator co-located scenario in the study where no infrastructure sharing is assumed, and if infrastructure sharing is intended as well, a revision on the SID is required.

Proposal 3: RAN4 to follow the current SID allowing up to 6 dB received PSD difference at UE and start with equal transmitted PSD with each CC transmitting at its maximum output power.

# Reference

[1] RP-242404, “Revised SID: Study on NR FR1 DL fragmented carriers”, MediaTek

[2] R4-2414457, “WF on fragmented DL carriers study”, MediaTek