3GPP TSG-RAN WG4 Meeting #112bis	R4-2415128

Hefei, China, Oct 14th – 18th, 2024

Title: 	Discussion on UE RF requirements for RedCap PC2

Agenda Item:	6.7.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#105, the WID on RedCap PC2 was revised by removing the target band table [1], hence the feature is aimed in a band agnostic way, which is partially aligned with the agreement made in RAN4#112:

Issue 1-1-1: PC2 RedCap UE for TDD bands Agreement: Add one sentence in the clause of 6.2I.1 to state that PC2 also applies for RedCap/eRedCap UE in a band agnostic way for TDD bands

In our understanding, the RAN4 agreements may be extended for FDD bands as well to follow the revised WID.

# 2 Discussion

As discussed in our paper in the previous meeting, Tx requirements in the current specs are not completely specified for RedCap UE as Rx requirements do, therefore a similar general sub-clause is needed. The following shows one example for Rel-17 and Rel-18 respectively:

Rel-17 6.1I General For Redcap, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Rel-18 6.1I General For Redcap and eRedCap UE, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

In the WF, a starting point is proposed in the background section [2], however, it is not suitable only under 6.2I with a hanging text since not only transmitter power but also all other Tx requirements may be covered.

Proposal 1: RAN4 to add a general sub-clause 6.1I as the following into the specs for Rel-17 and Rel-18 respectively:

Rel-17 6.1I General For Redcap UE, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Rel-18 6.1I General For Redcap and eRedCap UE, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Regarding the maximum output power part, it should reflect the revised WID and be concise. One candidate example is illustrated below:

### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 and power class 2 specified in clause 6.2.1 apply.

Proposal 2: RAN4 to update the sub-clause 6.2I.1 as following:

### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 and power class 2 specified in clause 6.2.1 apply.

Our companion draft CR captures the above revisions [4].

# 3 Conclusion

In this contribution, we have the following proposals for UE requirements for RedCap PC2:

Proposal 1: RAN4 to add a general sub-clause 6.1I as the following into the specs for Rel-17 and Rel-18 respectively:

Rel-17 6.1I General For Redcap UE, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Rel-18 6.1I General For Redcap and eRedCap UE, the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Proposal 2: RAN4 to update the sub-clause 6.2I.1 as following:

### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 and power class 2 specified in clause 6.2.1 apply.

# Reference

[1] RP-241997, “Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom

[2] R4-2414286, “WF on RedCap PC2 requirements”, China Telecom

[3] R4-2411115, “On UE RF requirements for PC2 RedCap”, CATT

[4] R4-2415129, “draftCR to TS 38.101-1 for RedCap PC2”, CATT