3GPP TSG-RAN WG4 Meeting #112bis	R4-2415142

Hefei, China, Oct 14th – 18th, 2024

Title: 	On Solution to enable HPUE maximum transmit power in downlink CA with single UL transmission

Agenda Item:	8.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For the RAN task concerning the enablement of HPUE maximum transmit power in DL CA with a single UL configuration [1], papers submitted prior to the meeting were not addressed, and as a result, no online discussion took place. Only a final attempt at making a way forward was made [2], but ultimately, no consensus was reached during RAN4#112 with the WF not approved.

There are several different proposals in the previous meeting:

- Enable the feature with only MSD verified for the default power class [3][4].

- Enable the feature with defined MSD for high power class of the band combination [5].

- Enable the feature as long as high power class of the single UL band is specified [6], which is a bit aggressive at this stage.

To facilitate the discussion, we can primarily focus on resolving two key issues:

(1) How to handle cases where single UL transmission is in a higher power class while the corresponding MSD is missing for the band combination;

(2) How to report the higher power capability for DL CA with single UL configuration.

In this contribution, we further discuss these two issues and present our views.

# 2 Discussion

Issue on the missing MSD

To enable HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, either it is required to only verify MSD for the default power class, or wait until MSD for the high power class is defined.

In our view, nothing comes without a cost. In this case, the trade-off could be a degradation in downlink performance, which the network should be aware of and prepared for, e.g., compensated by more re-transmissions in time domain. Essentially, downlink performance degradation is exchanged for higher uplink power transmission. Under such consideration, the feature could be enabled.

Proposal 1: A UE is allowed for HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, with awareness by network that the downlink performance may suffer from a degradation and is able to be compensated in other means.

Issue on the capability reporting

If a UE support such a feature, should a new UE capability signaling be introduced or some existing UE capability, e.g., ue-PowerClass in BandNR or ue-PowerClassPerBandPerBC-r17 for the band combination.

If an existing UE capability is reused for this purpose, it could lead to NBC issues. This is because legacy UEs that do not support the feature might share the same existing UE capability, making it impossible for the network to differentiate between the two types of UEs. Therefore, a new UE capability would be necessary if RAN4 agrees to support this feature.

Proposal 2: RAN4 to introduce a new UE capability if agreeing to support this feature.

# 3 Conclusion

In this contribution, we have the following proposals for the solution to enable HPUE maximum transmit power in DL-only CA with single UL configuration:

Proposal 1: A UE is allowed for HPUE maximum transmit power in DL-only CA with single UL configuration for a band combination whose MSD for the high power class is not specified yet but the high power class of the said UL band is supported, with awareness by network that the downlink performance may suffer from a degradation and is able to be compensated in other means.

Proposal 2: RAN4 to introduce a new UE capability if agreeing to support this feature.

# Reference

[1] RP-241687, “Moderator's summary for offline discussion on CA Power Class Support”, Moderator(AT&T)

[2] R4-2414436, “Way Forward for RAN task on HPUE with 1UL and DL CA”, Qualcomm, et., al.

[3] R4-2411677, “Enabling HPUE maximum transmit power in downlink CA with a single configured UL”, Ericsson

[4] R4-2413140, “Support for HPUE with DL CA with single UL CC configured”, Qualcomm

[5] R4-2412354, “RAN task R17 power class applicability related”, OPPO

[6] R4-2411098, “On solution to enable HPUE maximum transmit power in downlink CA with single UL transmission”, CATT