3GPP TSG RAN WG4 Meeting #112bis	R4-2415147

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.24.2.2

Source:	vivo

Title:	Initial discussion on Conditional Intra-CU LTM

Document for:	Discussion

- Introduction

The new WID of NR mobility enhancements Phase 4 was approved in the RAN plenary RAN#105 meeting [1], which included the update of objective for supporting conditional Intra-CU LTM. The details of the objective are shown below. As per the working arrangement, the corresponding discussion will be triggered in RAN2 and RAN4 parallelly this meeting.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide our initial thinking on the potential RRM impacts for supporting conditional Intra-CU LTM.

- Discussion

For conditional mobility, CHO and CPAC were introduced in Rel-16 and Rel-17. The corresponding procedure can be executed without necessitating a signaling exchange with the source cell. Further enhancements including CHO with (conditional) PSCell addition/change and subsequent CPAC were developed in Rel-18. The corresponding delay requirements for each procedure are defined in RRM spec.

For LTM, it was introduced to achieve a short interruption time compared with the L3 measurement. The related RRM requirements were specified in Rel-18 and the further enhancements are discussed in Rel-19.

To benefit from high robustness by enabling a similar mechanism in LTM procedure, as per the objective described in WID above, it aims to support both conditional LTM and subsequent conditional LTM, which are somehow like the current conditional mobility, i.e., CHO/CPAC and subsequent CPAC.

From RRM perspective, the work mainly focuses on the definition of delay requirements, including conditional LTM and subsequent conditional LTM, respectively. In general, the discussions on CHO/CPAC and subsequent CPAC that we ever had can be the reference.

As for the details of delay requirements, e.g., the supportive scenarios and the concrete delay components, it highly depends on the discussion in RAN2. For example, the fundamental design such as 1) Whether both RACH-based and RACH-less based conditional LTM are supported; 2) The execution condition and corresponding event, shall be determined in RAN2 first. Considering this is the first meeting for both RAN2 and RAN4, to avoid duplicate discussion, our suggestion is to suspend detailed discussion in RAN4 until enough RAN2 progress on the supportive scenario and procedure is made.

Proposal 1: RAN4 to define delay requirements respectively for conditional LTM and subsequent conditional LTM and suspend the detailed discussion until enough RAN2 progress on the supportive scenarios and procedure is made

- Summary

Proposal 1: RAN4 to define delay requirements respectively for conditional LTM and subsequent conditional LTM and suspend the detailed discussion until enough RAN2 progress on the supportive scenario and procedure is made

- References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom