3GPP TSG RAN WG4 Meeting #112bis	R4-2415148

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.26.4.2

Source:	vivo

Title:	Discussion on potential RRM impacts on DL coverage and regenerative payload for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 initially analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress were summarized in WF [1]. As per the arrangement on agenda, in this contribution, we would like to provide further analysis on aspects of potential RRM impacts on DL coverage and regenerative payload.

- 2.	Discussion

  - Downlink coverage enhancements

#### Issue 2-1-1: RRM impact of SSB periodicity enhancement <Way Forward> : Further discuss the following proposal - Proposal 1: - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a: - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b: RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.). #### Issue 2-2-1: RRM impact of DTX/DRX <Way Forward> : Further discuss the following proposal - Proposal 1: - RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES. - Proposal 2: - All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs. - Proposal 3: - RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of DRX periodicity.

On the enhancement of SSB periodicity extension, as per the latest progress in RAN1, for the initial access procedure, the candidate values of extended SSB periodicity are considered as follows. RAN1 will proceed with further feasibility evaluation on extension of SSB periodicity to these values. RAN4 needs to wait for further conclusion on the default values from RAN1.

Agreement in RAN1#118 meeting As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

Specifically, it can be observed that the default values larger than 160ms are also under consideration, i.e., 320ms and 640ms, which would bring potential impacts on RRM requirements. Some examples are listed as follows:

- For timing error requirements for NTN, it requires that UE shall meet Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. If SSB periodicity is extended larger than 160ms, there may be no SSB available during the last 160ms. Consequently, the current Te_NTN requirement shall be re-considered.

- For the gradual timing adjustment, one of the adjustment rules is that the maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms. When SSB periodicity is extended to 320ms or 640ms, the frequency shift may be larger, which may result in the current maximum aggregate adjustment rate not being sufficient to compensate for the frequency offset.

- For the requirements on SSB-based RLM/BFD, since the current maximum SSB periodicity can be configured is 160ms, the cases of evaluation period are divided by DRX configuration and the boundary is set as 320ms. If SSB periodicity is extended to 640ms, the boundary of evaluation/measurement period shall be updated accordingly.

From RAN4 perspective, the RRM requirements related to SSB periodicity need to be revisited if RAN1 agrees to introduce the extended SSB periodicity.

Proposal 1: RAN4 to wait for more progress from other WGs on SSB periodicity extension to discuss the RRM impacts of DL coverage enhancement.

On the DRX/DTX aspects, it is still under discussion in other working groups and related to the conclusion on the concrete value of extended SSB periodicity. The potential impacts of RAN4 can wait until RAN1/RAN2 figure out a clear solution.

Proposal 2: RAN4 to wait for more conclusions from RAN1/2 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

  - Support of regenerative payload

#### Issue 5-3-1: RRM requirements for RRC_INACTIVE state mobility with regenerative payload <Way Forward> - Whether to update the specific RRM requirements for inactive state with regenerative payload should wait for RAN2/RAN3 progress. #### Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload <Way Forward> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

RAN3 agreement

RAN2 agreement

For regenerative payload, whether to update RRM requirements on the specific scenarios as above, depends on the RAN2/RAN3 process. Based on the latest progress in RAN2/RAN3 as shown above, it is assumed that the current mechanisms of RRC connection re-establishment and RACH-less HO can be applied. From RAN4 perspective, the existing requirements for these procedures in NR NTN can be reused for regenerative payload.

On whether to update RRM requirements for inactive state, according to the agreement in RAN3, the mechanism of RRC connection resume is supported, which is performed specifically in inactive mode. From RRM requirement perspective, the requirements defined for inactive mode are similar as idle mode. There is no potential issue or difference observed for supporting inactive mode with regenerative payload. Based on these two points, our view is that the current specific RRM requirements for inactive states can be reused in regenerative architecture and no further update is needed.

Proposal 3: The existing RRM requirements for inactive state, RRC re-establishment and RACH-less handover can be reused for regenerative payload.

For supporting of satellite switching without PCI change with gNB on aboard, it was also proposed and discussed in RAN plenary. According to the latest outcome in RAN#105 meeting, so far this scenario is not supported in regenerative architecture. The related discussion on supporting of satellite switching without PCI change with regenerative payload is out of scope. Thus, from RRM perspective, the clarification on requirement of satellite switching without PCI change is not applied for regenerative mode of Rel-19 NTN.

Observation 1: The discussion on supporting of satellite switching without PCI change with regenerative payload is out of Rel-19 scope.

Proposal 4: RAN4 to clarify the requirement of satellite switching without PCI change is not applied for regenerative mode of Rel-19 NTN

Besides, as we copy below, there is an LS on common TA sent from RAN2#127[2], which is mainly about whether negative values for ta-Common should be introduced in a regenerative payload scenario.

LS on common TA in a regenerative payload scenario (R2-2407623) # 1. Overall Description: In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0. # 2. Actions: # To RAN1 and RAN4: # RAN2 kindly requests RAN1 and RAN4 to provide feedback on above question.

In the case of regenerative payload, the value of common TA is typically deemed as zero. As for the question raised by RAN2 to consider introducing negative values for ta-Common, the LS doesn’t further clarify the motivation of introducing negative values. Based on some analysis provided in RAN2 wording group, the main reason is that, in NTN scenario, UE needs to compensate a part of TA, which is implemented by GNSS capability and calculation of ephemeris information. Considering there may be some error caused by the outdated GNSS position, UE may both overestimate and underestimate its TA. The introduction of the negative value of comment TA can help adjust TA more flexible.

For the above consideration, our understanding is that the total error from UE side, including both GNSS error and calculation error of satellite position, has already been considered in the timing requirement. From UE perspective, even if the error occurs, the requirement requires that the UE initial transmission timing error is within Te_NTN range. From this point, this is no different from the transparent payload as specified in Rel-17 and Rel-18. Thus, we don’t see strong motivation to introduce negative value of common TA specifically for regenerative payload. We prefer to stick to 0 as the minimum possible value for TA-Common and not to introduce negative value of common TA.

Proposal 5: For regenerative payload, RAN4 to conclude that stick to 0 as the minimum possible value for TA-Common and not introduce negative value of common TA

- Summary

Proposal 1: RAN4 to wait for more progress from other WGs on SSB periodicity extension to discuss the RRM impacts of DL coverage enhancement.

Proposal 2: RAN4 to wait for more conclusions from RAN1/2 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

Proposal 3: The existing RRM requirements for inactive state, RRC re-establishment and RACH-less handover can be reused for regenerative payload.

Observation 1: The discussion on supporting of satellite switching without PCI change with regenerative payload is out of Rel-19 scope.

Proposal 4: RAN4 to clarify the requirement of satellite switching without PCI change is not applied for regenerative mode of Rel-19 NTN

Proposal 5: For regenerative payload, RAN4 to conclude that stick to 0 as the minimum possible value for TA-Common and not introduce negative value of common TA

- References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R2-2407623, LS on common TA in a regenerative payload scenario, CMCC