3GPP TSG RAN WG4 Meeting #112bis	R4-2415149

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.26.4.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 initially analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress were summarized in WF [1]. As per the arrangement on agenda, in this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - Support of (e)RedCap UEs with NR FR1-NTN

#### Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN Online agreement <Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps <Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

In the last meeting, RAN4 initiated general discussion on defining RRM requirements in case of (e)Red-cap NTN. The requirements agreed to/NOT to define and the requirements to be further determined are shown above. Next, we would like to further analyze each of them in detail.

- 2.1.1	The requirements defined for both NTN and Red-cap

2.1.1.1	Agreed requirements

- Cell Re-selection for RRC_IDLE state mobility

- Cell Re-selection for RRC_INACTIVE state mobility

Regarding the cell re-selection requirements on NTN scenario, time-based and location-based cell reselection measurement criteria were introduced. For NTN-capable UE, these two are generic criteria for both redcap and non-redcap equipment. From this point, both of them shall be supported in Redcap NTN.

Specifically, the current cell reselection measurement delay requirements for NTN include normal requirements and enhanced requirements. For 2Rx, both of these two requirements can be the baseline when defining corresponding requirements for Red-cap NTN. For 1Rx, more evaluation may be needed on whether the measurement accuracy can be satisfied when the current enhanced cell reselection requirement for NTN LEO is directly applied.

Moreover, regarding the relaxation on cell reselection measurement, it was discussed in both NTN and Red-cap work items. Compared with NTN, the stationary case is additionally considered in Red Cap. From our point of view, it can be considered for Red-cap NTN as well. Thus, the existing relaxed measurement criteria defined in Red-cap can be applied for Red-cap NTN UE and the corresponding measurement requirements for Red-cap can be the baseline.

In addition to NTN-NTN mobility, the cases of NTN-TN and TN-NTN mobility were introduced in Rel-18 NTN WI. For these two cases, our understanding is that, for Red-cap NTN UE, both handheld and IOT equipment are mentioned in WID. Thus, the related mobility needs to be considered. From this point of view, we slightly prefer to support these two scenarios in Red-cap NTN and define corresponding requirements. The current requirements for NTN can be reused.

Proposal 1: For cell reselection requirement on Red-cap NTN, RAN4 to agree

- Time-based and location-based cell reselection measurement criteria are supported in Red-cap NTN

- The existing cell reselection measurement requirements for NTN can be the baseline for defining corresponding requirements for Red-cap NTN

  - FFS on whether enhanced cell reselection requirement for NTN LEO can be directly applied for 1Rx

  - Specifically, in the case that UE is configured with relaxed measurement criterion, the existing measurement criteria for Redcap would be applied. The corresponding requirements for Red-cap can be the baseline

- To Define requirements on NTN-TN and TN-NTN cell reselection for Red-cap NTN and the corresponding requirements defined in NTN can be reused.

- NR Handover

For handover, generally, the delay requirements defined for NTN and Rep-cap are quite similar if NCD-SBB is not considered. RAN4 can reuse HO on NTN requirement for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified

For RedCap UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

Proposal 2: RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified,

- For RedCap NTN UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

- RRC Connection Mobility Control

For RRC Connection Mobility Control, it includes three parts shown as follows:

- SA: RRC Re-establishment

- Random access

- SA: RRC Connection Release with Redirection

On the requirements on RRC Re-establishment, for Red-cap, the requirements for 2Rx totally reuse the requirements in clause 6.2.1. Based on this, the requirements for 1Rx are defined with 1 sample relaxation. For NTN, it is inherited from legacy requirements in clause 6.2.1 with minor adaptation. Therefore, as for Red-cap NTN, it is straightforward that the existing RRC Re-establishment requirements for NTN can be reused for 2Rx. For 1Rx, the existing RRC Re-establishment requirements for Red-cap can be reused with necessary modifications to adapt the NTN scenario.

For the requirements on Random access and RRC Connection Release with Redirection, the situations are similar to the above. Thus, we have the following proposal after summarizing these three requirements:

Proposal 3: For RRC Connection Mobility Control requirements for Red-cap NTN, including RRC Re-establishment, Random access and RRC Connection Release with Redirection, RAN4 to agree

- For 2Rx, the existing RRC Connection Mobility Control requirements for NTN can be reused

- For 1Rx, the existing RRC Connection Mobility Control requirements for Red-cap can be reused with necessary modification to adapt NTN scenario

- Timing

- UE transmit timing

- UE timer accuracy

- Timing advance

For UE timer accuracy, the corresponding requirements defined in NTN and Red-cap are the same. For UE transmit timing requirements and timing advance requirements, in NTN, the NTN-specific factors are additionally taken into account. As a Red-cap NTN UE, the same principles should be followed. To sum up, the timing requirements for NTN can be reused to define the corresponding requirements for Red-cap NTN.

Proposal 4: For timing requirement for Red-cap NTN, RAN4 to reuse the existing timing requirements for NTN

- Signaling characteristics

- Radio Link Monitoring

- Link Recovery Procedures

- Active BWP switch delay

For the requirements on Radio Link Monitoring, for 2Rx, the existing requirements for NTN can be reused. And for 1Rx the current parameters and requirements for 1Rx defined in Red-cap can be the reference and make necessary modifications to adapt NTN scenario. Besides, considering for 60kHz SCS in FR1, RAN4 didn’t introduce requirements for Red-cap RLM requirement. Moreover, there are some of the applicable rules defined for HD-FDD case in Red-cap, we think the same rules need to be adopted in Red-cap NTN as well.

For the Link Recovery Procedures, a similar analysis in RLM can be referred.

For Active BWP switch delay, the main difference between NTN and Red-cap is that the NR slot length 0.125ms is additionally considered in Red-cap. For Red-cap NTN, we considered the minimum applicable sets of these two features. Thus, the current requirements on Active BWP switch delay for NTN can be reused.

Proposal 5: For requirement on Radio Link Monitoring for Red-cap NTN, RAN4 to agree

- For 2Rx, the existing Radio Link Monitoring requirements for NTN can be reused

- For 1Rx, the existing Radio Link Monitoring requirements for Red-cap can be used as the baseline and make necessary modifications to adapt NTN scenario

- Additionally, add the applicable rules for HD-FDD case as specified in Red-cap

- SCS = 60kHz is not applicable

Proposal 6: For requirement on Link Recovery Procedures for Red-cap NTN, RAN4 to agree

- For 2Rx, the existing requirements on Link Recovery Procedures for NTN can be reused

- For 1Rx, the existing requirements on Link Recovery Procedures for Red-cap can be used as the baseline and make necessary modification to adapt NTN scenario

Proposal 7: For Active BWP switch delay requirement for Red-cap NTN, RAN4 to reuse the existing Active BWP switch delay requirements for NTN

- Measurement Procedure

- General measurement requirement

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

For General measurement requirements, it mainly focuses on the measurement gap. For Red-cap, both per-UE gap and per-FR gap are supported. While for NTN, only per-UE gap is supported. For Red-cap NTN, it is straightforward to only consider per-UE gap. Specifically, for the gap pattern, the current gap patterns (i.e., 0-11, 24, 25) defined for NTN need to be supported. Besides, in NTN, the Concurrent gap is introduced. This capability can be supported for Red-cap NTN UE as well. Based on the above analysis, we think the current General measurement requirement For NTN can be reused for 2RX. And for 1Rx, the current 1Rx requirements defined in Red-cap can be reused.

For NR intra-frequency measurements, NR inter-frequency measurements and L1-RSRP measurements for Reporting, the corresponding requirements defined in NTN can be reused.

Proposal 8: For Measurement Procedure requirement for Red-cap NTN, for 2Rx, RAN4 to reuse the existing Measurement Procedure requirements for NTN. For 1Rx, the corresponding 1Rx requirements defined in Red-cap can be reused.

2.1.1.2	Requirements to be further determined

- NR Rel-17/18 Conditional Handover, and RACH-less Handover

For CHO and RACH-less handover, regarding the following scenarios and further enhancements introduced in Rel-17 and Rel-18 NTN, they are not supported in Redcap so far. In our view, considering mobility management is one of the most important features in NTN, we prefer to support this part as the essential feature of Redcap NTN.

- NR RACH-less HO in Rel-18

- Conditional handover in Rel-17(including time-based and location-based CHO for fixed cell)

- Enhancements on CHO in Rel-18

  - CHO without L3 measurement criteria

  - Support earth moving cell scenario

Proposal 9: RAN4 to define the following mobility requirements in connected mode. The corresponding requirements defined for NTN can be the baseline

- NR RACH-less HO

- Conditional handover (including time-based and location-based CHO for fixed cell)

- Enhancements on CHO

  - CHO without L3 measurement criteria

  - Support earth moving cell scenario

- 2.1.2	The requirements that have only been defined for NTN

- NR SAN Satellite switching with re-synchronization

For satellite switch with unchanged PCI, basically, the current requirements defined in NTN can be reused. One minor change to be considered is, in current HO requirements for Red-cap, the applicability rules for HD-FDD case are defined in current HO requirements for Red-cap. Considering the current requirements on Satellite switching with re-synchronization is inherited from the HO requirement, the same principles shall be adopted in Satellite switching with re-synchronization as well.

•	For RedCap UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

Proposal 10: For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case:

•	For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

- Pathloss reference signal switching delay

For the requirements on Pathloss reference signal switching delay, the corresponding requirements for NTN can be reused.

Proposal 11: For requirements on Pathloss reference signal switching delay for Red-cap NTN, RAN4 to reuse Pathloss reference signal switching delay requirements for NTN

- Network verified UE location

For Network verified UE location, the corresponding requirements for NTN were discussed in Rel-18. Basically, the measurement period and measurement accuracy requirements can be reused. Specifically, the requirement to be newly introduced is UE Rx-Tx accuracy requirement for 1Rx. For this, a similar discussion occurred in Red-cap positioning. The corresponding accuracy can be referred.

Proposal 12: For Network verified UE location, the measurement period and measurement accuracy requirements for NTN can be reused. Additionally, regarding the measurement accuracy requirements for 1Rx, the measurement accuracy requirements for 1Rx defined in Red-cap positioning can be referred.

    - eDRX enhancement

#### Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

For eDRX, our preference is to support eDRX configuration in Redcap NTN. For NTN, for earth-moving LEO deployment, considering the UE is not required to meet the requirements for 2.56s DRX cycle length. We prefer to add the corresponding applicability rules in the spec. RAN4 can firstly agree that for GEO deployment, all DRX and eDRX cycle can be supported and further discuss on the value of eDRX cycle to be supported, respectively for earth-moving and earth-fixed LEO deployment.

Proposal 13: RAN4 to support eDRX configuration in Redcap NTN and to discuss the applicable rules:

- For GEO deployment, all DRX and eDRX cycle can be supported.

- For LEO deployment, FFS on the eDRX cycle to be supported, respectively for earth-moving and earth-fixed LEO deployment

- Summary

Proposal 1: For cell reselection requirement on Red-cap NTN, RAN4 to agree

- Time-based and location-based cell reselection measurement criteria are supported in Red-cap NTN

- The existing cell reselection measurement requirements for NTN can be the baseline for defining corresponding requirements for Red-cap NTN

  - FFS on whether enhanced cell reselection requirement for NTN LEO can be directly applied for 1Rx

  - Specifically, in the case that UE is configured with relaxed measurement criterion, the existing measurement criteria for Redcap would be applied. The corresponding requirements for Red-cap can be the baseline

- To Define requirements on NTN-TN and TN-NTN cell reselection for Red-cap NTN and the corresponding requirements defined in NTN can be reused.

Proposal 2: RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case as current HO requirements for Redcap specified,

- For RedCap NTN UE with HD-FDD, the handover requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

Proposal 3: For RRC Connection Mobility Control requirements for Red-cap NTN, including RRC Re-establishment, Random access and RRC Connection Release with Redirection, RAN4 to agree

- For 2Rx, the existing RRC Connection Mobility Control requirements for NTN can be reused

- For 1Rx, the existing RRC Connection Mobility Control requirements for Red-cap can be reused with necessary modification to adapt NTN scenario

Proposal 4: For timing requirement for Red-cap NTN, RAN4 to reuse the existing timing requirements for NTN

Proposal 5: For requirement on Radio Link Monitoring for Red-cap NTN, RAN4 to agree

- For 2Rx, the existing Radio Link Monitoring requirements for NTN can be reused

- For 1Rx, the existing Radio Link Monitoring requirements for Red-cap can be used as the baseline and make necessary modifications to adapt NTN scenario

- Additionally, add the applicable rules for HD-FDD case as specified in Red-cap

- SCS = 60kHz is not applicable

Proposal 6: For requirement on Link Recovery Procedures for Red-cap NTN, RAN4 to agree

- For 2Rx, the existing requirements on Link Recovery Procedures for NTN can be reused

- For 1Rx, the existing requirements on Link Recovery Procedures for Red-cap can be used as the baseline and make necessary modification to adapt NTN scenario

Proposal 7: For Active BWP switch delay requirement for Red-cap NTN, RAN4 to reuse the existing Active BWP switch delay requirements for NTN

Proposal 8: For Measurement Procedure requirement for Red-cap NTN, for 2Rx, RAN4 to reuse the existing Measurement Procedure requirements for NTN. For 1Rx, the corresponding 1Rx requirements defined in Red-cap can be reused.

Proposal 9: RAN4 to define the following mobility requirements in connected mode. The corresponding requirements defined for NTN can be the baseline

- NR RACH-less HO

- Conditional handover (including time-based and location-based CHO for fixed cell)

- Enhancements on CHO

  - CHO without L3 measurement criteria

  - Support earth moving cell scenario

Proposal 10: For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case:

•	For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

-	SSB is available at the UE once every SMTC period during Tsearch

-	One SSB is available during T∆

-	One SSB is available during TIU.

Proposal 11: For requirements on Pathloss reference signal switching delay for Red-cap NTN, RAN4 to reuse Pathloss reference signal switching delay requirements for NTN

Proposal 12: For Network verified UE location, the measurement period and measurement accuracy requirements for NTN can be reused. Additionally, regarding the measurement accuracy requirements for 1Rx, the measurement accuracy requirements for 1Rx defined in Red-cap positioning can be referred.

Proposal 13: RAN4 to support eDRX configuration in Redcap NTN and to discuss the applicable rules:

- For GEO deployment, all DRX and eDRX cycle can be supported.

- For LEO deployment, FFS on the eDRX cycle to be supported, respectively for earth-moving and earth-fixed LEO deployment

- References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT