3GPP TSG RAN WG4 Meeting #112bis	R4-2415150

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.15.3

Source:	vivo

Title:	Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

For the objective of fast SCell activation, as per the working arrangement, the corresponding work was triggered in the last meeting. In more details, the discussion mainly focused on scope of the fast SCell activation delay requirements and general principle on how to define delay requirements with valid eEMR reporting. In this contribution, we further provide our views based on the latest progress captured in WF [1].

- Discussion

Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

For the case on FFS part, according to the description specified in the spec, when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, UE is not required to perform validity check. As per the WID description, the objective aims to enhance the SCell activation delay based on ‘valid eEMR reporting’. In our understanding, the measurement results without validity checking are not considered as ‘valid’ results to some extent. On the other hand, from SCell activation perspective, the reported results may have been obtained a long time ago and the quality of SCell to be activated cannot be guaranteed.

Clause 4.7.3 in TS38.133 If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

Proposal 1: RAN4 not to define the fast SCell activation delay requirements for the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

For the scope of fast SCell activation for UE supporting Rel-18 EMR, for the case of Direct SCell Activation at SCell addition, it is the most typical use case for supporting ast SCell activation with eEMR reporting. With only two meetings left, we prefer to focus on this scenario for discussion and to further check if the solutions can be applied for the FFS scenarios together with other SCell activation after RAN4 has conclusions for the first case.

Proposal 2: RAN4 to start from the case of Direct SCell Activation at SCell addition for fast SCell activation for UE supporting Rel-18 EMR.

Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

For the FFS on the applicability of valid EMR reporting for the Rel-19 fast SCell activation, as we mentioned in Issue 1-1-2, currently RAN4 has clear definition of ‘valid’ measurement result. For the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured, the measurement results cannot be regarded as the ‘valid’ results even the accuracy requirement is satisfied.

On how to update the known condition, the basic solutions proposed by companies mainly include:

- Option 1	SCell is known when the UE has sent a valid measurement report of the SCell being activated during IDLE/INACTIVE state for fast CA/DC setup

- Option 2	SCell is known when valid measurement report is sent within [Y] seconds before SCell activation command reception

For Option 1, it means if the results pass the validity check or new check, the SCell to-be-activated can be regarded as the known cell. For the measurement validation in Rel-18 EMR, it mainly means that UE will verify if the measurements are performed before Msg1 transmission within the last [measIdleValidityDuration or measReselectionValidityDuration] seconds and the results satisfy measurement accuracy requirement at the measurement instance. The duration of validity timer is indicated by NW and the candidate values are {5s,10s,20s,50s,100s}, which are shown below.

MeasurementValidityDuration-r18 ::= ENUMERATED { s5, s10, s20, s50, s100, spare3, spare2, spare1

When validity timer is configured such as 50s, or 100s, the measurement results may be out of date for SCell activation. If we go with Option 1, it may define an applicability rule, that is only workable when validity window is short.

Therefore, we prefer to introduce a new ‘known’ condition window where that valid measurement report is sent within the specific time gap before SCell activation command reception. Accordingly, the known condition can be updated (take FR1 as an example):

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to Y second for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for fast CA/DC setup which includes the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to Y seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

Proposal 3: For the fast SCell activation delay requirements with valid eEMR reporting, RAN4 to update the known condition that the valid measurement report is sent within [Y] seconds before SCell activation command reception

- Summary

Proposal 1: RAN4 not to define the fast SCell activation delay requirements for the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

Proposal 2: RAN4 to start from the case of Direct SCell Activation at SCell addition for fast SCell activation for UE supporting Rel-18 EMR.

Proposal 3: For the fast SCell activation delay requirements with valid eEMR reporting, RAN4 to update known condition that the valid measurement report is sent within [Y] seconds before SCell activation command reception

- References

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT