3GPP TSG-RAN WG4 Meeting #112bis	R4-2415152

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.8.4

Source:	MediaTek Inc.

Title:	Discussion on NTN testing for NGSO

Document for:	Discussion

# Introduction

In RAN4#112 meeting, a WF for NGSO testing was agreed [1]. In this contribution, we provide our views on outstanding issues.

# Discussion

There were lots of options about TE emulated channel model for satellite mobility proposed in the last meeting [1].

  - Option 1: Eckstein-Hechler model

  - Option 2: Keplerian model

  - Option 3: Existing model from TR 38.811 5.3.4.3

  - Option 4: Proposal 2 from R4-2412552

  - Option 5: Proposal 2 from R4-2412783

For Option 1 and 2, we think it is difficult to obtain closed forms about time function of Doppler and time delay variation. For Option 4, the model is too simplified as the movement of satellite is assumed to be in a straight line.

Observation 1: It is difficult to obtain a closed from about time function of Doppler and time delay variation with Eckstein-Hechler model and Keplerian model.

Observation 2: The model in R4-2412552 model is too simplified as the movement of satellite is assumed to be in a straight line.

RAN4 agreed to use predefined files generated by GMAT in [2] as a reference for the comparison purpose of proposed channel model.

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters

Observation 3: RAN4 agreed to use the predefined files generated by GMAT in R4-2413333 as a reference for the comparison purpose of proposed channel model.

As RAN4 agreed to use the predefined files generated by GMAT in R4-2413333 as a reference, we propose to use polynomial curve fitting to obtain the time function of Doppler, i.e.,

fDt=c0tn+c1tn-1+…+cn

where fDt is Doppler, t is time sample, and cn is polynomial coefficient. Similarly, polynomial curve fitting can also be used to obtain the time function of delay, i.e.,

fTt=d0tn+d1tn-1+…+dn

where fTt is delay, t is time sample, and dn is polynomial coefficient.

In Figure 1 and 2, we provide curves with polynomial fitting for Doppler and Delay in different order. It shows that the curves with polynomial fitting for Doppler and Delay are close to the predefined files generated by GMAP when polynomial order is high enough. Therefore, we propose to use polynomial fitting method to generate time function of Doppler and delay for TE emulated channel model.

Figure 1. Polynomial curve fitting for Doppler for whole duration of observation time

Figure 2. Polynomial curve fitting for Delay for whole duration of observation time

It is also possible to use a smaller order to obtain accurate polynomial curve fitting when considering some portion of observation time.

Figure 3. Polynomial curve fitting for Doppler for different duration of observation time

The testing time

Observation 4: The curves with polynomial fitting for Doppler and Delay are close to the predefined files generated by GMAP when polynomial order is high enough if considering the whole duration of observation time.

Observation 5: The curves with polynomial fitting for Doppler and Delay are close to the predefined files generated by GMAP with small order, e.g., 3rd order, if considering some portion of observation time to define the requirements.

Proposal 1: To generate time domain function of Doppler and delay variation (closed from) by using the method of polynomial curve fitting based on the predefined files from GMAT.

Issue 1-2-2 Parameters for TE-emulated channel model - Agreement - RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose: - Satellite altitude: - Option 1: LEO -600km only - Option 2: Both LEO-600km and LEO-1200km Elevation angle range - UE position - Whether to consider a non-zero time-varying feeder link delay - Other parameters not precluded

For the necessary parameters with associated channel model, we propose to assume UE as stationary and configured with “Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0”, which is the assumption to derive the predefined files by GMAT. As for satellite altitude, we propose to consider LEO -600km only.

Observation 6: The predefined files generated by GMAT are based on stationary UE with “Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0”.

Proposal 2: Consider stationary UE with “Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0”.

Proposal 3: Consider LEO -600km only.

Issue 1-3-2 demodulation requirements - Agreement - RAN4 further discuss how to apply the varying Doppler and delay shifts to the UE performance requirements and necessary simulation assumption on UE side pending on channel model discussion progress. - UE assumption on propagator model for satellite motion for RAN4 simulation purpose - Option 1: Keplerian model - other options not precluded - RAN4 further discuss test applicability if new PDSCH demodulation requirements introduced pending on channel model discussion progress.

Regarding the assumption on propagator model for satellite motion at the UE side, we propose to evaluate the performance based on the agreed TE emulated channel model first. Generally, there is no assumptions on the UE implementation when deriving the requirements. If the simulation results from companies based on the agreed TE emulated channel model are within some span. Then it is not necessary to have assumptions on the propagator model for satellite motion at the UE side.

Proposal 4: We propose to evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

# Conclusion

In this contribution, we provide our views on NTN testing for NGSO. Observations and proposals are summarized as below.

Observation 1: It is difficult to obtain a closed from about time function of Doppler and time delay variation with Eckstein-Hechler model and Keplerian model.

Observation 2: The model in R4-2412552 model is too simplified as the movement of satellite is assumed to be in a straight line.

Observation 3: RAN4 agreed to use the predefined files generated by GMAT in R4-2413333 as a reference for the comparison purpose of proposed channel model.

Observation 4: The curves with polynomial fitting for Doppler and Delay are close to the predefined files generated by GMAP when polynomial order is high enough if considering the whole duration of observation time.

Observation 5: The curves with polynomial fitting for Doppler and Delay are close to the predefined files generated by GMAP with small order, e.g., 3rd order, if considering some portion of observation time to define the requirements.

Proposal 1: To generate time domain function of Doppler and delay variation (closed from) by using the method of polynomial curve fitting based on the predefined files from GMAT.

Observation 6: The predefined files generated by GMAT are based on stationary UE with “Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0”.

Proposal 2: Consider stationary UE with “Latitude: 25.08439333, Longitude: 121.56076999, Altitude: 0”.

Proposal 3: Consider LEO -600km only.

Proposal 4: We propose to evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

# Reference

- R4-2413516, Way Forward for [112][329] NTN_testing_NGSO_channel_model, Samsung

- R4-2413333, Discussion on dynamic NTN testing for NGSO, THALES