3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415153

# Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on less than 5MHz for NTN RRM requirement

Source: 	CATT

Agenda item:	6.8.3.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved in RAN#103 [1], and the WID have been revised in RAN#104 and RAN#105.The latest objectives of core part WI for less than 5MHz for NTN are as following [2]:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

This document will discuss the general RRM issues about less than 5MHz for NTN, and give our understanding and proposals.

- Discussion

In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID ‘NR support for dedicated spectrum less than 5 MHz for FR1’ by extending support for NTN.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. Therefore, as a baseline, RAN4 should define the related RRM requirement based on NTN requirements. This objective is to extend NR-NTN by considering support for dedicated spectrum less than 5 MHz for FR.

Handover

In the last meeting, RAN4 discussed handover related requirements and agreed to take extension of delay for TN less than 5MHz as baseline. The agreements are copied as follows [3]:

Agreement: Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements.

Regarding HO delay requirements for NTN less than 5MHz, RAN4 needs to take extension on Tsearch as TN with less than 5MHz. For the details of NTN specific HO requirements with less than 5MHz, we think that the existing NTN HO requirements can be reused as baseline, including time/location-based CHO delay requirement and satellite switch time requirement.

Proposal 1: Regarding HO delay requirements for NTN less than 5MHz, RAN4 needs to take extension on Tsearch as TN with less than 5MHz.

Proposal 2: The existing NTN specific HO requirements can be reused as baseline, including time/location-based CHO delay requirement and satellite switch time requirement.

For time/location-based CHO delay requirement, at least the measurement time in DCHO will be affected by less than 5MHz. Since Tmeasure is related to Tidentify_intra/inter_with_index or Tidentify_ intra/inter_without_index, it will be related to the time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB. Therefore, the measurement time in DCHO should also consider the impact of the target cell with 12 PRB SSB.

Proposal 3: For time/location-based CHO delay requirement, at least the measurement time in DCHO should consider the impact of the target cell with 12 PRB SSB.

For satellite switch time requirements, we believe that RAN4 also need to take extension of delay on Tsearch in interruption time for hard satellite switch with re-sync and satellite switch delay for soft satellite switch with re-sync because of less than 5MHz.

Proposal 4: For satellite switch time requirements, RAN4 need to take extension of delay on Tsearch in interruption time for hard satellite switch with re-sync and satellite switch delay for soft satellite switch with re-sync because of less than 5MHz.

Timing

      - Whether timing requirements for NTN in current TS38.133 will be impacted becasue of less than 5MHz bandwidth

The WF on whether timing requirements for NTN in current TS 38.133 will be impacted because of less than 5MHz bandwidth is copied as follows [3]:

WF: FFS on - Option 1: - No impacts

In Rel-18 less than 5MHz WI, there was no impact on timing requirements becasue of less than 5MHz bandwidth in TN network according to current TS 38.133 [4], so we think that the same consideration for NTN network is reasonable.

Observation 1: There was no impact on timing requirements because of less than 5MHz bandwidth in TN network.

      - Proposal 5: There is no impact on timing requirements for NTN because of less than 5MHz bandwidth.

Signalling characteristics

      - RLM/BFD/CBD

The WF on RLM/BFD/CBD is copied as follows [3]:

WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: - The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario

In Radio Link Monitoring requirements, the applicable scenarios should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

For SSB based RLM/BFD requirements, we believe that the same principle can be followed as TN with less than 5MHz, RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation and beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN. Since the PDCCH transmission parameters defined in NTN are the same as TN with less than 5MHz [4], so we support to reuse the PDCCH transmission parameters for out-of-sync/ in-sync evaluation and beam failure instance for a UE operating on a cell with less than 5MHz BW in TN for FR1-NTN.

Proposal 6: The applicable scenarios for RLM requirements should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

Observation 2: The PDCCH transmission parameters for out-of-sync/ in-sync evaluation  and beam failure instance defined in NTN are the same as TN with less than 5MHz.

Proposal 7: For SSB based RLM requirements, RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

- The parameters defined in TN with less than 5MHz can be reused.

Proposal 8: For SSB based BFD requirements, RAN4 need to define PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

- The parameters defined in TN with less than 5MHz can be reused.

Measurement Procedure

      - Measurement (SSB index detection requirements)

The WF on measurement is copied as follows [3]:

WF: FFS on: - Proposal 1: Whether the time period for time index detection within NTN intra/inter-frequency measurements with/without MG shall be redefined. - Option 1: Yes - Option 2: FFS - Proposal 2a: - The new SSB index detection requirements can be defined by add number of SSB samples as Rel18 TN with less than 5MHz. - Proposal 2b: - RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap

We believe that the same principle can be followed as TN with less than 5MHz. For NR intra/inter-frequency measurements without and with measurement gaps for SAN, the time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined.

For how to define new SSB index detection requirements for a target cell with 12 PRB SSB for NTN, firstly, the current NTN requirements should be a baseline, it means that NTN related Klayer1_measurement , Kmulti_SMTC and Kgap will be considered and then RAN4 need to consider how to revise the existing SSB index detection requirements defined for NTN. We think that the same principle can be followed as TN with less than 5MHz, that is, by adding number of SSB samples based on the existing NTN requirements.

Proposal 9: The time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined for the following cases:

- NR intra-frequency measurements without measurement gaps for SAN.

- NR intra-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements without measurement gaps for SAN.

      - Proposal 10: The new SSB index detection requirements can be defined based on the existing NTN requirements.

- NTN related Klayer1_measurement , Kmulti_SMTC and Kgap will be considered.

- Add number of SSB samples as Rel-18 TN with less than 5MHz.

      - L1-RSRP Measurement

The WF on L1-RSRP measurement is copied as follows [3]:

WF: FFS on - Proposal 1: - For L1-RSRP measurement based on SSB there is no RRM impacts,

For the requirements defined in TN with less than 5MHz, there was no RRM impact on L1-RSRP measurement because of less than 5MHz. Therefore, we believe that there is no RRM impact on L1-RSRP measurement for NTN with less than 5MHz.

Proposal 11: There is no RRM impact on L1-RSRP measurement for NTN with less than 5MHz.

- Conclusion

This document discussed preliminary the issues for less than 5MHz for NR NTN RRM requirements and presented following observations and proposals:

Proposal 1: Regarding HO delay requirements for NTN less than 5MHz, RAN4 needs to take extension on Tsearch as TN with less than 5MHz.

Proposal 2: The existing NTN specific HO requirements can be used as baseline, including time/location-based CHO delay requirement and satellite switch time requirement.

Proposal 3: For time/location-based CHO delay requirement, at least the measurement time in DCHO should consider the impact of the target cell with 12 PRB SSB.

Proposal 4: For satellite switch time requirements, RAN4 need to take extension of delay on Tsearch in interruption time for hard satellite switch with re-sync and satellite switch delay for soft satellite switch with re-sync because of less than 5MHz.

Observation 1: There was no impact on timing requirements because of less than 5MHz bandwidth in TN network.

      - Proposal 5: There is no impact on timing requirements for NTN because of less than 5MHz bandwidth.

Proposal 6: The applicable scenarios for RLM requirements should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

Observation 2: The PDCCH transmission parameters for out-of-sync/ in-sync evaluation  and beam failure instance defined in NTN are the same as TN with less than 5MHz.

Proposal 7: For SSB based RLM requirements, RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

- The parameters defined in TN with less than 5MHz can be reused.

Proposal 8: For SSB based BFD requirements, RAN4 need to define PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

- The parameters defined in TN with less than 5MHz can be reused.

Proposal 9: The time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined for the following cases:

- NR intra-frequency measurements without measurement gaps for SAN.

- NR intra-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements without measurement gaps for SAN.

      - Proposal 10: The new SSB index detection requirements can be defined based on the existing NTN requirements.

- NTN related Klayer1_measurement , Kmulti_SMTC and Kgap will be considered.

- Add number of SSB samples as Rel-18 TN with less than 5MHz.

Proposal 11: There is no RRM impact on L1-RSRP measurement for NTN with less than 5MHz.

- Reference

[1]	  RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei

[2]	RP-241985, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[3]      R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, Moderator (Xiaomi)

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0