3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415154

Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on enhancements for Rel-19 ATG RRM requirements

Source: 	CATT

Agenda item:	6.10.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Air-to-ground network for NR was approved in RAN#103 [1] and the revised WID was approved in RAN#104 and RAN#105 [2]. The core part objectives of the work item are as following:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

In the Justification of WID, the following clarifications are presented:

Considering the particularity of ATG network deployment, the following aspects should be addressed in Rel-19 ATG work item.

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

This document will discuss the general RRM issues about ATG and give our understanding and proposals.

- Discussion

Sub-topic 1-1 General

Specification structure

The candidate options on specification structure in the last meeting are copied as follows [3]:

- Option 1: The CA related requirements can be directly added in the sections defined for R18 ATG UE - Other Options are not precluded.

R18 ATG RRM requirements have been defined with a new suffix in TS 38.133 [4]. In R19 ATG, just DL CA scenario will be added, so we are fine to add the CA related requirements directly in the current sections defined for R18 ATG UE.

Proposal 1: For the specification structure for R19 ATG in TS 38.133, the CA related requirements can be directly added in the sections defined for R18 ATG UE.

  - Sub-topic 1-2 RRC_IDLE/ RRC_INACTIVE state mobility

Idle/Inactive mode CA measurements (R16)

The requirements of idle/inactive mode CA/DC measurements were specifically introduced for CA/DC scenarios in TN, so these requirements were not introduced in R18 ATG. Considering that R19 ATG will introduce intra-band co-located and inter-band co-located DL CA, the related requirements should be discussed for R19 ATG. The options on R16 Idle/Inactive mode CA measurements are copied as follows [3]:

- Option 1: Introduce idle/inactive mode CA measurements requirements for ATG UE in Rel-19 - Option 2: No need to introduce idle/inactive mode CA measurements requirements in ATG CA.

In our view, idle/inactive mode CA measurements requirements have been introduced in TN network for helping the network to configure SCell.

Even though ATG only consider co-located scenario, we think that network may still benefit from EMR, because in TN network there are no application scenarios for EMR mechanism, and we think that EMR is an optional capability for UE, and whether to use the report depends on NW. If the idle/inactive mode CA measurements requirements are introduced in ATG CA, the related requirements defined in TN can be reused as baseline.

Observation 1: In TN network there are no application scenarios for EMR mechanism, and we think that EMR is an optional capability for UE, and whether to use the report depends on NW.

Proposal 2: If the idle/inactive mode CA measurements requirements are introduced in ATG CA,

- EMR is an optional capability for ATG UE, and whether to use the report is up to NW.

- The related requirements defined in TN can be reused as baseline.

However, considering that the network may have a little chance to go into the idle/inactive mode, from this point of view, it is also acceptable for us to lower the priority of idle/inactive mode CA measurement requirements in the ATG CA.

Observation 2: The network may have a little chance to go into the idle/inactive mode in ATG scenario.

Proposal 3: Deprioritize to introduce idle/inactive mode CA measurements requirements in ATG CA.

Measurement report for fast CA/DC setup (R18)

In Rel-18 mobility WI, measurement report for fast CA/DC setup in idle/inactive mode has been introduced, and RAN4 need to consider whether to introduce Rel-18 measurement report for fast CA/DC setup in ATG CA. The options on R16 Idle/Inactive mode CA measurements are copied as follows [3]:

- Option 1: RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline - Option 2: No need to introduce Rel-18 NR CA EMR in ATG CA. - Option 3: Consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE

In our view, if R16 idle/inactive mode CA measurements requirements are not introduced in ATG CA, R18 enhanced EMR will also not be introduced in ATG CA. Even though R16 idle/inactive mode CA measurements requirements are not introduced in ATG CA, we prefer not to enhance the requirements for EMR in idle/inactive mode, as the network may have a little chance to go into the idle/inactive mode.

Proposal 4: Prefer not to further introduce Rel-18 enhanced EMR requirements in idle/inactive mode for ATG CA.

  - Sub-topic 1-3 Timing

Due to the requirements of Maximum Transmission Timing Difference and Maximum Receive Timing Difference are only defined for the scenarios of CA and DC, they were not defined in R18 ATG. Considering that R19 ATG will introduce intra-band co-located and inter-band co-located DL CA, the FR1 related requirements of MTTD and MRTD for Carrier Aggregation (clauses 7.5.4 and 7.6.4 in TS 38.133[3]) should be discussed.

MTTD

In the last meeting, RAN4 discussed whether MTTD requirement is needed or not, but there was no consensus. The WF is copied as follows [3]:

- Option 1: FFS whether MTTD requirement is needed or not

From our understanding, MTTD requirement in CA is only specified for inter-band CA and intra-band non-contiguous CA for non-collocated scenario, and they are to ensure maximum uplink transmission timing difference in either UL carriers [4]. However, only DL CA scenario is considered for R19 ATG, so we believe that there is no need to define MTTD requirements in R19 ATG CA.

Proposal 5: There is no need to define MTTD requirements in R19 ATG CA.

MRTD

In the last meeting, RAN4 discussed whether to define the MRTD requirements and agreed to not define MRTD requirements for ATG intra-band contiguous CA. For ATG FR1 inter-band CA, there was still controversy about the value of the maximum receive timing difference, the WF is copied as below [3]:

Agreement: - For ATG intra-band contiguous CA, not define MRTD requirement. - For ATG inter-band CA, MRTD requirement is: - Option 1: A little larger than 3us, i.e., 3.545 µs - Option 2: 33us - Option 3: 3us - Option 4: further discuss

For inter-band CA, 33us was defined for MRTD requirements in FR1 CA, and it included TAE (3us) and the propagation delay (30us) in FR1. For ATG FR1 inter-band CA, since only co-location scenario is considered, so the propagation delay can be 0. In addition, considering that ATG UE has stronger capabilities, we also can consider a little relaxation on 3us. But for whether it can be extend to 33us, we think that 33us may be too long.

Proposal 6: For ATG inter-band CA, MRTD requirement is a little larger than 3us in FR1.

  - Sub-topic 1-4 Signalling characteristics

Interruption requirements

Due to interruption requirements are only defined for the scenarios of CA and DC, they were not defined in R18 ATG. Since CA is considered in R19 ATG, the existing interruption requirements can also apply to ATG supporting CA as a baseline, RAN4 could further discuss the specific requirements and consider ATG related features.

Proposal 7: The existing interruption requirements can also apply to R19 ATG CA as a baseline, RAN4 could further discuss the specific requirements and consider ATG related features.

SCell Activation and Deactivation Delay

The candidate proposals on SCell activation and deactivation delay in the last meeting are copied as follows [3]:

- Proposal 1: SCell Activation and Deactivation Delay requirements should be introduced for ATG - Proposal 1-1: SCell activation requirements shall be further investigated for potential impact, taking into account the band combinations stated in the WID and the antenna types agreed in RF - Proposal 1-2: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and scellWithoutSSB capability can be considered. - Proposal 1-3: Introduce following features for ATG, reuse the current FR1 requirements as the starting point, and further study whether further improvement is needed or not due to co-located deployment: - SCell Activation Delay Requirement for Deactivated SCell - SCell Deactivation Delay Requirement for Activated SCell - Direct SCell Activation at SCell addition - Direct SCell Activation at Handover - Direct SCell Activation at RRC Resume - Fast SCell Activation Delay Requirement for Deactivated SCell - SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. - Proposal 1-4: RAN4 to define SCell activation for single-SCell for at least: - SCell activation - Direct SCell activation at SCell addition - Direct SCell activation at Handover - Proposal 1-5: Due to co-located deployment, similar beam coverage shared between multiple serving cells, the component of cell search can be ignored, whether L1 meas&report can be ignored, which can be discussed.

The SCell activation and deactivation delay requirements were defined for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC in TN network (clause 8.3 in TS 38.133 [4]). For R19 ATG CA, RAN4 need to introduce the related requirements at least for the UE configured with single downlink SCell in standalone NR carrier aggregation, and the FR1 related requirements can be reused as the starting point. The related requirements at least include the following:

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRC Resume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 8: The SCell activation and deactivation delay requirements should be defined at least for the ATG UE configured with single downlink SCell in standalone NR CA, and the FR1 related requirements can be reused as the starting point, which at least include the following:

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRC Resume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Active BWP switch delay on multiple CCs

In the R18 ATG, the requirements of DCI and timer based BWP switch delay and RRC based BWP switch delay only apply to the case that the BWP switch is performed on the PCell with more than one BWP configurations configured. Since DL CA scenario has been considered for R19 ATG UE, so the DCI based, timer based and RRC based BWP switch delay on multiple CCs should be defined. The candidate proposals on active BWP switch delay on multiple CCs in the last meeting are copied as follows [3]:

- Proposal 1: For Active BWP switch delay on multiple CCs, introduce following requirements, which can reuse the legacy: - Simultaneous DCI based BWP switch delay on multiple CCs - Simultaneous RRC based BWP switch delay on multiple CCs - Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Since non-simultaneous DCI based and RRC based BWP switch delay on multiple CCs is defined for NR-DC and it is not in the scope, so only simultaneous DCI based and RRC based BWP switch delay on multiple CCs need to be considered, and the legacy requirements can be reused as baseline. For timer based BWP switch delay on multiple CCs, both simultaneous and non-simultaneous related requirements should be defined.

Proposal 9: For active BWP switch delay on multiple CCs requirements for ATG UE, introduce the following requirements and the legacy can be reused as baseline:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

The candidate proposals on pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation in the last meeting are copied as follows [3]:

- Proposal 1: Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

The existing requirements of pre-configured measurement gap activation/deactivation delay only apply for an ATG UE configured with PCell in standalone NR. Considering that CA is introduced in R19 ATG, we support to introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, and the legacy requirement can be reused.

Proposal 10: Introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

  - Sub-topic 1-5 Measurement procedure

Measurement gap

The candidate proposals on measurement gap in the last meeting are copied as follows [3]:

- Proposal 1: RAN4 to reuse NR carrier aggregation measurement gap as the baseline for ATG DL CA. - Proposal 2: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration. - Proposal 3: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

The existing measurement gap only defined for SA single carrier in R18 ATG, since only FR1 CA is considered in R19 ATG CA, so we believe that the measurement gap applicability rule for SA single carrier shall be also applied for NR CA configuration. In addition, for SCC interruption time, since only co-location is considered in R19 ATG, there is no need to consider the case of CA with non-aligned frame boundaries, and only aligned frame boundaries scenario is reasonable.

Proposal 11: The measurement gap applicability rule for SA single carrier shall be also applied for NR CA configuration.

Proposal 12: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

CSSF

The candidate proposals on CSSF in the last meeting are copied as follows [3]:

- Proposal 1: Study and accommodate CSSF requirements for ATG DL CA - Proposal 2: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

The existing CSSF defined in R18 ATG only consider for SA single carrier. Since FR1 only CA is introduced in R19 ATG, we believe that the legacy scheme for defining CSSF can be reused. For CSSFoutside_gap, PCC will occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap will share another measurement searcher resource.

Proposal 13: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that is, PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

- Conclusion

This document discussed preliminary issues for R19 ATG RRM requirements and presented following observations and proposals:

Proposal 1: For the specification structure for R19 ATG in TS 38.133, the CA related requirements can be directly added in the sections defined for R18 ATG UE.

Observation 1: In TN network there are no application scenarios for EMR mechanism, and we think that EMR is an optional capability for UE, and whether to use the report depends on NW.

Proposal 2: If the idle/inactive mode CA measurements requirements are introduced in ATG CA,

- EMR is an optional capability for ATG UE, and whether to use the report is up to NW.

- The related requirements defined in TN can be reused as baseline.

Observation 2: The network may have a little chance to go into the idle/inactive mode in ATG scenario.

Proposal 3: Deprioritize to introduce idle/inactive mode CA measurements requirements in ATG CA.

Proposal 4: Prefer not to further introduce Rel-18 enhanced EMR requirements in idle/inactive mode for ATG CA.

Proposal 5: There is no need to define MTTD requirements in R19 ATG CA.

Proposal 6: For ATG inter-band CA, MRTD requirement is a little larger than 3us in FR1.

Proposal 7: The existing interruption requirements can also apply to R19 ATG CA as a baseline, RAN4 could further discuss the specific requirements and consider ATG related features.

Proposal 8: The SCell activation and deactivation delay requirements should be defined at least for the ATG UE configured with single downlink SCell in standalone NR CA, and the FR1 related requirements can be reused as the starting point, which at least include the following:

- SCell Activation Delay Requirement for Deactivated SCell

- SCell Deactivation Delay Requirement for Activated SCell

- Direct SCell Activation at SCell addition

- Direct SCell Activation at Handover

- Direct SCell Activation at RRC Resume

- Fast SCell Activation Delay Requirement for Deactivated SCell

- SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.

Proposal 9: For active BWP switch delay on multiple CCs requirements for ATG UE, introduce the following requirements and the legacy can be reused as baseline:

- Simultaneous DCI based BWP switch delay on multiple CCs

- Simultaneous RRC based BWP switch delay on multiple CCs

- Simultaneous and non-simultaneous Timer based BWP switch delay on multiple CCs

Proposal 10: Introduce pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, the legacy requirement can be reused.

Proposal 11: The measurement gap applicability rule for SA single carrier shall be also applied for NR CA configuration.

Proposal 12: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Proposal 13: Reuse the legacy scheme when defining the CSSFoutside_gap for ATG FR1 only CA, that is, PCC occupy one measurement searcher resource, SCC and other inter-frequency MO with no measurement gap share another measurement searcher resource.

- Reference

[1]	RP-240839, New WID on Enhancements for Air-to-ground network for NR, Apple

[2]	RP-242381, Revised WID on Enhancements for Air-to-ground network for NR, CMCC

[3]      R4-2413959, WF on RRM requirements for NR_ATG_enh, CMCC

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0