3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415155

# Hefei , CN, Oct 14 – 18, 2024

Title: 	Discussion on measurements related enhancements for Rel-19 LTM

Source: 	CATT

Agenda item:	6.24.2.1

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#105. The issues in this paper are related to measurements related enhancements and the objectives are copied as follows [2]:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1]

  - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

- Specify RRM requirements related to the above objectives as necessary [RAN4]

Considering some restrictions on Rel-18 mobility in the justification of WID, the following aspects should be addressed in Rel-19 work item.

- Rel-18 LTM mobility doesn’t support event triggering for L1 measurements.

- L1 measurements for Rel-18 LTM procedures are limited to SSB measurements.

This document will discuss the RRM issues about event triggered L1 measurement and CSI-RS based L1 measurement for LTM, and give our understanding and proposals.

- Discussion

Topic 1: Event triggered L1 measurement reporting

- Sub-topic 1-1 RRM scope of Event triggered L1 measurement reporting

In the last meeting, the candidate solutions for RRM scope of event triggered L1 measurement reporting are copied as below [3]:

- Candidate solutions: - Option 1: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value. RAN4 can wait for more RAN2 input. (CATT) - Option 2: (Apple) - Requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. Details are FFS. - Existing framework in L3 event triggered reporting RRM requirements can be reused to cover L1 event triggered reporting. - Option 3: define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. (CMCC, Apple, Samsung, HW, ZTE, Nokia, E///, vivo,CTC) - Option 3a: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3. - Option 3b: RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed. (Samsung) - Option 3c: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. (Nokia, E///, vivo, CMCC) - Option 4: RAN4 should discuss the accuracy requirement for event triggered L1 measurement reporting. (ZTE) - Option 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example (Nokia) - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements - early DL sync (Tfirst-RS = 0 conditions) - early ASN.1 decoding and validity check - LTM L1 measurement requirements - Option 6: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC, vivo)

From RAN4’s perspective, we need to define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. Based on the current spec for LTM in R18, we agree that event triggered reporting for LTM L1 measurement can to be introduced in section 9.14.3 and section 9.15.3 [4].

In addition, regarding the supported RS types for event-triggered L1 reporting, the agreements in RAN1#118 are as follows:

Agreement (RAN1#118) - SSB based L1-RSRP measurements is supported for - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting

According to the agreements from RAN1, both CSI-RS and SSB based L1-RSRP measurements are supported for event-triggered reporting. Therefore, RAN4 also need to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

Proposal 1: Define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay.

- RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

- Event Triggered Reporting for LTM L1 measurement can to be introduced in section 9.14.3 and section 9.15.3.

- Sub-topic 1-2 Filtering assumption in event evaluation

In the last meeting, RAN4 discussed the RRM impacts caused by filtering assumption in event evaluation, the related discussion are under discussion in RAN1/2, and companies were still controversial about the type of filtering. The candidate solutions are copied as follows [3]:

- Candidate solutions: - Option 1: Define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Wait for more RAN1/2 progress to discuss the detailed requirements. (MTK)

As we know, the filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 filtering. However, there is still no conclusion on whether to introduce a L1 filtering in RAN1. Since the current measurement requirements defined in R18 LTM didn’t consider the any filtering, if L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Otherwise, there is no impact on RAN4.

Observation 1: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching.

Observation 2: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 2: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

Topic 2: CSI-RS based L1 measurement

According to the WID, Rel-19 mobility enhancements will support CSI-RS measurements for LTM procedures and enable CSI-RS based beam management[2]. In the last meeting, RAN4 discussed many issues for CSI-RS based L1 measurement and we will further give our proposals for the following related issues.

- Sub-topic 2-1 Measurement metrics for CSI-RS based L1 measurement

In RAN1#118 meeting, the measurement metrics for CSI-RS based L1 measurement has been discussed. RAN1 has agreed to support L1-RSRP measurement based on CSI-RS, but still FFS for supporting L1-SINR measurement based on CSI-RS. The agreements in RAN1 are copied as foolows:

Agreement (RAN1#118) - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS

The L1-SINR measurement results can better reflect the signal quality and channel status. For the CSI-RS based L3 measurement, SS-SINR measurement has been supported. If SS-SINR measurement is also introduced for CSI-RS based L1 measurement, it will have RAN4 impact and RAN4 needs to define the requirements for CSI-RS based L1-SINR measurement.

Observation 3: RAN1 is discussing whether to support L1-SINR measurement based on CSI-RS. If SS-SINR measurement is also introduced for CSI-RS based L1 measurement in RAN1, it will have RAN4 impact.

Proposal 3: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement.

- RAN4 to wait for more RAN1 input.

- Sub-topic 2-2 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

In the last meeting, RAN4 discussed the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, but there was no consensus among the companies. Some companies supported to reuse the definition of intra-frequency and inter-frequency for CSI-RS based L3 measurement as legacy, other companies believed that the center frequency of the CSI-RS resource of the serving cell in the legacy definition is not clear for CSI-RS based L1 measurement. The candidate solutions are copied as below [3]:

- Candidate solutions: - Option 1: following CSI-RS based L3 measurement, (Apple) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK) - Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo)

In legacy, CSI-RS for mobility was supported for L3 measurement, and the CSI-RS resource is configured based on the frequency in Serving MO, so the centre frequency of the CSI-RS resource of the serving cell is clear. For CSI-RS based L1-RSRP measurement, there may be multiple CSI-RS resources of the serving cell and multiple centre frequencies of the CSI-RS resource of the serving cell, so the reference centre frequency of the CSI-RS resource in the definition of intra- and inter-frequency is not clear.

For backward compatibility, having a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement is better. One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

Also, if most companies think that it doesn't make much sense to have a definition, we also can compromise to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

Observation 4: For CSI-RS based L1-RSRP measurement, there may be multiple CSI-RS resources of the serving cell and multiple centre frequencies of the CSI-RS resource of the serving cell, so the reference centre frequency of the CSI-RS resource in the definition of intra- and inter-frequency is not clear.

Proposal 4: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy.

- One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

Proposal 5: Another way is to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

- Sub-topic 2-3 Supported measurement types

In the last meeting, RAN4 discussed the supported measurement types, but there was no consensus among the companies. The candidate solutions are copied as below [3]:

- Candidate solutions: - Option 1: (CATT) - Intra-frequency - Inter-frequency without gap - Inter-frequency with gap - Option 2: (Apple) - Intra-frequency without gap - Inter-frequency with gap - Option 3: deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK) - Option 4: (vivo) - For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases: - CSI-RS based L1 measurement without gap

As far as we know, the bandwidth of SSB is fixed, and the transmission bandwidth of the actual system may be much larger than that of SSB, and the measurements based on SSB may not be able to truly reflect the quality of the wireless link of DL outside the transmission bandwidth of SSB, which is one of the main reasons for the introduction of CSI-RS based L3 measurement.

For CSI-RS based L1 measurement, if RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited. Generally speaking, we think that CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario.

Observation 5: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 6: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario.

For the supported measurement types, we beilieve that RAN4 can follow the supported types for CSI-RS based L3 measurement. In the legacy CSI-RS based L3 measurement requirements, RAN4 has supported intra-frequency without gap and inter-frequency with gap. Since the defination of intra-/inter-frequency should wait for more RAN1/2 peogress, so RAN4 can firstly discuss the supported measurement types from wth/without gap perspective, and we support to define RRM requirements for CSI-RS based L1 without and with gap measurement.

Observation 7: The requirements for CSI-RS based L3 intra-frequency without gap and inter-frequency with gap measurement have been defined in the current spec.

Proposal 6: RAN4 at least support the following measurement types for CSI-RS based L1 measurement：

- Without gap

- With gap

- Sub-topic 2-4 Measurement gap related issues

In Rel-18, RAN4 only define the requirement for inter-frequency L1-RSRP measurement with type 1 MG. In Rel-19, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement. In our view, at least the type 1 MG should be supported, and whether to introduce the new gap should be discussed and wait for more RAN1 conclusions.

Proposal 7: In Rel-19 LTM, RAN4 needs to discuss whether to reuse or introduce the new gap for CSI-RS based L1 measurement.

In the last meeting, for the case of gap-based measurement, some companies were concerned about whether the existing gap can cover the CSI-RS configuration in the same frequency layer. In our view, the same problem exists in the legacy CSI-RS based L3 measurement, and the CSI-RS configuration is also flexible. RAN4 defined the requirements applicability for CSI-RS based L3 measurement and used time window to limit CSI-RS resourse transmission, the details in clause 9.10 of TS 38.133 are copied as follows [4]:

#### 9.10.3.2 Requirements applicability The associated SSB layer of the CSI-RS follows the same requirements as SSB based measurements defined in 9.3. The requirements in clause 9.10.3 apply, provided: - The associated SSB of the cell being identified or measured is detectable, and - All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and - The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and - CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

Therefore, at the very least, RAN4 can follow the same approach as CSI-RS based L3 measurement to address companies' concerns, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement.

Observation 8: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 8: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement.

In additon, another feasible approach is to introduce related gap enhancements. In R18 LTM, RAN4 just supported R16 type 1 gap as a baseline. Considering that many gap enhancements have been specified in Gap WI, which also can be used for CSI-RS based L1 measurement in R19 LTM. If RAN4 introduce the concurrent gap, the above issue can also be resolved.

Observation 9:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 9: For CSI-RS based L1 with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM.

- RAN4 to discuss whether to support other types of gap and wait for more RAN1 conclusions.

- Sub-topic 2-5 Supported frequency range

In the last meeting, the candidate solutions for supported frequency range are copied as below [3]:

- Candidate solutions: - Option 1: only FR1 and FR2-1 (Apple)

We think that the supported frequency range can follow R18 LTM, and FR1 and FR2-1 can be supported for CSI-RS based L1 measurement in R19.

Proposal 10: RAN4 define the requirements in FR1 and FR2-1 for CSI-RS based L1 measurement.

- Sub-topic 2-6 Other applicability of RRM requirements

In the last meeting, RAN4 discussed applicability of RRM requirements for CSI-RS based L1 RSRP measurement, but there was no consensus among the companies. The candidate solutions are copied as below [3]:

- Candidate solutions: - P1: Consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement. (Apple) - P1a: In FR1, UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells. (HW) - P2: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss two possible options: (HW) - Option 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and perform L1 CSI-RS measurement with Rx beam sweeping on candidate cells. - Option 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. - P3: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK)

In the last meeting, since companies thought that the behavior of the UE side would be related to the configuration of the CSI-RS repetition parameter, so firstly we want to clarify that what do CSI-RS repetition ON and OFF mean. This parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side.

Specifically, if the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. That is, the UE needs to perform Rx beam sweeping. If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, while the UE can do without Rx beam sweeping.

Observation 10: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side.

Observation 11: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. That is, the UE needs to perform Rx beam sweeping.

Observation 12: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, while the UE can do without Rx beam sweeping.

In FR1, there is no need to consider beam sweeping. UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

Proposal 11: In FR1, UE shall firstly perform SSB based L3 measurement on candidate neighbour cells, then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

In FR2, we want to further analyse two options proposed by company. The first step for UE of both of Option 1 and Option 2 is the same. The difference is that Option 1 selects the Rx beam by beam sweeping based on CSI-RS resources and switches based on the CSI-RS L1 measurement results. Option 2 is to use the Rx beam based on the SSB beam sweeping in R18 LTM and switches based on the CSI-RS L1 measurement results.

In contrast, the CSI-RS selected beam in Option 1 will be narrower and more accurate, but the total beam sweeping and L1 measurement latency will be longer since the beam sweeping factor will be larger.

Option 2 will use the selected beam based on SSB based L1 measurement, which will be wider than the CSI-RS, and eventually UE will perform a CSI-RS based L1 measurement on the selected beam, with a shorter total beam sweeping time (SSB based only) and L1 measurement latency (both SSB based L1 measurement and CSI-RS based L1 measurement). For option 2, it will only be available for UEs that support R18 LTM, and the advantage of UE performing additional CSI-RS based L1 measurement is that switching based on CSI-RS L1 measurement results will improve the accuracy because the measurement accuracy is more accurate.

Observation 13: The advantage of UE performing additional CSI-RS based L1 measurement compared to R18 LTM is that switching based on CSI-RS L1 measurement results will improve the accuracy, as the measurement accuracy is more accurate.

In general, we believe that in the case of CSI-RS repetition ON, since the gNB transmits the CSI-RS with a fixed beam, the UE should perform Rx beam sweeping, so Option 1 is better since the beam sweeping is more accurate. In the case of CSI-RS repetition OFF, the Tx beam sweeping on the UE side is not required because the gNB performs the Tx beam sweeping, and whether to perform Rx beam sweeping is up to UE. Considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam, so Option 2 is acceptable. However, whether additional CSI-RS based L1 measurement is necessary in this case depends on whether the gain of the measurement accuracy is necessary.

Observation 14: For FR2, in the case of CSI-RS repetition ON, since the gNB transmits the CSI-RS with a fixed beam, the UE should perform Rx beam sweeping, and Option 1 is better since the beam sweeping is more accurate.

Proposal 12: In FR2, RAN4 to define the L1 measurement requirements for the UE behavior in Option 1 with CSI-RS repetition ON.

Observation 15: For FR2, in the case of CSI-RS repetition OFF, considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam, so Option 2 is acceptable.

Proposal 13: For option 2 in FR2, whether additional CSI-RS based L1 measurement compared to R18 LTM procedure is necessary depends on whether the gain of the measurement accuracy based on CSI-RS is necessary.

Proposal 14: If the gain of additional CSI-RS based L1 measurement based on R18 LTM procedure is meaningful,

- RAN4 to define the L1 measurement requirements for the UE behavior in Option 2 with CSI-RS repetition OFF in FR2.

- UE is capable of supporting R18 LTM.

- Conclusion

This document discussed preliminary the issues for measurements related enhancements in Rel-19 and the following proposals and observations are presented:

Proposal 1: Define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay.

- RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

- Event Triggered Reporting for LTM L1 measurement can to be introduced in section 9.14.3 and section 9.15.3.

Observation 1: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching.

Observation 2: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 2: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

Observation 3: RAN1 is discussing whether to support L1-SINR measurement based on CSI-RS. If SS-SINR measurement is also introduced for CSI-RS based L1 measurement in RAN1, it will have RAN4 impact.

Proposal 3: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement.

- RAN4 to wait for more RAN1 input.

Observation 4: For CSI-RS based L1-RSRP measurement, there may be multiple CSI-RS resources of the serving cell and multiple centre frequencies of the CSI-RS resource of the serving cell, so the reference centre frequency of the CSI-RS resource in the definition of intra- and inter-frequency is not clear.

Proposal 4: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy.

- One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

Proposal 5: Another way is to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

Observation 5: If RAN4 only considers CSI-RS based L1-RSRP measurement within active BWP, the gain of CSI-RS based measurement will be limited.

Observation 6: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario.

Observation 7: The requirements for CSI-RS based L3 intra-frequency without gap and inter-frequency with gap measurement have been defined in the current spec.

Proposal 6: RAN4 at least support the following measurement types for CSI-RS based L1 measurement：

- Without gap

- With gap

Proposal 7: In Rel-19 LTM, RAN4 needs to discuss whether to reuse or introduce the new gap for CSI-RS based L1 measurement.

Observation 8: The CSI-RS configuration is also flexible in the legacy CSI-RS based L3 measurement, and RAN4 defined the requirements applicability for CSI-RS based L3 measurement.

Proposal 8: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement.

Observation 9:  Considering that gap enhancement has been specified in Gap WI, concurrent gap can be used for CSI-RS-based L1 measurements to cope with flexible CSI-RS configurations.

Proposal 9: For CSI-RS based L1 with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM.

- RAN4 to discuss whether to support other types of gap and wait for more RAN1 conclusions.

Proposal 10: RAN4 define the requirements in FR1 and FR2-1 for CSI-RS based L1 measurement.

Observation 10: CSI-RS repetition parameter indicates whether the Tx beam repetition of CSI-RS is turned on or not from gNB side.

Observation 11: If the CSI-RS repetition is configured with ON, the Tx beam repetition of CSI-RS is turned on, and the corresponding gNB will transmit CSI-RS resource in a fixed beam direction. That is, the UE needs to perform Rx beam sweeping.

Observation 12: If the CSI-RS repetition is configured with OFF, the Tx beam repetition of CSI-RS is turned off, and the corresponding gNB will transmit CSI-RS resource with beam sweeping, while the UE can do without Rx beam sweeping.

Proposal 11: In FR1, UE shall firstly perform SSB based L3 measurement on candidate neighbour cells, then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

Observation 13: The advantage of UE performing additional CSI-RS based L1 measurement compared to R18 LTM is that switching based on CSI-RS L1 measurement results will improve the accuracy, as the measurement accuracy is more accurate.

Observation 14: For FR2, in the case of CSI-RS repetition ON, since the gNB transmits the CSI-RS with a fixed beam, the UE should perform Rx beam sweeping, and Option 1 is better since the beam sweeping is more accurate.

Proposal 12: In FR2, RAN4 to define the L1 measurement requirements for the UE behavior in Option 1 with CSI-RS repetition ON.

Observation 15: For FR2, in the case of CSI-RS repetition OFF, considering the overall sweeping time and measurement latency, it is also feasible for UE to use SSB based coarse beam, so Option 2 is acceptable.

Proposal 13: For option 2 in FR2, whether additional CSI-RS based L1 measurement compared to R18 LTM procedure is necessary depends on whether the gain of the measurement accuracy based on CSI-RS is necessary.

Proposal 14: If the gain of additional CSI-RS based L1 measurement based on R18 LTM procedure is meaningful,

- RAN4 to define the L1 measurement requirements for the UE behavior in Option 2 with CSI-RS repetition OFF in FR2.

- UE is capable of supporting R18 LTM.

- Reference

[1]	RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]      RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[3]      R4-2414060 WF on NR mobility enhancements Phase 4, Apple

[4]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0